| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text to improve reliability and coverage of broadcast messages (16.3.6) |  |
| Date Submitted | 2010-1-13 |  |
| Source(s) | Yi Hsuan, Hujun Yin, Jiacheng Wang, Roshni Srinivasan Intel Corporation |  |
| Re: | Category: AWD comments / Area: Chapter 16.3.6 (DL-CTRL) “Comments on AWD 16.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text to improve broadcast message coverage |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text to improve reliability and coverage of broadcast messages (16.3.6)

Yi Hsuan, Hujun Yin, Jiacheng Wang, Roshni Srinivasan

Intel Corporation

- Introduction

Broadcast messages are transmitted like regular data burst with the following properties:

- Diversity mode transmission is used because there is no specific target receiver for the burst.

- HARQ is not applied.

- The target error rate is lower than the initial target error rate for data bursts. While the error rate for initial data transmission can be 10% or higher, broadcast message error rate should be 10% or even 1%.

Without HARQ, we need to make sure that the physical structure of the broadcast message can guarantee the performance and coverage. Our study shows that the existing code rate is not enough to ensure coverage. We propose a solution to fix the issues.

- Simulation of Broadcast Message Error Rates

The lowest code rate generated from burst size signaling depends on several factors: number of LRUs occupied by the burst, modulation level used, and number of data tones per LRU. For broadcast messages, the modulation level should be QPSK and the data tones per LRU can be either 80, 96, or 112 assuming SFBC with 5, 6 or 7 symbol subframe, respectively. The lowest code rates are summarized in the following table:

|  | 5 symbol subframe | 6 symbol subframe | 7 symbol subframe |
| --- | --- | --- | --- |
| Lowest code rate | 0.1 | 0.08 | 0.07 |

The following simulation parameters are used to verify the performance and coverage of broadcast messages.

- System Bandwidth: 10 MHz (1024 FFT)

- Carrier Frequency :2.5 GHz

- Number of OFDM symbols per subframe: 5

- Number of Antennas: 2 Tx by 2 Rx

- Antenna correlation: Tx: 0.4, Rx: 0.2

- MIMO mode 2x2 SFBC

- Modulation: QPSK

- Data boosting: 0dB

- Pilot boosting: 5dB

- MIMO receiver: MMSE

- Channel model: eITU-PedB 3 km/h, eITU-VehA 120 km/h

- Channel estimation: 2-D MMSE

- Interference mitigation (IM): On /Off

- Number of interferers: 3

- Interference scenarios: INR=Infinity(1000dB), Interference profile: 0 dB, -3 dB, -6 dB (-6 dB interferer has the same pilot pattern as the serving BS)

Figure 1: Broadcast message PER in eITU PedB channel

Figure 2: Broadcast message PER in eITU VehA channel

The 95% coverage of the 802.16m EMD baseline cell is at about -4 dB SINR. For the 5km radius cell, the 95% coverage is about -6 dB SINR. As shown in Figure 1 and 2, rate 0.1 with interference mitigation can reach 1% PER at -3.5 to -4 dB. For 10% PER, the required SINR is still around -5 dB, which is still not low enough for larger cell sizes.

To fix this issue, we want to allow time domain repetition of a broadcast burst to reduce the effective code rate. Broadcast A-MAP IE can signal that the same burst will be transmitted several more times with certain periodicity. This should give us an effective code rate of 1/20 or lower as well as some time domain diversity.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal (16.3.6.5.2.4.13, page 443, line 25, Table 829) ]

Table 829 – Broadcast Assignment A-MAP IE*

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| Broadcast-A-MAP_IE() { | - | - |
| A-MAP IE Type | 4 | Broadcast A-MAP IE |
| Burst Size | 6 | Burst size as indicated in the first 64 entries of Table 923 |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| Long TTI Indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0: 1 AAI subframe (default) 0b1: 4 DL AAI subframes for FDD or all DL AAI subframes for TDD |
| BCID | 2 | Broadcast channel ID |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst will be transmitted one more time 0b10: the same burst will be transmitted two more times 0b11: the same burst will be transmitted three more times |
| Periodicity of Repetition | 2 | 0b00: 1 frame periodicity 0b01: 2 frame periodicity 0b10: 3 frame periodicity 0b11: 4 frame periodicity |
| Reserved | TBD | Reserved bits |
| } |  |  |

-------------------------------------- End of Text Proposal -------------------------------------------