| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to define the S-SFH IE extension method in the 802.16m AWD (15.3.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yi Hsuan, Shantidev Mohanty, Roshni Srinivasan, Hujun Yin Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes a method to allow S-SFH subpacket extension |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to define the S-SFH IE extension method in the 802.16m AWD (15.3.6)

Yi Hsuan, Shantidev Mohanty, Roshni Srinivasan, Hujun Yin

Intel Corporation

- Introduction

To allow future development of 802.16m, S-SFH IEs need to be extended beyond the basic content, which is the content of the first release of the standard, in order to support new features and needs. The current AWD allows the possibility to extend S-SFH by using a 4-bit field in P-SFH which indicates the size of S-SFH in units of LRUs. The intention is to allow different numbers of LRUs for S-SFH. Given the MCS level, different LRU number can translate to different sizes of information bits. However there are some drawbacks:

- Signaling the number of LRUs for S-SFH requires more bits. 4-bits can signal up to 16 LRUs. In practice, we don’t need some many possibilities of extensions. It’s more preferable to use a smaller number of bits in P-SFH to signal the extension.

- Signaling the number of LRUs means that the size of S-SFH IE needs to be calculated based on the MCS level. Because there are more than one MCS levels of S-SFH, it implies more choices of extension sizes which is not necessary and can cause confusion.

We propose an S-SFH IE extension method with lower P-SFH overhead and less complexity as shown in the next section.

- S-SFH extension signaling in P-SFH

The purpose of S-SFH IE extension is to allow a S-SFH subpacket to contain more information bits. Therefore it makes more sense to indicate how many more bits are included in the subpacket beyond the default subpacket size. The default subpacket size is the subpacket size described in the first release of the 16m standard. Different subpakets can have different default subpacket sizes.

Two bits are adequate to signal three different extensions. 00 means that the default subpacket size is used. 01, 10 and 11 mean that the subpacket size is extended by 8, 16, and 24 bits respectively. To decode S-SFH, MS first decodes P-SFH to find out S-SFH IE size from S-SFH scheduling information bitmap and the 2-bit S-SFH extension field. The S-SFH scheduling information bitmap indicates which subpacket is transmitted and therefore the default subpacket size is known. The S-SFH extension field indicates how many more bits should be added on top of the default subpacket size.

With the subpacket size known, MS should calculate how many LRUs are needed for S-SFH and decoding can be performed on data tone retrieved from those LRUs. Extra padding bits can be applied in order to fill up the required number of LRUs. The number of required LRUs for an n-bit subpacket is given by

Required S-SFH LRU number = ceil (n/R/2/80), where R is the effective code rate of S-SFH; 2 is for 2 bits per QPSK symbol; 80 is the number of data tones per SFH LRU.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal 1 (Section 16.3.6.5.1.1, page 375, line 34) ]

| - S-SFH Size - S-SFH subpacket size extension | - 4 - 2 | - The units of LRU - 0b00: default subpacket size - 0b01: extended by 8 bits - 0b10: extended by 16 bits - 0b11: extended by 24 bits |
| --- | --- | --- |

- [ Recommended AWD Text Proposal 2 (Section 16.3.6.5.1.1, page 376, line 4) ]

S-SFH subpacket size extension

Each S-SFH subpacket can be extended up to 24 bits beyond the default subpacket size to support new standard releases. The default size of a subpacket is the size defined in the first release. To decode S-SFH, AMS first finds out the S-SFH IE size from S-SFH scheduling information bitmap and S-SFH subpacket size extension in P-SFH.

With the S-SFH IE size known, AMS shall calculate how many LRUs are needed for S-SFH based on the number of repetitions of S-SFH. Decoding is performed on data tones retrieved from those LRUs. The number of required LRUs for an n-bit S-SFH IE is given by

Required S-SFH LRU number = ceil (n/R/2/80), where R is 1/(4*number_of_repetition) of S-SFH; 2 is for 2 bits per QPSK symbol; 80 is the number of data tones per SFH LRU.

-------------------------------------- End of Text Proposal -------------------------------------------