| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to Multi-carrier Operation for Femtocell ABS (16.4) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Motoki Morita, Nader Zein, Linghang Fan, Hassan Al-Kanani, Tetsu Ikeda NEC | m-morita@bx.jp.nec.com |
| Re: | LB comment to 802.16m Amendment Working Document D3 |  |
| Abstract | This contribution provides a scheme allowing multi-carrier operation for Femtocell ABS. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for the IEEE 802.16m AWD: Multi-carrier Operation for Femtocell ABS

Motoki Morita, Nader Zein, Linghang Fan, Hassan Al-Kanani, Tetsu Ikeda

NEC

# Introduction

This contribution proposes a scheme that allows multi-carrier operation for Femtocell ABS without increasing interference to macro/micro cells.

# Multi-carrier Operation

## Multi-carrier Operation for Femtocell ABS

Multi-carrier operation may be supported by Femtocell ABS as depicted in Fig.1. All operational principles for multi-carrier operation also apply to a system involving Femtocell ABS unless explicitly stated otherwise. In this way, Femtocell ABS can assign a secondary carrier to an MS in addition to a primary carrier.

Femtocell

BS can assign the

secondary carrier to the MS

Femtocell

BS can assign a

secondary carrier to the MS

in addition to a primary one.

Fig.1  Illustration of the concept of multi-carrier operation for Femtocell ABS

## Interference in Multi-carrier Operation for Femtocells

Femtocell ABS happens to be the same carrier as the one used by macro/micro cell , the interference from Femtocell to macro/micro cell will subsequently increase.

RF2

Carrier assignment for a MS

in macro cell

Primary carrier

If the secondary carrier

with RF2 is to be added,

interference from

femtocell

to macro cell will increase.

RF1

RF2

Primary

carrier

Secondary carrier

(to be added)

Carrier assignment for a MS in

Femtocell

RF2

Carrier assignment for a MS

in macro cell

Primary carrier

RF2

Carrier assignment for a MS

in macro cell

Primary carrier

If the secondary carrier

with RF2 is to be added,

interference from

femtocell

to macro cell will increase.

RF1

RF2

Primary

carrier

Secondary carrier

(to be added)

Carrier assignment for a bsequently bythe Femto ABStbeen used by NEC UK as input to their feedback for this rnquiry.ge per feature package and per majobsequently bythe Femto ABStbeen used by NEC UK as input to their feedback for this rnquiry.ge per feature package and per majo  bsequently bythe Femto ABStbeen used by NEC UK as input to their feedback for this rnquiry.ge per feature package and per majoMS in

Femtocell

Fig.2  Illustration of the scenario that interference increase in multi-carrier operation for Femtocell ABS.

## Proposed Scheme for Interference Mitigation in Multi-carrier Operation

In order to enable the necessary interference avoidance or interference mitigation schemes, the Femtocell ABS shall be capable to scan the signals transmitted from neighbor ABSs and measure the signal strength. Based on these measurements, the Femtocell ABS selects a primary carrier which has a minimum interference impact on macro/micro MSs. In this case  the macro MSs will receive little interference from the Femtocell ABS even if they move closer to the Femtocell ABS.

In case of multi-carrier operation for Femtocell ABS, Femtocell ABS should also consider interference avoidance or mitigation when assigning the secondary carrier.

According to the secondary carrier management, macro ABS decides the activation or deactivation of secondary carrier(s) based on load condition of carriers and may transmit the list of active carriers to the AMS. Transmit power for each carrier of the macro ABS may be used on the load condition. If the interference from Femtocell ABS to macro MS becomes large, the transmit power increase of the macro ABS will not help. Subsequently if the load condition for a carrier exceeds certain threshold, the macro ABS requests a deactivation of the secondary carrier and transmit the decision to Femtocell ABS as shown in Fig.3 below. The Femtocell ABSs can receive the deactivation information at an initial setting or during an unavailable interval in the low duty operation mode. The Femtocell ABSs will in this case avoid using this carrier as the secondary carrier.

Fig.3  Proposed Scheme for interference mitigation in multi-carrier operation

The carrier activation/deactivation request from the macro ABS to the Femto ABS may be transmitted using a load indicator Information Element “LD-indicator” field in S-SFH SP3. This Information Element may be implemented through a 1 bit indicator such that when “LD-indicator” is set to 1 the load is “High” while 0 value is used to indicate “Not High Load”

The above mentioned load indicator is controlled by a resource utilization protocol and by adopting pre-defined thresholds. The protocol triggers the appropriate changes in the load indicator based on resource utilization or used power.  When the resource utilization or used power exceeds the predefined threshold, then the load indicater is set to “High”. In any other condition, the load indicator is set to “Not High Load”. The load indicator changes would, like other IEs within S-SFH, trigger a broadcast of the S-SFH. Alternatively the activation and deactivation request may be communicated with Femto ABS over the direct link or backhaul.

The difference between multicarrier in macro cell and multicarrier in femto cell is that the multicarrier in femto cell is limited due to the interference to macro cell while the multicarrier in macro cell is not limited due to another cell condition. If the interference to macro cell in a certain carrier will be high and performance of macro cell will degrade, the femto cell can not use the multicarrier in the carrier.

# Proposed Text

[Insert text in subclause 16.4 as follows]

-------------------------------------------------------Start of the Text-------------------------------------------------------

16.4 Support for Femto ABS

16.4.x Multi-carrier Operation

Multi-carrier operation may be supported by Femtocell ABS. All operational principles for multi-carrier operation also apply to a system involving Femtocell ABS unless explicitly stated otherwise. Femtocell ABS may assign a secondary carrier to an AMS in addition to a primary carrier.

In order to enable the interference avoidance or mitigation schemes, the Femtocell ABS shall be capable to scan the signals transmitted from neighboring ABSs and measure the signal strength. The Femtocell ABS selects a primary carrier which has minimum interference impact on macro/micro MSs based on the measurement.

According to the secondary carrier management, macro ABS decides the activation or deactivation of secondary carrier(s) based on load condition of carriers and may transmit the list of active carriers to the AMS. If the load condition for a carrier exceeds certain threshold, the macro ABS requests  a deactivation  of the secondary carrier and transmit the decision to Femtocell ABS. The Femtocell ABSs can receive the deactivation information at an initial setting or during an unavailable interval in low duty operation mode. The Femtocell ABSs shall in this case avoid using this carrier as the secondary carrier.

The carrier activation/deactivation request from the macro ABS to the Femto ABS may be transmitted using a load indicator Information Element “LD-indicator” field in S-SFH SP3. This Information Element may be implemented through a 1 bit indicator such that when “LD-indicator” is set to 1 the load is “High” while 0 value is used to indicate “Not High Load”

The above mentioned load indicator is controlled by a resource utilization protocol and by adopting pre-defined thresholds. The protocol triggers the appropriate changes in the load indicator based on resource utilization or used power.  When the resource utilization or used power exceeds the predefined threshold, then the load indicator is set to “High”. In any other condition, the load indicator is set to “Not High Load”. The load indicator changes would, like other IEs within S-SFH, trigger a broadcast of the S-SFH. Alternatively the activation and deactivation request may be communicated with Femto ABS over the direct link or backhaul.

--------------------------------------------------------End of the Text-------------------------------------------------------