| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Signaling support for Power Management in Connected Mode |  |
| Date Submitted | 2009-31-12 |  |
| Source(s) | N. Himayat, M. Venkatchalam, S. Mohanty, Tom Harel Intel Corporation S. Andreev, P. Gonchukov, A. Turlikov, SUAI | Voice: 408-765-5043 E-mail: nageen.himayat@intel.com |
| Re: | Signaling support for Power Management in Connected Mode (section 16.2.22) |  |
| Abstract | Section 16.2.22 of P802.16m/D2 (Power Management in Connected Mode) allows for the AMS to initiate BS controlled power savings at the AMS. This contribution proposes specific signaling to enable this feature. |  |
| Purpose | Review and adopt. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Signaling support for Power Management in Connected Mode

Intel Corporation

# Introduction

Section 16.2.22 of the P802.16m/D2 on “Power management in connected mode” includes support for the AMS to report its battery level to the ABS. This contribution defines the signaling necessary to exchange this information.

The protocol to exchange AMS battery level is simple. When the AMS’s battery level goes below a threshold, the AMS will indicate a “low battery” level. When the battery level improves beyond the threshold or the AMS is plugged into a power source, the AMS will indicate a “high battery” level.  This single bit of information can be communicated by using the first reserved bit of the PMC_REQ message defined in [1].  The threshold to determine the “high or low battery” state may be negotiated between the AMS and ABS during the AMS basic capability exchange with the ABS, using the SBC_REQ/RSP messages. The specific text changes are indicated in the next section.

# Text Proposal

Insert the following text in section 16.2.22 of P802.1m6/D3 after line 20 of page 285.

----------------------------------------------Start of the text proposal (section 16.2.22) -------------------------------

### 15.3.16.1 Reporting AMS Battery Level

The AMS may signal its battery level by setting the one-bit “Battery Level” field.  Table x.x.1 defined the Battery Level field.  The Battery Level field shall be sent using the reserved bits in the PMC_REQ message.

Table x.x.1 Battery Level Field

| Field | length | Value |
| --- | --- | --- |
| Battery Level | 1 bit | 0: AMS battery level is high or AMS is plugged into a power source. 1: AMS battery level is low. |

The threshold for indicating a low battery level may be negotiated by the AMS, during the AMS basic capability exchange using the SBC_REQ/SBC_RSP messages.    The capability exchange will also include the time duration over which the AMS will average the battery level measurements before it reports the battery level indication to the ABS.

----------------------------------------------End the text proposal (section 16.2.22) -----------------------------------

# References

- IEEE STD 802.16, Rev2, D9, Jan. 2009.

- IEEE P02.16m/D3.