| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ Feedback Extended Header (16.2.2) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30b on P802.16m/D3 |  |
| Abstract | This contribution proposes a new extended header for ARQ feedback operation. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ Feedback Extended Header (16.2.2)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Kwangjae Lim, Chulsik Yoon

ETRI

# Introduction

In the last meeting, we agreed to put the total length of extended headers right after AGMH to speed up the processing of payload at receiver side (comment #73). However, if ARQ feedback IE is piggybacked with a MAC PDU, the receiver didn't know the starting position of the payload because the length of ARQ feedback IE is not included in Extended Header Group Length. It degrades the merits which came from the decision made in the last meeting.

In this contribution, we propose a new extended header which contains ARQ feedback IE to clearly define the location and length of the piggybacked ARQ feedback IE.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Remove ‘AFI’ from Table 663]

[Add the following new section under section 15.2.2.2]

15.2.2.2.x ARQ Feedback Extended Header (AFEH)

The format of ARQ feedback extended header is defined in Table xxx. This header is used by an ARQ receiver to signal positive or negative acknowledgement.

Table xxx – AFEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AFEH () { |  |  |
| Type | 4 | AFEH type |
| ARQ Feedback IE(s) | Variable | See 16.2.13.1.1 |
| } |  |  |

---------------------------------------------------------End of the Text----------------------------------------------------------