| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures to Indicate the Super-Frame where the new S-SFH SP Contents are used in IEEE 802.16m (16.2.23) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty Intel | Emails: Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.23” |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m LB#30b. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m L#30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedures to Indicate the Super-Frame where the new S-SFH SP Contents are used in IEEE 802.16m (16.2.23)

Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty

Intel

- 1 Text Proposal for Change of S-SFH IE information fields

In D3 it is not clear when the contents of SFH is changed, the super-frame number when the new information is used.

This comment proposes procedures used by an AMS to learn when the new SFH information is used.

Proposed Text

-------------------------------  Proposed Text Start  ---------------------------------------------------

[Note to Editor: Modify the text after line 28 in Page 285 in “16.2.23 Update of S-SFH IEs” as follows]

If the AMS determines that the “S-SFH change count” field in P-SFH has not changed, then the AMS determines that it has up to date information.

When AMS determines that the “S-SFH change count” field in P-SFH has changed, it uses the “S-SFH SP change bitmap” to determine the SPs whose contents have changed. If the difference of “S-SFH change count” in the current P-SFH and the “S-SFH change count” stored at the AMS is one, then the AMS determines that the S-SFH has changed only once since it has last received S-SFH. Else if the difference of  “S-SFH change count” in the current P-SFH and the “S-SFH change count” stored at the AMS is greater than one, then the AMS determines that the S-SFH has changed multiple times since it has last received S-SFH.

When the difference of  “S-SFH change count” in the current P-SFH and the “S-SFH change count” stored at the AMS is one, if the “S-SFH SP Change Bitmap” is not “000”, then the AMS determines the S-SFH SP(s) that have changed by observing the bits of this Bitmap that are non-zero. If only one of the S-SFH SPs has changed, then the AMS determines if the current S-SFH carries this S-SFH SP. If the changed S-SFH SP is carried in the current S-SFH, then AMS uses the “Start super-frame offset where new SP information” field of this S-SFH SP to determine if the new information is already applied. A non-zero value of the “Start super-frame offset where new SP information” field indicates that the new information is not applied. The AMS then determines the new information fields of this S-SFH SP. Similarly, the AMS determines the new information fields of the other S-SFH SPs.

After learning that the contents of S-SFH SPn (n=1, 2,3) is changed, AMS uses the “Start Super-frame offset where the new SPn (n=1, 2, 3) is used” field of the corresponding S-SFH SPs indicate(s) to determine whether or not the new information is applied. This field functions as a countdown counter where its value is set to 11 in the super-frame where the new information is transmitted for the first time. In the subsequent super-frames its value is decremented sequentially (11->10->01->00). The new information is applied starting from the super-frame where the value of this field is 00.  After the new information is applied, the “S-SFH SP change bitmap” is reset to 000. Thus, when an AMS determines that the “S-SFH change count” field in P-SFH has changed but “S-SFH SP change bitmap” is 000, it realizes that the new information is already applied. In this case, the AMS can not determine the S-SFH SP(s) that are changed. Therefore, the AMS receives all the S-SFH SPs during their earliest transmission occurrences. When the AMS does not have the latest S-SFH SPs that are already applied, the AMS does not transmit in the uplink.

The process of S-SFH SP update is illustrated in Figure xxx where the content of S-SFH SP1 is changed. Similarly, Figure yyy illustrates the scenario where contents of SP1 and SP2 are changed. Figure zzz shows the procedures used by an AMS to determine that contents of one or more S-SFH SP(s) have changed and to have up-to-date S-SFH SP contents.

When the AMS learns the super-frame number where a particular S-SFH SP contents has changed, it uses the “Rate of change of SP (1-3) Info” field in S-SFH SP3 to determine the super-frame number until which the contents of this SP remains the same. For example, when “Rate of change of SP (1-3) Info” field in S-SFH SP3 specifies that the contents of the S-SFH SP1 remains same over 10 SFHs, and AMS learns that S-SFH SP1 contents are updated in SFH K, then the AMS determines that the contents of S-SFH SP1 remains same until SFH K+10. Thus, the AMS may not receive the S-SFH SP1 that are transmitted between SFH K+1 and SFH K+9 considering that the AMS receives the S-SFH SP1 transmitted during SFH K. The AMS may use similar process to skip SFH that carry SP2 and SP3.

Figure xxx: Illustration of SP1 content update.

Figure yyy: Illustation of SP1 and SP2 content update.

Figure zzz: Procedures uses by an AMS to have up-to-date S-SFH contents.

-------------------------------  Proposed Text End ---------------------------------------------------