| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Procedures to Indicate the Super-Frame where the new S-SFH SP Contents are used in IEEE 802.16m (16.2.23) |  |
| Date Submitted | 2009-01-13 |  |
| Source(s) | Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty Intel | Emails: Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “16.2.23” |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m LB#30b. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m L#30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Procedures to Indicate the Super-Frame where the new S-SFH SP Contents are used in IEEE 802.16m (16.2.23)

Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty

Intel

- 1 Text Proposal for Change of S-SFH IE information fields

In D3 it is not clear when the contents of SFH is changed, the super-frame number when the new information is used.

This comment proposes procedures used by an AMS to learn when the new SFH information is used.

Proposed Text

-------------------------------  Proposed Text Start  ---------------------------------------------------

[Note to Editor: Modify the text after line 28 in Page 285 in “16.2.23 Update of S-SFH IEs” as follows]

[Change1: Modify the text in section 16.2.23, line 22, page 285 as follows]

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text  - - - - - - - - - - - - - - - - - - - - - - - -

16.2.23 Update of S-SFH IEs

The S-SFH SP IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP 3 IE) are transmitted at scheduling periods by the ABS at the scheduled period of each SP, respectively, with different periodicity (i.e., TSP1 < TSP2 < TSP3). This SP scheduling periodicity information is transmitted in the S-SFH SP 3 IE. Otherwise, an AMS may implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE. If AMS has been this information, it shall assume that this information is valid until this information is changed.

Every superframe, the ABS transmits P-SFH IE containing the S-SFH Scheduling information bitmap, SCDS-SFH change count, and S-SFH SP change bitmap, and Start super-frame offset where new S-SFH SP1 and SP2 information is used. The S-SFH Scheduling information bitmap indicates S-SFH SP IE(s) which is transmitted in a superframe. AMS can implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE. The SCDS-SFH change count shall remain unchanged as long as anyall of the values (except MSB of superframe number in S-SFH SP 1 IE) of S-SFH SP IEs remain unchanged. The SCDS-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH IEs changes. The changed S-SFH SP IE(s) shall be transmitted at scheduling superframesthe scheduled superframes of period corresponding to each S-SFH SP IE.

If the AMS determines that the “S-SFH change count” field in P-SFH has not changed, then the AMS determines that it has up to date information.

When the AMS learns the super-frame number where S-SFH contents has changed, it uses the “Rate of change of S-SFH SP1 and SP2 Info” field in S-SFH SP3 to determine the super-frame number until which the contents of this S-SFH remain the same. For example, when “Rate of change of S-SFH SP1 and SP2 Info” field in S-SFH SP3 specifies that the contents of the S-SFH IE remains same over 32 * the periodicity of S-SFH SP1 superframes.

Each bit of the S-SFH SP change bitmap indicates the changing status of the corresponding S-SFH SP IE in association with the S-SFH change count. The bit # 0 (LSB), bit #1 and bit #2 (MSB) are mapped to S-SFH SP1 IE, S-SFH SP 2 IE and S-SFH SP3 IE, respectively. If any of the values (except MSBs of superframe number in S-SFH SP 1 IE) of an S-SFH SP IE are changed, the corresponding bit corresponding to the changed S-SFH SP IE is toggledset to ‘1’.

The Start super-frame offset where new S-SFH SP1 and SP2 information is used in the P-SFH IE is used to indicate which S-SFH change count shall be considered for applying the system parameters in the S-SFH SPx IEs. If the Start super-frame offset where new S-SFH SP1 and SP2 information is used is set to ‘00’, the AMS shall use the system parameters of S-SFH SPx IEs associated with the current S-SFH change count. Otherwise, if the Start super-frame offset where new S-SFH SP1 and SP2 information is used is not ‘00’, the AMS shall store the changed parameters of S-SFH SP IEs associated with the current S-SFH change count, but use the parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16).

During the time duration where the Start super-frame offset where new S-SFH SP1 and SP2 information is set to ‘11’, AMSs shall use the system parameters in the S-SFH SPx IEs associated with the previous S-SFH change count, while storing the system parameters for the changed S-SFH SPx IE(s) for the future use when the configuration transition flag is set to ‘0’. During the time duration where the Start super-frame offset where new S-SFH SP1 and SP2 information is set to ‘11’ and before toggling it to ‘0’.

According to the SCDS-SFH change count, and S-SFH SP change bitmap, and Start super-frame offset where new S-SFH SP1 and SP2 information, AMS previously decides whether it shall decode S-SFH IE or not AMS knows if it is needed to decode S-SFH IE in the current superframe to update the system parameters broadcasted within the S-SFH SP IE, and the AMS may decide to disregard the S-SFH IE with the presumption of there being no change in S-SFH SP IE compared to that already stored in the AMS.

AMS shall compare values of each SCDS-SFH change count in the last received P-SFH IE and the last stored P-SFH IE whenever it receives P-SFH IE.

a) If there is no difference of two SCDS-SFH change counts, AMS may not decode S-SFH IE in the superframe.

b) Else, AMS shall check that change bits are toggled as the value of the difference.

1) If the number of toggled bits in the last received S-SFH SP change bitmap is the same as the difference, AMS shall update the S-SFH SP IE(s) whose bit of the S-SFH SP change bitmap is toggled. Additionally, AMS shall update SCD change count and S-SFH SP change bitmap.

2) Else, AMS shall update all S-SFH SP IEs, SCD change count and S-SFH SP change bitmap.

b) Else, AMS shall check the bit(s) set to ‘1’ in the S-SFH change bitmap for the S-SFH SP IE changed in association with the current S-SFH change count.

If the S-SFH SP IE indicated by S-SFH scheduling information corresponds to the changed S-SFH SP IE as indicated in the S-SFH change bitmap, the AMS shall decode the following S-SFH SP IE and store the parameters in association with the current S-SFH change count. The AMS shall update and store all the changed S-SFH SP IEs associated the increment of S-SFH change count at the scheduled superframe for each S-SFH SP IE.

According to the Start super-frame offset where new S-SFH SP1 and SP2 information, if the Start super-frame offset where new S-SFH SP1 and SP2 information is set to ‘00’, the AMS shall use the system parameters of S-SFH SPx IEs associated with the current S-SFH change count. Otherwise, if the Start super-frame offset where new S-SFH SP1 and SP2 information is not ‘00’, the AMS shall store the changed parameters of S-SFH SP IEs associated with the current S-SFH change count, but use the parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16).

During the listening interval, if an AMS doesn't receive the changed S-SFH SP IE(s), the AMS shall try to update the S-SFH SP IE(s). The AMS may only wake up at the first superframe where the S-SFH SP IE(s) will be scheduled in the sleep interval by using the valid SP scheduling periodicity information or implicit scheduling information. After performing update of the S-SFH SP IE(s), the AMS may power down one or more physical operation components or perform other activities that do not require communication with the ABS during the rest of sleep interval.

When the AMS detects that the changed S-SFH SP IE(s) is not scheduled as defined in the last SP scheduling periodicity information or as recognized implicitly, the AMS shall wake up until it receives the S-SFH SP IE(s). Before receiving the S-SFH SP IE(s), if the AMS receives SP scheduling periodicity information through the S-SFH SP 3 IE, it canmay use this information to update S-SFH SP IE(s). When scheduling periodicity of the S-SFH SP1 IE or S-SFH SP2 IE is changed, ABS may additionally transmit S-SFH SP3 IE at old scheduling periods of the S-SFH SP1 IE or S-SFH SP2 IE during a certain period of time. AMS uses SP scheduling periodicity information in the S-SFH SP3 IE to update the changed S-SFH SP IE(s).

Figure 458 – S-SFH IE update

[Change2: Modify the texts in page 375, section 16.3.6.5.1.1 as follows]

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text  - - - - - - - - - - - - - - - - - - - - - - - -

16.3.6.5.1.1 P-SFH IE

The P-SFH IE contains essential system information and it is mapped to the P-SFH. The format of the P-SFH IE is shown in Table 789.

Table 799—P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSB of Superframe number | 4 | Part of superframe number |
| S-SFH change count | 4 | Describes the S-SFH SPx IE that apply to this superframe. Indicates the value of S-SFH change count associated with the S-SFH SPx IE(s) transmitted in this superframe |
| S-SFH Size | 4 | The units of LRU |
| S-SFH Transmission Format | 2 | Indicate the transmission format (repetition) used for S-SFH. |
| S-SFH Scheduling information bitmap | 3 | 0b000: no S-SFH If 1st bit#0 (LSB) = 1, S-SFH includes SP1 IE. Otherwise no SP1 IE. If 2nd bit#1 = 1, S-SFH includes SP2 IE. Otherwise no SP2 IE. If 3rd bit#2 (MSB) = 1, S-SFH includes SP3 IE. Otherwise no SP3 IE. |
| S-SFH SP change bitmap | 3 | Indicate the change of S-SFH SPx IE. The bit#0 to bit#2 is mapped to S-SFH SP1 IE to S-SFH SP3 IE, respectively. Indicates the change of S-SFH SPx IE(s) associated with the S-SFH change count. If bit#0 (LSB) = 1, change in S-SFH SP1 IE. Otherwise no change in SP1. If bit#1 = 1, change in S-SFH SP2 IE. Otherwise no change in SP2 IE. If bit#2 (MSB) = 1, change in S-SFH SP3 IE. Otherwise no change in SP3 IE. |
| Start super-frame offset where new S-SFH SP1 and SP2 information is used | 2 |  |
| Reserved | 4 3 | The reserved bits are for future extension. |
| } |  |  |

S-SFH change cCount

Incremented by one (modulo TBD16) by the ABS whenever any of the values (except MSBs of superframe number in S-SFH SP1 IE) of the S-SFH IEs changes. If the value of this count in a subsequent P-SFH IE remains the same, the AMS can quickly decidemay assume that the S-SFH IEs have not changed and may be able decide to disregard the following S-SFH IEs in this superframe.

S-SFH SP change bitmap

Toggled change bit of S-SFH SP IEs. If any field of S-SFH SP IE is changed (except MSBs of superframe number in S-SFH SP1), the corresponding bit is toggled. A bitmap indicating the S-SFH SPx IE changed in association with the current S-SFH change count. If any field of S-SFH SPx IE is changed (except MSB of Superframe Number in S-SFH SP1), the bit corresponding to the S-SFH SPx IE is set to ‘1’.

- - - - - - - - - - - - - - - - - - - - - - - - - - - End Proposed Text - - - - - - - - - - - - - - - - - - - - - - - - -

[Changes: Adopt the following modifications the Tables 801 ~ 803 in section 16.3.6.5.1.2]

- - - - - - - - - - - - - - - - - - - - - - - - - - - Begin Proposed Text  - - - - - - - - - - - - - - - - - - - - - - - -

Change 3: Remove the row corresponding parameter in line 60, Page 376, Table 801 as follows,

Table 801 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| … |  |  |
| } |  |  |

Change 4: Remove the row corresponding parameter in line 21, Page 379, Table 802 as follows,

Table 802 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| …. |  |  |
| } |  |  |

Change 5: Remove the row corresponding parameter in line 39, Page 380, Table 803 as follows,

Table 803 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Start superframe offset where new SP3 information is used | 2 |  |
| Rate of change of S-SFH SP1 and SP2 SP (1-3) info. | 4 | Minimum duration over which the contents of the S-SFH SP1 and SP2 does not change 0b0000: 16 0b0001: 32 0b0010: 64 0b0001 ~ 0b1111: reserved |
| … |  |  |
| } |  |  |

- - - - - - - - - - - - - - - - - - - - - - - - - - - End Proposed Text - - - - - - - - - - - - - - - - - - - - - - - - -