| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods for the AMS to Send its Battery Status to the ABS in IEEE 802.16m (16.2.22) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Shantidev Mohanty, Nageen Himayat Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “ 16.2.22” |  |
| Abstract | This contribution proposes methods using which AMS in can report its battery status to the ABS in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods for the AMS to Send its Battery Status to the ABS in IEEE 802.16m (16.2.22)

Shantidev Mohanty, Nageen Himayat

- Intel Corporation

- Introduction

Power Management for the Connected Mode is used in D3 by an AMS to report its battery level to the ABS. However, currently no MAC message is specified using which AMS can report its battey level to the ABS.

This comment provides methods for the AMS to send its battery status to the ABS.

- Text proposal for inclusion in the P802.16m/D3

========================== Start of Proposed Text ==============================

[Editor’s Note: Add the following text after line 20 in page 285 in “16.2.22 Power Management for the Connected Mode”]

MAC management message is used by the AMS to send its battery status to the ABS.

========================== End of Proposed Text ==============================