| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications about MAC Transmission and Delivery (16.2.4) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC Youngbin Chang Samsung Electronics | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30b on P802.16m/D3 |  |
| Abstract | The contribution proposes clarifications about MAC transmission and delivery (16.2.4). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications about MAC Transmission and Delivery (16.2.4)

Lei Wang

InterDigital Communications, LLC

- Introduction

There are two issues with the 802.16m/D3 regarding the MAC transmission and delivery that need clarifications:

- Due to the MAC encoding changes introduced in the 802.16m, it is necessary to check and reconfirm the MAC data transmission convention, i.e., transmission orders, specified by section 6.3.3.1 in 802.16-2009,

- The MAC delivery issue also needs a clarification. This issue is not actually clearly addressed in the 802.16-2009. The 802.16m spec shall properly address this issue.

This contribution proposes clarifications regarding MAC transmission and delivery.

- Discussions and Proposed Solutions

For the MAC transmission issue, we propose to start with the text in the 802.16-2009, i.e., copy the current 802.16-2009 MAC transmission convention text to the 802.16m draft, and make necessary modifications.

For the MAC delivery issue, the 802.16-2009 does not actually address it clearly. It only mentions the MAC delivery order issue in subsection 11.13.17.6 ARQ_DELIVERY_IN_ORDER_TLV for ARQ connection. This TLV used in DSx message. It indicates whether or not the order of delivery is preserved, i.e., whether data is to be delivered by the receiving MAC to its client application in the order in which the data was handed off to the originating MAC.

Considering today's all-IP network structure, for the network applications over IP that requires packet in-order delivery, there shall be above-IP mechanisms to ensure the in-order packet delivery to the applications. This makes MAC in-order delivery subject to further discussion and consideration. For sure, MAC in-order delivery will benefit some applications, but it may not needed for all network applications. Therefore, we propose MAC in-order-delivery is a service flow specific feature in 16m MAC, and it is signaled by a service flow parameter, called, MAC in-order-delivery, in the AAI_DSx messages.

- Suggested changes in the 802.16m/D3

The following is the proposed change in the 802.16m/D3. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: page 121, line 1

Insert the following text in line 1 on page 121:

16.2.4.1 Convention

The MAC data shall be transmitted in accordance with the following rules:

- Fields of MAC messages and TLV encodings are transmitted in the same order as they appear in the corresponding tables in this standard.

- Fields of MAC messages and fields of TLV encodings, which are specified in this standard as binary numbers, are transmitted as a sequence of their binary digits, starting from MSB. Bit masks (for example, in ARQ) are considered numerical fields. TLV encodings are transmitted in the order of Type, Length and Value. If the Value of a TLV or a field within the TLVs Value is explicitly specified as a numbered sequence of bits, then the order of transmission shall be from highest sequence number to lowest sequence number. For signed numbers MSB is allocated for the sign. Length field in the “definite form” of ITU-T X.690 is also considered a numerical field.

- Fields specified as SDUs or SDU fragments (for example, MAC PDU payloads) are transmitted in the same order of bytes as received from upper layers.

- Fields specified as strings are transmitted in the order of symbols in the string.

In cases c) and d), bits within a byte are transmitted in the order “MSB first.”

Suggested change #2: page 121, line 1

Change the subsection numbers of 16.2.4.x….. by increasing the 4th subsection number by 1, i.e., x+1.

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”