| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods For Explicit Listening Window Extension by AMS during Sleep Operation in IEEE 802.16m (16.2.2.1.3.4) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Shantidev Mohanty, Maruti Gupta, Muthaiah Venkatachalam Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “ 16.2.2.1.3.4” |  |
| Abstract | This contribution proposes methods using which AMS in sleep mode can ensure that it has up-to-date system information in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods For Explicit Listening Window Extension by AMS during Sleep Operation in IEEE 802.16m (16.2.2.1.3.4)

Shantidev Mohanty, Maruti Gupta, Muthaiah Venkatachalam

- Intel Corporation

- Introduction

Currently the explicit signaling to extend the listening window is sent by the ABS. This comment proposes the AMS to have the ability to use explicit signaling to extend/terminate the listening window.

- Text proposal for inclusion in the P802.16m/D3

========================== Start of Proposed Text ==============================

[Editor’s Note: Modify Table 660 in page 25 in “16.2.2.1.3.4” as follows]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| if(Listening Window End or Extension==0){ | - | - |

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Last frame of Extended Listening Window | 7 | The value is only valid with Listening Window End or Extension is set to 0; LSB of frame sequence. Indicate the frame that extended listening window is terminated. When Listening Window End or Extension is set to 0, this specifies the frame where extended listening window is terminated. When Listening Window End or Extension is set to 1, this specifies the last frame up to which the listening window is extended. |

[Editor’s Note: Modify Table 668 in page 33 in “16.2.2.2.5” as follows]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| if(Listening Window End or Extension==0){ | - | - |

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Last frame of Extended Listening Window | 7 | The value is only valid with Listening Window End or Extension is set to 0; LSB of frame sequence. Indicate the frame that extended listening window is terminated. When Listening Window End or Extension is set to 0, this specifies the frame where extended listening window is terminated. When Listening Window End or Extension is set to 1, this specifies the last frame up to which the listening window is extended. |

========================== End of Proposed Text ==============================