| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods For DCR Mode Operation and Coverage Loss in IEEE 802.16m (16.2.18) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Shantidev Mohanty, Xiangying Yang, Muthaiah Venkatachalam, Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: | Call for LB #30b on “ P802.16m/D3”: Target topic: “ 16.2.18” |  |
| Abstract | This contribution proposes methods using which AMS in sleep mode can ensure that it has up-to-date system information in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 30b. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods For DCR Mode Operation and Coverage Loss in IEEE 802.16m (16.2.18)

Shantidev Mohanty, Xiangying Yang, Muthaiah Venkatachalam

- Intel Corporation

- Introduction

The network entity that keeps the context of an AMS in DCR mode is not specifed in D3. This contribution proposes the network entity that keeps AMS's context when the AMS is in DCR mode.

In addition, this contribution also proposes the use of an ID different than DID to identify AMSs that are in DCR mode. This new ID is also proposed to be used to identify users experiencing coverage loss.

- Text proposal for inclusion in the P802.16m/D3

========================== Start of Proposed Text ==============================

[Editor’s Note: Modify text in Section 16.2.18 starting from line 27 in page 268 and 269 as follows]

16.2.18 Deregistration with content retention (DCR) mode

Deregistration with content retention (DCR) mode is a mode in which an AMS is deregistered from the network while its context is kept in a network entity such as anchor authenticator until the idle mode retention timer is valid.

While the idle mode retention timer is valid, the network retains AMS’s information which is used to expedite AMS’s network reentry. Deregistration identifier (DID) An ID, referred to as NAP MSID is assigned to AMS in DCR mode that is assigned to the AMS during its initial network entry with paging cycle and paging offset value of 0. This NAP MSID is also used to identify the AMS experiencing coverage loss.

DID NAP MSID together with ID of the network entity that assigns/retains the context is used to uniquely identify the DCR mode AMSs.

16.2.18.2 DCR mode initiation from idle mode

AMS may initiate DCR mode in idle mode state by performing the location update with ranging purpose code of 0x05 in AAI_RNG-REQ message for transition to DCR mode.

When the ABS decides to allow AMS’s DCR mode request, the ABS shall send a AAI_RNG-RSP with action code 0x04. When ABS decides to reject AMS’s DCR idle mode request, the ABS shall send an AAI_RNG-RSP with action code 0x05.

Upon successful DCR mode change request, the network shall initiate DCR mode operation by retaining AMS’s information until the idle mode retention timer is valid. At the time of DCR mode change, paging

cycle, paging offset, paging controller values shall be updated to be 0 for the AMS NAP MSID and the ID of the network entity that assigns/retains the context shall be used to uniquely identify DCR mode AMS.

========================== End of Proposed Text ==============================