| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AMS battery level report (16.2.22) |  |
| Date Submitted | 2010-01-05 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: LB30b / Area: Chapter 16.2.22 (Power Management for the Connected Mode) |  |
| Abstract | This contribution proposes changes to the Section 16.2.22 of IEEE P802.16m/D3 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AMS Battery Level Report (16.2.22)

Ming-Hung Tao, Ying-Chuan Hsiao

ITRI

- Introduction

In section 16.2.22/D3 [1], it is said an AMS may report its battery level when the AMS supports the power management in Connected Mode. However, there is no definition for “Connected Mode” in either SDD [2] or D3; there are only definitions for “connected state” and “active mode” in SDD. So we suggest changing the title of section 16.2.22 from “Power Management for the Connected Mode” to “AMS Battery Level Report”, since what should be standardized is the report mechanism/message of the AMS batter level; other details are belong to implementation. The contribution also introduces a new MAC signaling header for the AMS to report its battery level to the ABS.

- Proposed Text for IEEE P802.16m/D3

================================= Remedy 1 =======================================

[Accept the following modifications marked in blue and red from line 34 on page 22.]

#### 16.2.2.1.3 MAC Signaling Header

Table 656—Type field encodings for MAC sig-

naling header type

| Type field (4 bits) | MAC Signaling Header Type |
| --- | --- |
| 0000 | BR with STID |
| 0001 | BR without STID |
| 0010 | Service specific BR without STID |
| 0011 | Sleep Control |
| 0100 | AMS Battery Level Report |
| 01010- 1111 | Reserved |

#### 16.2.2.1.3.5 AMS Battery Level Report Header

The AMS Battery Level Report Header should be used to convey AMS battery level information from AMS to ABS. The specific information provided by AMS Batter Level Report Header is given in Table xxx.

Table xxx—AMS Battery Report header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AMS Batter Level Report header () { | - | - |
| FID | 4 | Flow Identifier. This field indicates MAC signaling header |
| Type | 4 | MAC signaling header type = 0b0100 |
| AMS Battery Level | 3 | 0b000: The AMS is plugged into a power source. 0b001: Battery level is between 75% and 100 % 0b010: Battery level is between 50% and 75 % 0b011: Battery level is between 25% and 50 % 0b100: Battery level is between 5 % and 25 % 0b101: Battery level is below 5 % 0b110 – 0b111: Reserved |
| Reserved | 37 | Shall be filled by 0 |
| } |  |  |

#### 16.2.22 Power Management for the Connected Mode AMS Battery Level Report

Enhanced power savings when an AMS is in connected mode and is actively communicating with an ABS may be supported. In this mode, the ABS may allocate resources, adjust operation modes, and set transmission parameters to optimize energy savings at the AMS.

An AMS may report its battery level when the AMS supports the power management in Connected Mode and its the battery level changes goes below a certain threshold. The AMS shall cancel the previous battery report as soon as its battery level has returned to a certain threshold, or as soon as the AMS is plugged in a charger.

Enhanced power savings when an AMS is in active mode and is actively communicating with an ABS may be supported. In this mode, the ABS may allocate resources, adjust operation modes, and set transmission parameters to optimize energy savings at the AMS.

Power update mechanism as specified in section 8.4.10.3 may be used when an ABS receives an AMS’s battery level report and the ABS supports power management in Connected Active Mode.

References

- IEEE P802.16m/D3. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Dec 2009.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.