| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Emergency Alert Service (16.2.12) |  |
| Date Submitted | 2009-1-8 |  |
| Source(s) | Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai Institute for Information Industry (III) | hcy@iii.org.tw |
| Re: | In response to IEEE 802.16 Working Group Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes text for Emergency Alert Service |  |
| Purpose | For discussion and approval by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Emergency Alert Service (16.2.12)

Chiu-Wen Chen, Chun-Yen Hsu, Kanchei(Ken) Loa, Youn-Tai Lee, Yi-Ting Lin, Tsung-Yu Tsai
Institute for Information Industry (III)

Proposed text for the P80216m/D2:

-----------------------------------------------Start of the Text Proposal # 1---------------------------------------------

[Make the following changes to Section 16.2.12.7 in Page 216 beginning Line 5 in P802.16m/D3.]

#### 16.2.12.7 Emergency Alert Service

Emergency alert service is defined as a service that would provide the public with alerts on imminent emergency events, such as earthquake, storm, etc. The alerts would target subscribers in a specific geographical location.

The emergency alert service includes the transmissions of emergency information and emergency alert indicator. The emergency information is broadcasted through AAI_L2_XFER. The emergency indicator is used to notify AMSs in idle state the reception of emergency information as described in section 16.2.17.2.1.

-----------------------------------------------End of the Text Proposal # 1---------------------------------------------

-----------------------------------------------Start of the Text Proposal # 2---------------------------------------------

[Make the following changes to Section 16.2.3.28 in Page 94 beginning Line 37 in P802.16m/D3.]

#### 16.2.3.28 L2 Transfer message (AAI_L2_XFER)

The enumeration of Tranfer-Type is as follows:

h) Transfer-Type = 8; Emergency Alert

ih) Transfer-Type = 98-127; reserved

ji) Transfer-Type = 128-255; Vendor specific types

-----------------------------------------------End of the Text Proposal # 2---------------------------------------------

[1] IEEE P80216m/D3, “P802.16m/D3: Advanced Air Interface”