| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Clarifications on DL/UL Resource Mapping Signaling (16.3.5&16.3.8) |  |
| Date Submitted | 2010-01-11 |  |
| Source(s) | Yanfeng Guan, Ding Ning, Huiying Fang ZTE Corporation | Voice: E-mail: {guan.yanfeng, ning.ding}@zte.com.cn |
| Re | IEEE 802.16 Working Group Letter Ballot #30a: P802.16m/D3 |  |
| Abstract | This contribution proposes some modifications on Section 16.3.5.2.1&16.3.8.2.1 (Subband Partitioning) for simplifying specification and implementation |  |
| Purpose | For discussion and adoption by IEEE 802.16 TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Clarifications on DL/UL Resource Mapping Signaling

Yanfeng Guan, Ding Ning, Huiying Fang

ZTE Corporation

- Introduction

Because the irregular system bandwidth is supported by tone dropping, the used FFT size can’t reflect the real resource number, the corresponding resource mapping signaling should be more clarifications. For example:  the table 774-776 in section 16.3.5.2.1 and table 867-869 in section 16.3.8.2.1 can not explain the mapping between DSAC/USAC and KSB when the system bandwidth is not 20MHZ, 10MHZ and 5MHZ, since the available number of subband may be less than the some value in the tables. In this contribution, we propose some clarifications mainly on the irregular system bandwidths.

- Reference

[1] IEEE C802.16m-P80216m_D3, “Advanced Air Interface (Draft 3)”

- Text proposal

- [Reply Contribution to Comment B0652]

[Insert below text in line 26 on the page 317, in 16.3.5.2.1 as follows:]

For those system bandwidths in range of, the relation between the system bandwidth and supported number of NPRU is listed in Table 768. The mapping between DSAC and KSB is based on Table 774, the maximum valid value of KSB is.

For those system bandwidths in range of, the relation between the system bandwidth and supported number of NPRU is listed in Table 769. The mapping between DSAC and KSB is based on Table 775, the maximum valid value of KSB is.

[Insert below text in blue in line 26 on the page 327, in 16.3.5.3.1, as follows:]

For those system bandwidths in range of, the mapping between DCASMB,0 and number of miniband-based CRUs for FP0 is based on Table 782, the maximum valid value of LMB-CRU,FP0 is less than floor(88*NPRU/96).

For those system bandwidths in range of, the mapping between DCASMB,0 and number of miniband-based CRUs for FP0 is based on Table 783, the maximum valid value of LMB-CRU,FP0 is floor(42*NPRU/48).

[Insert below text in line 35 on page 491, in 16.3.8.2.1, as follows:]

For those system bandwidths in range of, the relation between the system bandwidth and supported number of NPRU is listed in Table 768. The mapping between USAC and KSB is based on Table 867, the maximum valid value of KSB is.

For those system bandwidths in range of, the relation between the system bandwidth and supported number of NPRU is listed in Table 769. The mapping between USAC and KSB is based on Table 868, the maximum valid value of KSB is.

[Insert below text in line 30 on page 501, in 16.3.8.3.1, as follows:]

For those system bandwidths in range of, the mapping between UCASMB,0 and number of miniband-based CRUs for FP0 is based on Table 875, the maximum valid value of LMB-CRU,FP0 is less than floor(88*NPRU/96).

For those system bandwidths in range of, the mapping between UCASMB,0 and number of miniband-based CRUs for FP0 is based on Table 876, the maximum valid value of LMB-CRU,FP0 is floor(42*NPRU/48).