| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply Comment on #B0516 |  |
| Date Submitted | 2010-01-08 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | LB30b |  |
| Abstract | This contribution provides proposal text of comment #B0516. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply Comment on #B0516

Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

# Text Proposal

----------------------------------------------Start of the text proposal ------------------------------------------------------

Modify paragraph 16.2.15.3 as follows:

16.2.15.3.1 Contention-based initial ranging and automatic adjustments

An AMS that wishes to perform initial ranging shall take the following steps:

•The AMS, after acquiring downlink synchronization and uplink transmission parameters, shall select one Ranging Slot based on the random backoff. The random backoff shall use a binary truncated exponent algorithm. After selecting the Ranging Slot, the AMS shall choose a ranging sequence (from the Initial Ranging domain) using a uniform random process. The AMS shall send the selected ranging sequence to the ABS in the selected Ranging Slot.

•The ABS should respond with an AAI_RNG-ACK message in a predefined, subsequent DL AAI subframe as defined in 16.2.3.3 define in 16.2.3.19. The AAI_RNG-ACK may appear several frames within MAX_RNG_ACK_TX_Time after the frame where the AMS performed ranging. MAX_RNG_ACK_TX_Time is defined as global values. The AAI_RNG-ACK message provides responses to all the successfully received and decoded IR requests in initial ranging slots in a previous UL AAI subframe. If the AMS finds in the RNG-ACK bitmap that no initial ranging sequence has been successfully decoded in the ranging slot selected by the AMS, or it does not find a response in the AAI_RNG-ACK message to its IR access attempt, the AMS considers its initial ranging request is failed and restarts the initial ranging procedure..

Modify paragraph 16.2.3.3 and table 676 as follows:

16.2.3.3 AAI_RNG-ACK

The AAI_RNG-ACK message is sent by the ABS in response to the CDMA ranging request during initial ranging, period ranging and HO ranging in order to provide PHY-level adjustment (e.g. timing offset, power level and frequency offset). Upon reception of ranging codes, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE using broadcast STID. When the AAI_RNG-ACK message is sent unsolicitedly to specific user, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE with unicast STID. If all the ranging statuses of the detected ranging preambles are equal to 'success', the AAI_RNG-ACK may be omitted. The AAI_RNG-ACK shall include the following parameters:

Table 676 – Parameters for AAI_RNG-ACK

| Name | Value | Description |
| --- | --- | --- |
| Management Message Type | AAI_RNG-ACK |  |
| Number of frames (N_Frames) | Indicates the number of frame identifiers included in this RNG-ACK message | The size of this N_Frames is decided by MAX_RNG_ACK_TX_Time. |
| Frame identifier | Indicates the frame which contains the ranging opportunities to which this message refers. The frame identifier is produced by concatenating the following two values: - The TBD least significant bits of the superframe number - The frame index within the superframe (ranging from 0b00 to 0b11). | The size of this Frame identifier is decided by MAX_RNG_ACK_TX_Time. Shall be included for each frame if N_Frames > 0 |
| RNG-ACK Bitmap | The size of this bitmap is decided by the number of ranging slots in the previous referenced frame. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. |  |

-------------------------------------------------- End of the text proposal --------------------------------------------------