| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text to improve A-A-MAP reliability in FFR (16.3.6) |  |
| Date Submitted | 2010-1-11 |  |
| Source(s) | Yi Hsuan, Yuan Zhu, Hujun Yin, Jiacheng Wang Intel Corporation |  |
| Re: | Category: AWD comments / Area: Chapter 16.3.6 (DL-CTRL) “Comments on AWD 16.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text to improve broadcast message coverage |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text to improve A-A-MAP reliability in FFR(16.3.6)

Yi Hsuan, Yuan Zhu, Hujun Yin, Jiacheng Wang

Intel Corporation

- Introduction

The power levels of the frequency reuse 3 partition have been proposed to TGm [1]. We evaluate the A-A-MAP performance given those power levels and propose to use a more robust MCS when A-A-MAP is in the reuse 3 partition of FFR.

- SINR distribution of the reuse 3 partition in FFR and A-A-MAP performance

The following figure is the SINR distribution of the power-boosted reuse 3 for the EMD baseline cell (1500m ISD). PHigh denotes the power boosting value of the power-boosted reuse 3 partition. Plow is the power deboosting level of the power-deboosted reuse 3 partitions. As shown, the 95% SINR of the power-boosted reuse 3 partition ranges from about -3 dB to 2.5 dB depending on the boosting level. The 95% SINR of reuse 1 is about -4 dB.

Figure 1: SINR distribution of the power-boosted reuse 3 partition

For the 5000m radius cell size, the SINR distributions of the FFR reuse 3 partition are shown in Figure 2.

Figure 2: SINR distribution of pure reuse 1 and 3 scenarios for 5000 radius cells

To see if the current A-A-MAP design satisfies the requirement of 1% PER for 95% of the cell, simulation is performed based on the following configurations.

- System Bandwidth: 10 MHz (1024 FFT)

- Carrier Frequency :2.5 GHz

- Number of OFDM symbols per subframe: 5

- Number of Antennas: 2 Tx by 2 Rx

- Antenna correlation: Tx: 0.4, Rx: 0.2

- MIMO mode 2x2 SFBC

- Modulation: QPSK

- Data boosting: 0dB

- Pilot boosting: 5dB

- MIMO receiver: MMSE

- Channel model: eITU-PedB 3 km/h, eITU-VehA 120 km/h

- Channel estimation: 2-D MMSE

- Interference mitigation (IM): On /Off

- Number of interferers: 3

- Interference scenarios: INR=Infinity(1000dB), Interference profile: 0 dB, -3 dB, -6 dB (-6 dB interferer has the same pilot pattern as the serving BS)

Figure 3: Broadcast message PER in eITU PedB channel

Figure 4: Broadcast message PER in eITU VehA channel

As shown in Figure 3 and 4 for A-A-MAP using QPSK ½ the required SIR is about 3 dB. With 8 dB power boosting, it can cover SINR down to -2 dB. This is not sufficient to cover some FFR power boosting levels and provide a safe margin for implementation loss. For A-A-MAP using QPSK ¼, the required SIR is about 0 dB. With 8 dB power boosting, it can cover SINR down to to -5 dB which gives a much better margin relative to the coverage requirement.

In addition, for 5000m radius cell size, about 7 dB boosting is needed for QPSK 1/8 in reuse 1 (with 1 dB implementation loss margin) and more than 8 dB is needed for QPSK ½ in reuse 3. If QPSK ¼ in reuse 3 is allowed, 4 dB boosting is sufficient. To reduce power boosting and improve coverage we propose to allow QPSK 1/4 to be used for A-A-MAP in the power-boosted reuse 3 partition.

- Reference

[1] IEEE C802.16m-09/2741r1, Proposed Cleanup Text for DL FFR (16.2.20.1) for the IEEE 802.16m/D3.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal (16.3.6.3.2.4, page 374, line 41) ]

Only 1/2 or 1/4 code rate is used in the highest-power reuse 3 partition and is signaled in the S-SFH.

- [ Recommended AWD Text Proposal (16.3.6.5.1.2, page 377, line 32) ]

| - A-A-MAP MCS selection | - 1 | - 0b0: QPSK ½ and QPSK ¼ can be used for assignment A-MAP. QPSK ½ is used for A-A-MAP in the power-boosted reuse 3 partition of FFR. - 0b1: QPSK ½ and QPSK 1/8 can be used for assignment A-MAP. QPSK ¼ is used for A-A-MAP in the power-boosted reuse 3 partition of FFR. |
| --- | --- | --- |

-------------------------------------- End of Text Proposal -------------------------------------------