| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to Update of the S-SFH IEs (D3 - 16.2.23) |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | This contribution proposes text for update procedure of the S-SFH IEs. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to Update of the S-SFH IEs (D3 - 16.2.23)

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for update procedure of the S-SFH IEs. In the IEEE 802.16m DRAFT Amendment [1], there is no specific description indicating which S-SFH change count is applied in the current superframe.

- References

[1] IEEE P802.16m/D3, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D3, page 285, line 21, make the following changes to subclause 16.2.23]

16.2.23 Update of S-SFH IEs

The S-SFH subpacket (SP) IEs (i.e., S-SFH SP1 IE, SP 2 IE and SP 3 IE) are transmitted at scheduling periods by the ABS at the scheduled period of each S-SFH SP, respectively, with different periodicity (i.e., TSP1 < TSP2 < TSP3). This SP scheduling periodicity information is transmitted in the S-SFH SP 3 IE. An AMS may implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE. If AMS has been this information, it shall assume that this information is valid until this information is changed.

Every superframe, the ABS transmits P-SFH IE containing the S-SFH Scheduling information bitmap, SCDS-SFH change count, and S-SFH SP change bitmap, and S-SFH transition flag. The S-SFH Scheduling information bitmap indicates S-SFH SP IE(s) which is transmitted in athe current superframe. AMS can implicitly know each S-SFH SP IE's scheduling period by receiving each subsequent S-SFH SP IE. The SCDS-SFH change count shall remain unchanged as long as anyall of the values (except MSB of superframe number in S-SFH SP 1) of the S-SFH SP IE(s) remain unchanged. The SCDS-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1) of S-SFH SP IEs change. The changed S-SFH SP IE(s) shall be transmitted at scheduling superframes the scheduled superframes of period corresponding to each S-SFH SP IE.

Each bit of the S-SFH SP change bitmap indicates the changing status of the corresponding S-SFH SP IE in association with the S-SFH change count. The bit # 0(LSB), bit #1 and bit #2(MSB) are mapped to S-SFH SP1 IE, S-SFH SP 2 IE and S-SFH SP3 IE, respectively. If any of the values (except MSBs of superframe number in S-SFH SP 1) of an S-SFH SP IE are changed, the corresponding bit corresponding to the changed S-SFH SP IE is toggled.

The S-SFH transition flag is used to indicate the changed system parameters in S-SFH are applied.

According to the SCDS-SFH change count, S-SFH Scheduling information bitmap and S-SFH SP change bitmap, AMS previously decides whether it shall decode S-SFH IE in the current superframe to update the system parameters broadcasted within the S-SFH or not.

AMS shall compare values of each SCDS-SFH change count in the last received P-SFH IE and the last stored P-SFH IE whenever it receives P-SFH IE.

a) If there is no difference of two SCDS-SFH change counts, AMS may not decode S-SFH IE in the superframe.

b) Else, AMS shall check that change bits in the S-SFH change bitmap are toggled as the value of the difference.

1) If the number of toggled bits in the last received S-SFH SP change bitmap is the same as the difference, AMS shall update the S-SFH SP IE(s) whose bit of the S-SFH SP change bitmap is toggled. Additionally, AMS shall update SCDS-SFH change count and S-SFH SP change bitmap.

2) Else, AMS shall update all S-SFH SP IEs, SCDS-SFH change count and S-SFH SP change bitmap.

[On the IEEE P802.16m/D3, page 375, line 14, make the following changes to subclause 16.3.6.5.1.1]

16.3.6.5.1.1 P-SFH IE

The P-SFH IE contains essential system information and it is mapped to the P-SFH. The format of the P-SFH IE is shown in Table 799.

Table 799—P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSB of Superframe number | 4 | Part of superframe number |
| S-SFH change count | 4 | Describes the S-SFH SPx IE that apply to this superframe. Indicates the value of S-SFH change count associated with the S-SFH SPx IE(s) transmitted in this superframe |
| S-SFH Size | 4 | The units of LRU |
| S-SFH Number of Repetitions | 2 | Indicate the transmission format (repetition) used for S-SFH. |
| S-SFH Scheduling information bitmatp | 3 | 0b000: no S-SFH If 1st bit#0 (LSB) = 0b1, S-SFH includes SP1 otherwise no SP1. If 2nd bit#1 = 0b1, S-SFH includes SP2 otherwise no SP2. If 3rd bit#2 (MSB) = 0b1, S-SFH includes SP3 otherwise no SP3. |
| S-SFH SP change bitmap | 3 | Indicate the change of S-SFH SPx IEs associated with the S-SFH change count. The bit #0 (LSB) to bit #2(MSB) is mapped to S-SFH SP 1 IE to S-SFH SP3 IE, respectively. |
| S-SFH Transition Flag | 1 | indicates whether the changed system parameters in S-SFH are applied or not. |
| Reserved | 4 | The reserved bits are for future extension. |
| } |  |  |

S-SFH Count

Incremented by one (modulo TBD16) by the ABS whenever any of the values (except MSBs of superframe number in S-SFH SP1) of the S-SFH SP IEs changes. If the value of this count in a subsequent P-SFH IE remains the same, the AMS can quickly decide assumes that the S-SFH IEs have not changed and may be able decides to disregard the following S-SFH IE(s) in this superframe.

S-SFH SP change bitmap

Toggled change bit of S-SFH SP IEs. If any field of S-SFH SP IE is changed (except MSBs of superframe number in S-SFH SP1), the corresponding bit is toggled.

S-SFH Transition Flag

A one bit flag that indicates AMSs the changed system parameters in S-SFH are applied.

[On the IEEE P802.16m/D3, page 377, line 1, make the following changes to subclause 16.3.6.5.1.2]

Table 801 – S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 |  |
| … |  |  |
| } |  |  |

Table 802 – S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 |  |
| …. |  |  |
| } |  |  |

Table 803 – S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Start superframe offset where new SP3 information is used | 2 |  |
| … |  |  |
| } |  |  |

-----------------------------------------------------------End of the Proposal Text ---------------------------------------------------------