| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Recommended resolution for comments on MAC PDU Construction |  |
| Date Submitted | 2010-01-13 |  |
| Source(s) | Anil Agiwal Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b |  |
| Abstract | Recommended resolution for comments on MAC PDU Construction |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D4. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Recommended resolution for comments on MAC PDU Construction

| Comment Number | Commenter | Proposed Resolution | Proposed Remedy/Reject reason |
| --- | --- | --- | --- |
| 266 | Lei | Accept | change the text "signaling headers" to "control info for forming signaling headers" in the figure 387 on page 118 |
| 268 | Anil | Accept Modified | Same resolution as comment # 270 |
| 269 | Joey | Accept | Modify line 12-13 on page 121 as follows: FPEHx and FPEHy in Figure 389 carry the information about the SDU/SDU fragments in the Payload-Xx and Payload-Yy respectively. Also in Figure 389 on page 121 change Payload-Xx Payload-Yy |
| 270 | Lei | Accept | Make the following changes in Figure 388 on page 120: 1. change the box with text " prepare MEH, FPEH" to the box with text "prepare MEH and other extended headers (if any)"; 2. change the box with text "apply AGMH, MEH and FPEH " to the box with text "apply AGMH, MEH, and other extended headers (if any)"; 3. change the box with text " prepare FPEH" to the box with text "prepare extended headers (if any)"; 4. change the box with text "apply AGMH & FPEH " to the box with text "apply AGMH and other extended headers (if any)"; |
| 271 | Lei | TBD | TBD |
| 272 | Joey | Accept | Modify the lines 5-6 on page 121 as follows: If 'n' connections are multiplexed, one MEH and 0 to 'n' FPEH and/or MCEH shall be present in a MAC PDU. |
| 273 | Lei | Accept modified | Same resolution as comment 272 |
| 274 | Lei | Accept | in line 35, Figure 389, on page 121, make the following changes: 1. change "FPEHx" to "FPEHx (if needed)"; 2. change "FPEHy" to "FPEHy (if needed)". |
| 276 | Lei | Accept | Make the following changes in Figure 390 on page 121: 1. change "FPEH / FEH" to "FPEH / MCEH (if needed)"; 2. change "FPEHx / FEHx" to "FPEHx / MCEHx (if needed)"; 3. change "FPEHy / FEHy" to "FPEHy / MCEHy (if needed)"; |
| 277 | Aran | Reject |  |
| 278 | Lei | Accept | Change the paragraph in line 43 on page 122 as follows: For transport connections, the FPEH (as defined in 16.2.2.2.1) shall be present in the MAC PDU with MAC SDU fragments. The FPEH provides the information about the SDU fragment. SN in FPEH is used for sequencing the SDU fragments and Fragmentation control (FC) bits in FPEH, are used to tag the SDU fragments with respect to their position in the parent SDU. |
| 280 | Lei | Accept | Change the first sentence of the paragraph in line 3 on page 123 as follows: For management connections, the MCEH (as defined in 16.2.2.2.2) shall be present in the MAC PDU with MAC control message fragment. The MCEH provides the information about the control message fragment. |
| 281 | Joey | Accept Modified | Modify the lines 5-7, page 123 as follows: SN and Channel ID in MCEH are is used for sequencing the control message fragments. and Fragmentation Control (FC) bits in MCEH, are used by the receiver to tag identify the control message fragments with respect to their position in the parent of a control message. |
| 282 | Lei | Accept | Make the following changes: 1. in section 16.2.4.3.2 on page 123, change all the occurrences of "Channel ID" to "Management Connection Channel ID (MCC ID)"; 2. on page 31, Table 665, change all the occurrences of "Channel ID" to "Management Connection Channel ID (MCC ID)". |
| 283 | Lei | Accept | Change the first sentence of the paragraph in line 21 on page 123 as follows: MAC may pack multiple MAC SDUs of the same connection into a single MAC PDU. |
| 284 | Joey | Accept | Change the line 10 on page 124 as follows a) Packing of Last MAC SDU Fragment, Unfragmented MAC SDUs & First MAC SDU PDU Fragment in a MAC PDUMAC |
| 285 | Joey | Accept | Change line 5 on page 123 as follows SN and Channel ID in MCEH are is used |