| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | PBREH Change |  |
| Date Submitted | 2009-01-12 |  |
| Source(s) | Aran Bergman Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text for PBREH Change. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- PBREH Change

- Aran Bergman, Joey Chou,

- Intel

- Introduction

The format of each FID in the PRBEH should be byte aligned

- Proposed text

---------------------------------------- Start of proposed text ----------------------------------------------

16.2.2.2.8 Piggybacked bandwidth request extended header (PBREH)

Table 671—PBREH formats

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| PBREH() { |  |  |
| Type | 4 | PBREH type |
| Num_Of_PBR | 4 | Number of piggybacked BW request |
| Do For (i = 0 ; i < Num_Of_PBR ; i++) { |  |  |
| FID | 4 | Flow identifier |
| Request type | 1 | 0: aggregate 1: incremental |
| BR size | 19 | Amount of bandwidth requested |
| End | 1 | Indication of more information: 0: another “FID”, Request type”, and “BR size” fields follow 1: no more “FID”, “Request type” or “BR size” fields follow |
| } while (!End) |  |  |
| Reserved | variable | Padding for byte alignment |
| } |  |  |

---------------------------------------- End of proposed text ----------------------------------------------