|  | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Recommendations of MAC header comments |  |
| Date Submitted | 2009-01-13 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution includes Recommendations of MAC header comments. |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC Header comments harmonization from 01/11/2010 meeting

Resolved comments with

| No. | Comment# | Commenters | Comments | Recommendation | Recommended Remedy |
| --- | --- | --- | --- | --- | --- |
| 1 | 73 | Youngbin | MAC header clarification | accept modify | There are three defined MAC header formats. - The first is the Advanced Generic MAC Header that begins each DL and UL MAC PDUs containing either MAC control messages or CS data. The second is the , compact MAC header that begins MAC PDUs of the connections using persistent allocation or group allocation. The third is the and MAC signaling header. These three MAC header formats shall not be used on the same connection. |
| 2 | 74 | Joey | Jumbo MPDU length clarification | Accept modified | This field indicates the length in bytes of MAC PDU including the AGMH and extended header if present. If MLEH presents in a MPDU, Length field indicates 11 LSB of length in byte of MAC PDU. |
| 3 | 81 | Lei | Variable-size MAC Signaling Header Format #3084 | Reject | Different size of signaling header increases the complexity of deconcatenation function at receiver |
| 4 | 83 | Lei | Extended MAC Signaling Header #3085 | Reject | We already have enough types (12) of signaling headers reserved. |
| 5 | 98 | Anil | EH shall not be sent w/o payload | Accept modified | Modify lines 21-22 on page 20 as follows: MAC PDUs shall be of the form illustrated in Figure 385. Each PDU shall begin with a MAC header. The MAC PDU may also contain payload. The header may be followed by one or more extended headers. Extended headers shall not be present in a MAC PDU without payload. The MAC PDU may also contain payload. |
| 6 | 100 | Joey | Clarify EH header format #2725r1 | Reject |  |
| 7 | 72 | Joey | AFP to include MEH connection | Reject | Proposed remedy already present in D3 |
| 8 | 105 | Jaesun | FPEH description clarification | Accept |  |
| 9 | 106 | Joey | 105, 106, 107 | Superseded by 105 |  |
| 10 | 107 | Lei |  | Superseded by 105 |  |
| 11 | 108 | Joey | FPEH usage clarifiation | Reject | Proposed remedy is redundant |
| 12 | 112 | Joey | Increase SSN to 11 bits | Accept |  |
| 13 | 109 | Anil | FPEH length clarification #2746 | Accept modified | Adopt #2746r1 |
| 14 | 111 | Lei | SN clarification | Reject | MAC PDU and ARQ block itself consist of MAC SDUs and/or MAC SDU fragments when packing. The text modification gives more ambiguity if only mentioning the packing case. |
| 15 | 113 | Joey | FPEH length clarification | Superseded by 109 |  |
| 16 | 115 | Youngbin | MCEH channel ID clarification | Accept modified | 16.2.2.2.2, P31 L3 The MCEH shall be used when MAC PDU contains payload from a control connection. When message fragments belonging to two different control messages are being sent, the transmitter shall assign different channel IDs to the MCEH of each MAC PDU. The MCEH format is defined in Table 665. |
| 17 | 117 | Lei | MCEH usage clarification | Reject | Incorrect remedy |
| 18 | 120 | Lei | Change SN size | Reject | Considering Maximum number of HARQ retransmission*Maximum HARQ retransmission delay, 3 bits are too small. |
| 19 | 119 | Youngbin | MEH usage clarification | Reject | CMH case multiplexing is not needed. The 7-bit lenght field of CMH, too short to allow muplexing multiple MAC SDUs/fragments from different connections. |
| 20 | 121 | Joey | MEH position | Accept modified | 16.2.2.2.3 Multiplexing extended header (MEH) The format of MEH is defined in Table 666. The MEH is used when payload from multiple connections associated with the same security association are multiplexed in the same MAC PDU. The AGMH carries the Flow ID corresponding to the payload of the first connection. MEH carries the Flow IDs corresponding to remaining connections. If both MEH and FPEH are present in a MAC PDU, then MEH shall be appear before FPEH. |
| 21 | 122 | Anil | MEH length clarification #2747 | Accept modified | Adopt #2747r1 |
| 22 | 124 | Tsung-Yu Tsai | MEH num flow clarification | Reject | Problem description is not correct |
| 23 | 125 | Lei | FPEH / MCEH indicator | Accept modified | Adopt C80216m-10_0020.doc |
| 24 | 129 | Joey | MEH length clarification | Superseded by 122 |  |
| 25 | 140 | Joey | The format of each FID in the PRBEH is not byte aligned | Accept modified | Adopt C80216m-10_0022r1.doc |
| 26 | 110 | Jaesun | #2752 | Accept | Adopt C80216m-09_2752r2.doc |
| 27 | 103 | Joey | Include MLEH in the table | Accept |  |