| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Recommended MAC Control (Sleep/Idle/DCR) comments resolution (16.2.3) |  |
| Date Submitted | 2009-01-13 |  |
| Source(s) | Shantidev Mohanty Intel Hyunjeong Kang Samsung | shantidev.mohanty@intel.com hyunjeong.kang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0073, IEEE 802.16 Working Group Letter Ballot Recirc #30b / Topic: MAC Control (Sleep/Idle/DCR) (Section 16.2. 3) |  |
| Abstract | This contribution proposes the recommended MAC Control (Sleep/Idle/DCR) comments resolution |  |
| Purpose | To be discussed and adopted to P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Recommended MAC Control (Sleep/Idle/DCR) comments resolution (Section 16.2. 3)

Shantidev Mohanty, Hyunjeong Kang

| LB30b Comment | Recommendation | Resolution |
| --- | --- | --- |
| 169 | Reject | The usage scenario is not clear. |
| 170 | Reject | The usage scenario is not clear. |
| 171 | Reject | The usage scenario is not clear. |
| 172 | Reject | The usage scenario is not clear. |
| 225 | Reject | The usage scenario is not clear. |
| 226 | Accepted-modified | Modify the sentence (from line 29 on page 82) as follows: 0x07: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x01 to allow AMS-initiated idle mode request to allow AMS-initiated idle mode request. This option is valid in response to a AAI_DREG-REQ message with De-registration-request-code= 0x01. 0x08: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to allow retention of the AMS’s connection information 0x09: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to reject retention of the AMS’s connection information. 0x10-0x15: reserved |
| 227 | Accepted-modified | Modify page 82, line 44 as follows: Paging offset (4bits) 0x00: 0 superframes (applied for DCR mode) |
| 228 | Accepted-modified | [Modify Initial Sleep Cycle on page 83, line 29 as follows] Initial_Sleep_Cycle This indicates an assigned duration for the Initial Sleep Cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). The length of Initial Sleep Cycle shall be equal or longer than the default Listening Window. [Modify Initial Sleep Cycle on page 89, line 24 as follows] Initial_Sleep_Cycle This indicates an assigned duration for the initial sleep cycle during which an AMS keeps sleep state in Sleep Mode (measured in frames). The length of Initial Sleep Cycle shall be equal or longer than the default Listening Window. |
| 229 | Rejected (already resolved in TGm WedPM) |  |
| 162 | Superceded by comment #210 | Superceded by comment #210 |
| 163 | Superceded by comment #210 | Superceded by comment #210 |
| 164 | Superceded by comment #210 | Superceded by comment #210 |
| 165 | Superceded by comment #210 | Superceded by comment #210 |
| 567 | Accept-modified | Adopt the text in C802.16m-09/3002r2 |
| 563 | Accepted-modified | Page 268 Line 27: Replace 'idle mode retention timer' with 'Resource Retain Time' Page 268 Line 29: Replace 'idle mode retention timer' with 'Resource Retain Time' Page 268 Line 65: Replace 'idle mode retention timer' with 'Resource Retain Time' Page 269 Line 7: Replace 'idle mode retention timer' with 'Resource Retain Time' Page269 Line24: Replace 'idle mode retention timer' with 'Resource Retain Time' |
| 566 | Accept |  |