| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for Co-Located Coexistence Messages / Parameters with ASN.1 Format |  |
| Date Submitted | 2026-07-31 |  |
| Source(s) | Jing Zhu Joey Chou Intel Corporation | +1-503-2647073 jing.z.zhu@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D3 for IEEE 802.16 WG Letter Ballot #31 |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D4 |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for Co-Located Coexistence Messages / Parameters with ASN.1 Format

Jing Zhu. Joey Chou

Intel Corporation

- Introduction

The contribution proposes the text changes to D4 [1]

- References

[1] IEEE P802.16m/D4

- Proposed Text I

-------------------------------------------------- Begin Proposed Text Change 1-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D4, starting on line 28, page 93. Proposed deletion of text has been shown with red font that has been struck through. New texts are underlined in blue font.>

16.2.3.16 AAI_CLC-REQ (Co-Located Coexistence Request)

The AMS send the AAI_CLC-REQ message to activate, terminate, or reconfigure one or several Type I, Type II, or/and Type III CLC classes. The AAI_CLC-REQ message is sent from the AMS to the ABS on the AMS's Control FID. The AMS may include CLC-INFO CLC Request or/and CLC Report parameter fields.

-------------------------------------------------- End Proposed Text Change 1-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 2-----------------------------------------------

<Note to Editor: Replace Table 691 in IEEE P802.16m/D4 with the following one>

Table 691—AAI_CLC-REQ message

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Request Action | 8 | each bit of the parameter set to 0 or 1 indicates whether AMS requests to activate or deactivate the corresponding CLC class | N/A |
| O | CLC Request | array | See Table 695 | present if AMS requests to activate or reconfigure one or multiple CLC classes, and may include up to 8 CLC request parameter sets |
| O | CLC Report | variable | See Table 698 | present if AMS reports the interference level from its co-located non-802.16 radios |

-------------------------------------------------- End Proposed Text Change 2-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 3-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D4, starting on line 61, page 93. Proposed deletion of text has been shown with red font that has been struck through. New texts are underlined in blue font.>

CLC-INFO

These parameters may be present more than once if AMS wants to include multiple CLC information fields (Table 693).

CLC Request

The parameter sets are present if AMS requests to active or configure one or multiple CLC classes, and may include up to 8 CLC Request parameter sets.

CLC Report

The parameters are present if AMS reports the interference level from its co-located non-802.16 radios

-------------------------------------------------- End Proposed Text Change 3-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 4-----------------------------------------------

<Note to Editor: Replace Table 692 in IEEE P802.16m/D4 with the following one >

Table 692—AAI_CLC-RSP message

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CLC Response | variable | see Table 696 | N/A |

-------------------------------------------------- End Proposed Text Change 4-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 5-----------------------------------------------

<Note to Editor: The following text appears in IEEE P802.16m/D4, starting on line 28, page 94. Proposed deletion of text has been shown with red font that has been struck through. New texts are underlined in blue font.>

16.2.3.18 CLC-INFO

The CLC-INFO parameters are applicable to AAI_REG-RSP, AAI_RNG-REQ, AAI_RNG-RSP, AAI_CLC-REQ, AAI_SBC-REQ, AAI_SBC-RSP messages. The following sets of parameters may be included into the CLC-INFO parameters encodings.

CLC Limit parameters are defined in Table 694, and may be present in AAI_REG-RSP.

CLC Request parameters are defined in Table 695, and may be present in AAI_SBC-REQ and AAI_CLC-REQ.

CLC Response parameters are defined in Table 696, and may be present in AAI_SBC-RSP, AAI_CLC-RSP, and CLC_RNG-RSP.

CLC Start Time parameters are defined in Table 697. They shall be included in CLC Request, and may be included in CLC Response.

CLC Report parameters are defined in Table 698, and may be present in AAI_CLC-REQ.

Table 693—CLC-INFO parameters

| Name | Size (bits) |
| --- | --- |
| Information Type | 8 |
| CLC Limit | variable |
| CLC Request | variable |
| CLC Response | variable |
| CLC Report | variable |

Parameters shall be as follows:

Information Type

This field indicates which of the following fields are included.

CLC Limit

This field is present if (Information Type == 0), and may be used in AAI_REG-RSP message. Table 694 lists all the parameters that are applied.

CLC Request

This field is present if (Information Type == 1), and may be used in AAI_RNG-REQ, AAI_SBC-REQ, or AAI_CLC-REQ messages. Table 695 lists all the parameters that are applied. It may be present more than once if AMS wants to request for multiple CLC classes.

CLC Response

This field is present if (Information Type == 2), and may be used in AAI_RNG-RSP or AAI_SBC-RSP messages. Table 696 lists all the parameters that are applied.

CLC Report

This field is present if (Information Type == 3), and may be used in AAI_CLC-REQ message. Table 698 lists all the parameters that are applied.

-------------------------------------------------- End Proposed Text Change 5-----------------------------------------------

-------------------------------------------------- Begin Proposed Text Change 6-----------------------------------------------

<Note to Editor: Replace Table 694, 695, 696, 697, 698 in IEEE P802.16m/D4 with the following ones >

Table 694—CLC Limit parameters

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Type I Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type I CLC class | N/A |
| M | Type II Indicator | 1 | Setting the bit to 1 indicates the CLC limit information are applicable to Type II CLC class | N/A |
| M | Active Class Limit | 3 | 0: the maximum number of active CLC classes is 8 x (x>0): the maximum number of active CLC classes is x | N/A |
| M | Active Ratio Limit | 6 | x: the maximum CLC active ratio is x/100 | N/A |
| M | Active Interval Limit | 5 | x: the maximum CLC active interval is x x m AAI subframes, where m is the total number of AAI subframes in a AAI frame. | N/A |

Table 695—CLC Request parameters

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CLC Start Time parameters | 8 | see Table 697 | N/A |
| M | Scheduling Impact | 2 | 0b00: both DL and UL allocations are prohibited in CLC active interval 0b01: only DL allocations are prohibited in CLC active interval 0b10: only UL allocations are prohibited in CLC active interval 0b11: reserved | N/A |
| M | Flag | 3 | 0b000: Type I 0b001: Type II subtype 1 0b010: Type II subtype 2 0b011: Type II subtype 3 0b100: Type III Others: reserved | N/A |
| O | Start AAI subframe Index | 3 | the subframe index of the first subframe of the CLC active interval | present if (Flag == 0b000) or (Flag == 0b010) |
| O | CLC active interval of Type I CLC class | 8 | the number of AAI subframes of the CLC active interval | present if (Flag == 0b000) |
| O | CLC active cycle of Type I CLC class | 21 | the number of microseconds of the CLC active cycle | present if (Flag == 0b000) |
| O | CLC active bitmap of Type II CLC class with subtype 1 | 8 | Setting a bit of the field to “1” indicates the corresponding AAI subframe in each frame is in CLC active interval | present if (Flag == 0b001) |
| O | CLC active interval of Type II CLC class with subtype 2 | 8 | the number of AAI subframes of CLC active interval | present if (Flag == 0b010) |
| O | CLC active cycle of Type II CLC class with subtype 2 | 8 | the number of AAI frames of the CLC active cycle | present if (Flag == 0b010) |
| O | CLC active cycle of Type II CLC class with subtype 3 | 2 | x (0, 1 or 2): the length of Extended CLC active bitmap of Type II CLC class with subtype 3 is x + 2 | present if (Flag == 0b011) |
| O | Extended CLC active bitmap of Type II CLC class with subtype 3 | variable | Setting a bit of the field to “1” indicates the corresponding AAI subframe in each CLC active cycle is in CLC active interval. If the CLC active cycle is x frames, and a frame consists of m AAI subframes, the length of the field shall be x x m. | present if (Flag == 0b011) |
| O | CLC active interval of Type III CLC class | 8 | the number of superframes of the CLC active interval | present if (Flag == 0b100) |

Table 696—CLC Response parameters

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Confirmed Action | 8 | Each bit of the parameter set to 0 or 1 indicates whether the corresponding CLC class is inactive or active | N/A |
| O | CLC Start Time parameters | array | see Table 697 | present for each Type II or III CLC class that ABS configures the starting time different from what is recommended by AMS, and may include up to 8 CLC Start Time parameter sets |

Table 697—CLC Start Time parameters

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CLC ID | 3 | 0~7 | N/A |
| M | Start Superframe Number | 3 | the 3 LSB of the superframe number of CLC start time | N/A |
| M | Start Frame Index | 2 | the frame index of CLC start time | N/A |

Table 698—CLC Report parameters

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Report Type | 8 | 0: Interference Level Others: reserved | N/A |
| O | Interference Level | 8 | signed integer: -127 ~ 127 | present if (Report Type == 0) |

-------------------------------------------------- End Proposed Text Change 6-----------------------------------------------

- Proposed Text II

Annex P.

(Normative)

Definition of AAI MAC control messages

Annex P.2 MAC Control Message Definitions

-------------------------------------------------- Begin Proposed Text Change 7-----------------------------------------------

[Add the following ASN.1 code to Annex P.2]

-- Co-Located Coexistence REQ Message

AAI-CLC-REQ ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

requestAction                  ClcAction,

clcRequest                     ClcRequestArray OPTIONAL,

clcReport                      ClcReportSet OPTIONAL

}

-- Co-Located Coexistence RSP Message

AAI-CLC-RSP ::= SEQUENCE {

messageType                    OCTET STRING (SIZE(1)),

clcResponse                    ClcResponseSet

}

ClcRequestArray ::= SEQUENCE {

clcRequest1                    ClcRequestSet OPTIONAL,

clcRequest2                    ClcRequestSet OPTIONAL,

clcRequest3                    ClcRequestSet OPTIONAL,

clcRequest4                    ClcRequestSet OPTIONAL,

clcRequest5                    ClcRequestSet OPTIONAL,

clcRequest6                    ClcRequestSet OPTIONAL,

clcRequest7                    ClcRequestSet OPTIONAL,

clcRequest8                    ClcRequestSet OPTIONAL}

ClcRequestSet ::= SEQUENCE {

clcStartTime                   ClcStartTimeSet,

schedulingImpact               ENUMERATED {

dlUlProhibited (0),

dlProhibited   (1),

ulProhibited   (2)},

clcType                        ENUMERATED {

typeI          (0),

typeIIsubtype1 (1),

typeIIsubtype2 (2),

typeIIsubtype3 (3),

typeIII        (1)},

startAaiSubframeIndex          INTEGER (0..7) OPTIONAL,

clcActIntervalTypeI            INTEGER (0..255) OPTIONAL,

clcActCycleTypeI               INTEGER (0..2097151) OPTIONAL,

clcActBitmapTypeIIsubTyp1      INTEGER (0..255) OPTIONAL,

clcActIntervalTypeIIsubTyp2    INTEGER (0..255) OPTIONAL,

clcActCycleTypeIIsubTyp2       INTEGER (0..255) OPTIONAL,

clcActCycleTypeIIsubTyp3       INTEGER (0..2) OPTIONAL,

clcActBitmapTypeIIsubTyp3      INTEGER (0..4294967295) OPTIONAL,

clcActIntervalTypeIII          INTEGER (0..255) OPTIONAL

}

ClcResponseSet ::= SEQUENCE {

confirmedAction                ClcAction,

clcStartTime                   ClcStartTimeArray OPTIONAL}

ClcReportSet ::= SEQUENCE {

reportType                     INTEGER (0..255),

interferencelevel              INTEGER (-127..127)}

ClcStartTimeArray ::= SEQUENCE {

clcStartTime1                  ClcStartTimeSet OPTIONAL,

clcStartTime2                  ClcStartTimeSet OPTIONAL,

clcStartTime3                  ClcStartTimeSet OPTIONAL,

clcStartTime4                  ClcStartTimeSet OPTIONAL,

clcStartTime5                  ClcStartTimeSet OPTIONAL,

clcStartTime6                  ClcStartTimeSet OPTIONAL,

clcStartTime7                  ClcStartTimeSet OPTIONAL,

clcStartTime8                  ClcStartTimeSet OPTIONAL}

ClcStartTimeSet ::=                SEQUENCE {

clcId                          INTEGER (0..7),

startSfNumber                  INTEGER (0..7),

startFrameIndex                INTEGER (0..3)}

ClcAction ::=                      BIT STRING {

clcClass0 (0),

clcClass1 (1),

clcClass2 (2),

clcClass3 (3),

clcClass4 (4),

clcClass5 (5),

clcClass6 (6),

clcClass7 (7)} (SIZE(8))

-------------------------------------------------- End Proposed Text Change 7-----------------------------------------------