| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Extended headers for ARQ operation (16.2.2.2) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot Recirc #31 / Topic: Header Format (Section 16.2.2.2) |  |
| Abstract | This contribution proposes RFPEH and APEH |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D5. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Extended headers for ARQ operation

Youngbin Chang, Anil Agiwal, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

The FPEH format as described in P802.16m/D4 [1] has several fields which need for ARQ operation, e.g AFP, RI, LSI and SSN. To reduce the FPEH overhead, especially fragmentation case (which is the most usual case), ARQ information in FPEH should be moved to another place. In this contribution, we removed the ARQ related fields (AFP, RI, LSI, SSN) and proposed new extended header for supporting ARQ re-arrangement and ARQ feedback polling operation. In the new header, same fields in FPEH is used, there is no overhead increasing.

# Proposed text

[Insert the new section at line 14 on page 60:]

16.2.2.2.x Rearrangement fragmentation and packing extended header (RFPEH)

The RFPEH shall be used for ARQ enabled transport connection. It shall be included in MAC PDU with AGMH if MAC PDU payload contains ARQ sub-block which is constructed from SDU(s) or SDU fragment(s) or both are packed. The RFPEH format is defined in table xxx.

Table xxx: RFPEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| RFPEH (){ |  |  |
| Type | 4 | RFPEH Type |
| FC | 2 | Fragmentation Control bits (see Table 664) |
| SN | 10 | SN is maintained per connection. 'SN' represents the ARQ block sequence number and the 'SN' value increments by one (modulo 1024) for each MAC PDU. |
| SSN | 11 | SUB-SN of the first ARQ sub-block |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| PI | 1 | Packing Indicator 0 = packing information is not present in RFPEH 1 = packing information is present in RFPEH |
| Reserved | 3 | For byte alignment |
| If (PI == 1) { |  |  |
| Do { |  |  |
| Length | 11 | This field indicates the length of MAC SDU or MAC SDU fragment in a MAC PDU payload. If a MAC PDU payload consists of 'N' MAC SDU/ MAC SDU fragments, N-1 'Length' fields are present to represent the length of MAC SDU/ MAC SDU fragment #1 to #N-1. No multiplexing: Length of last MAC SDU/ MAC SDU fragment = Length of MAC PDU (given by length field in AGMH/CMH & MLEH) - Length of AGMH /CMH - Extended header group length (if present) - Length of PN& EKS (3bytes, if present) - Length of ICV ( if present) - sum of length of MAC SDU/ MAC SDU fragment s #1 to #N-1 With multiplexing: Length of last MAC SDU/ MAC SDU fragment = Length of connection payload corresponding to this FPEH (see table 666, MEH format) - sum of length of MAC SDU/ MAC SDU fragments #1 to #N-1 |
| End | 1 | Indication of more information 0 = Indicating another 'Length' and 'End' fields are followed 1 = Indicating no more 'Length' and 'End' fields are followed |
| }while (!End) |  |  |
| } |  |  |
| Padding | variable | For byte alignment |
| } |  |  |

16.2.2.2.x+1 ARQ feedback polling extended header (APEH)

The format of ARQ feedback extended header is defined in Table xxx+1. This header shall be used by an ARQ transmitter to poll ARQ feedback.

Table xxx+1: APEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| APEH (){ |  |  |
| Type | 4 | APEH Type |
| Flow ID | 4 | Reference flow ID for ARQ feedback polling |

# Reference

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”