| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ Feedback IE modification (16.2.13.2.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot Recirc #31 / Topic: ARQ feedback IE modification (Section 16.2.13.2.1) |  |
| Abstract | This contribution proposes the correction to ARQ feedback IE for byte/nibble alignment and some text correction |  |
| Purpose | To be discussed and adopted to P802.16m/D5 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ Feedback IE modification

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Problem Description

- The location of padding is not correct as ARQ feedback IE can be part of extended header. So padding location needs to be changed.

- The do loop in the IE is not octet or nibble aligned. A reserved bit needs to be added. Padding bits are also required before the do loop so that it starts at octet boundary.

- Text Clarification for the definition of ‘0’ bit in selective ACK MAP based on NSI bit is missing

- The order of NACK ARQ blocks in the SEM field is not defined

- NAK needs to be changed to NACK

# Proposed Remedy

See proposed text.

# Proposed text

[Modify the table 760 on page 270-271 as follows:]

Table 760—ARQ feedback IE format for ARQ block

| Syntax | Size(bits) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST) { | variable |  |
| Last | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| Flow ID | 4 | The ID of the flow being referenced |
| FLAG | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG = 0, ARQ block up to and including SN has been received successfully. FLAG = 1, ARQ block less than SN has been received successfully. |
| if(FLAG==1){ |  |  |
| NSI | 1 | NACK Suspended Indicator 0 = Bit marked ‘0’ in the following Selective ACK MAP represents a ARQ block NACK 1 = Bit marked ‘0’ in the following Selective ACK MAP represents that ARQ block NACK decision is suspended (i.e. ARQ_ERROR_DETECTION_TIMER is running for corresponding ARQ block). |
| EXT | 1 | Extension for ARQ sub-block indication 0= No ARQ sub-block ACK/NACK indication 1= ARQ sub-block ACK/NACK indication follows |
| Selective ACK MAP | 5 | Each bit represents ACK or NACK or NACK Suspended of corresponding ARQ block. '0' is NACK if NSI equals zero, ‘0’ is NACK suspended if NSI equals one, and '1' is ACK. Selective ACK MAP represents ACK or NAK information of ARQ blocks after first selective ACK MAP. |
| FLAG | 1 | 0 = No more selective ACK MAP and FLAG 1 = Another set of selective ACK MAP and FLAG follows |
| while(FLAG==1){ |  |  |
| Selective ACK MAP | 7 | Each bit represents ACK or NACK or NACK Suspended of corresponding ARQ block. '0' is NACK if NSI equals zero, ‘0’ is NACK suspended if NSI equals one, and '1' is ACK. Selective ACK MAP represents ACK or NAK information of ARQ blocks after first selective ACK MAP. |
| FLAG | 1 | 0 = No more selective ACK MAP and FLAG1 = Another set of selective ACK MAP and FLAG follows |
| } |  |  |
| if(EXT==1){ |  |  |
| SEM | variable | Sub-block-Existence Map The numbers of SEM bits are same as NACKed ARQ blocks in the Selective ACK MAP. Each bit represents the existence of partially nacked ARQ blocks or completely nacked ARQ blocks. The most significant bit corresponds to the first NACKed ARQ block in the Selective ACK MAP and the least significant bit corresponds to the last NACKed ARQ block in the Selective ACK MAP. 1 = Partially nacked ARQ block (the ARQ block has some sub-blocks which have received correctly) 0 = Completely nacked ARQ blocks( the ARQ block has no sub-blocks which have received correctly) |
| Padding | Variable | For byte alignment of SEM field |
| For(i=0;i<Num_SN<i++){ |  | Num_SN is the number of bits equal to 1 in the SEM field |
| do{ |  | If NSI bit set to ‘0’, all ARQ sub-blocks that are not listed the Start_SSN and Num_SSN in ‘do loop’ are considered as NACK. If NSI bit set to ‘1’, All ARQ sub-blocks that are not listed the Start_SSN and Num_SSN in ‘do loop’ are considered that ARQ sub-block NACK decision is suspended. |
| START_SSN | 11 | Start of ARQ sub-block SN which was received correctly. |
| Num_SSN | 11 | Number of consecutive ARQ sub-blocks which were received correctly from START_SSN onwards |
| End | 1 | 0 = One more set of START_SSN, Length and End fields 1 = There are no more START_SSN, NUM_SSN and End fields |
| Reserved | 1 |  |
| }while(End==0) |  |  |
| } |  |  |
| Padding | variable | For Byte alignment |
| } |  |  |
| } |  |  |
| Padding | variable | For byte alignment |
| } |  |  |