| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference Reduction of CSG-Closed Femto ABS (16.4.11) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yung-Han Chen, Yan-Xiu Zheng, Chun-Yuan Chiu ITRI | chenyunghan@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution provides a method to reduce the interference from a CSG-Closed Femto ABS to a nearby non-member AMS by using low duty mode. |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference Reduction of CSG-Closed Femto ABS (16.4.11)

Yung-Han Chen, Yan-Xiu Zheng, Chun-Yuan Chiu

ITRI

1. Introduction

A non-member AMS interfered by a nearby CSG-Closed Femto ABS may send AAI_RNG-REQ with Femto Interference bit set to 1. Upon receiving the Femto Interference bit=1, the CSG-Closed Femto may use Low Duty Mode (LDM) to reduce the interference.

If there are no AMSs attached to the CSG-Closed Femto ABS and there are no AMSs in network (re)entry process, the CSG-Closed Femto ABS may directly enter LDM.

If there are connected AMSs served by the CSG-Closed Femto ABS, the ABS may command these AMSs to hand over to overlaid macro ABS or another accessible neighboring Femto ABS before it enters LDM for interference reduction.

A CSG-Closed Femto ABS may configure a time interval for its LDM due to receiving Femto Interference bit=1. In order to prevent any access or network entry trial during the time interval of LDM, the CSG-Closed Femto ABS may set Cell Bar to 1. Once the time interval of LDM ends, the CSG-Closed Femto ABS resumes the normal mode operation. The Cell Bar bit may be reset to 0 right after the end of LDM or several (configurable) superframes after the end of LDM. If the CSG-Closed Femto ABS receives Femto Interference bit=1 again, it may take any interference reduction method mentioned in 16.4.11 again.

2. Text Proposal

---------------------------------------------------------- Text Start ----------------------------------------------------------------

16.4.11 Interference Avoidance and Interference Mitigation

{ Modify texts from line 47, page 704 as follows}

Upon CSG-Closed Femto ABS receiving an AAI_RNG-REQ with the "Femto Interference" bit set to 1, it may either reduce its transmit power locally, or communicate with the network entity, or operate in LDM.

Upon communication with the network entity, the CSG-Closed Femto ABS may convert to a CSG-Open Femto ABS or reduce its transmit power as directed by the network entity or refrain from transmitting on certain resource regions as indicated by the network entity.

Upon receiving an AAI_RNG-REQ with the “Femto Interference” bit set to 1, the CSG-Closed Femto ABS may operate in LDM for a time interval to reduce interference. If there are no AMSs attached to the CSG-Closed Femto ABS and there are no AMSs in network (re)entry process, the CSG-Closed Femto ABS may directly enter LDM. If there are connected AMSs served by the CSG-Closed Femto ABS, it may command these AMSs to hand over to the overlaid macro ABS and then enter LDM for interference reduction. To prevent any access or network entry trials during the time interval of LDM, the CSG-Closed Femto ABS may set Cell Bar bit to 1. Once the time interval of LDM ends, the CSG-Closed Femto ABS returns the normal mode operation. The Cell Bar bit may be reset to 0 right after the end of LDM or several superframes after the end of LDM.

---------------------------------------------------------------Text End ------------------------------------------------------------