| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Comment on AAI-NBR-ADV (16.2.3.12) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Yang Liu, Nan Li, Yuqin Chen, Feng Xie，Lin Chen，Mary Chion, Hongyun Qu, Lei Zhang ZTE Corporation Daniel Cohn Alvarion Chunmei Liu, Masoud Olfat Clearwire | E-mail: liu.yang8@zte.com.cn daniel.cohn@alvarion.com Chunmei.liu@clearwire.com, masoud.olfat@clearwire.com |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.2.3.12” |  |
| Abstract | This contribution proposes the texts for change indication in fragment of AAI_NBR-ADV. |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Comment on AAI-NBR-ADV (16.2.3.12)

Yang Liu, Nan Li, Yuqin Chen, Feng Xie, Lin Chen，Mary Chion, Hongyun Qu, Lei Zhang

ZTE Corporation

# Introduction

In the current P802.16m/D3, an indication is defined for the purpose below (p.174), “ …and an ABS shall broadcast the existence of its neighboring WirelessMAN-OFDMA BS and LZone of ABS in its MZone using AAI_NBR-ADV. An AMS by reading the indication of existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS in AAI_NBR-ADV, always obtains the neighbor WirelessMAN-OFDMA BS information from the MOB_NBR-ADV message transmitted in its serving ABS LZone.”

However, the indication is not defined in AAI_NBR-ADV.

In addition, it is noticed that the indication should be better not impact the change count for AAI_NBR-ADV.

We propose to include the indication as (NBR-ADV period, NBR-ADV offset, NBR-ADV interval). For instance, if the MOB NBR-ADV message is broadcasted in a period of 1000 frames, the MOB NBR-ADV offset is 8 frames, and the MOB NBR-ADV will be transmitted in 10 frames (including all the fragments), the AMS will has to listen from the frame 1000*N+8 to 1000*N+8+10, (N=0,1,2…..).

# Text Proposal

======================== Start of Proposed Text 1=============================

Change text in P83,Line 55 as follows:

1)  ABS type (macro, micro, macro hotzone, Femto, relay, legacy)

a)  2) physical carrier index referring AAI_Global-Config message which provides carrier frequency, BW, CP info, TDD/FDD and related definitions (expected to be the same given carrier frequency)

a)  3) MAC version

4) NBR-ADV period: The period of MOB_NVR-ADV is sent in LZone. Unit: Frame.

5) NBR-ADV offset: The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame.

6) NBR-ADV interval: The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame.

The parameters NBR-ADV period, NBR-ADV offset, and NBR-ADV interval indicate the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS. It shall be included when ABS indicates the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS in the AAI_NBR-ADV message.

======================== Start of Proposed Text 2============================

16.2.3.12 AAI_NBR-ADV

Table 684—AAI_NBR-ADV message format

| Syntax | Size (bit) | Note |
| --- | --- | --- |
| AAI_NBR-ADV_Message_format() { | — | — |
| Management Message Type = NN | 8 | — |
| Change Count | 3 | NBR-ADV change count |
| NBR-ADV period | 10 | The period of MOB_NVR-ADV is sent in LZone. Unit: Frame. |
| NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. |
| NBR-ADV interval: | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. |
| Number of cell types | 3 |  |
| Cell type in this message | 3 |  |
| … | … | ... |

======================== Start of Proposed Text 3=============================

Change text from P220, Line 3 as follows:

16.2.6.4.2.1 Network Topology Acquisition

16.2.6.4.2.1.1 Network Topology Advertisement

In the WirelessMAN-OFDMA Reference System/WirelessMAN-OFDMA Advanced co-existing System, an ABS shall broadcast the system information of its neighboring WirelessMAN-OFDMA BS and LZone of ABS in its LZone using MOB_NBR-ADV message, and an ABS shall broadcast the existence of its neighboringWirelessMAN-OFDMA BS and LZone of ABS in its MZone using AAI_NBR-ADV. An AMS by reading the indication of existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS in AAI_NBR-ADV, always obtains the neighbor WirelessMAN-OFDMA BS information from the MOB_NBR-ADV message transmitted in its serving ABS LZone. The indication should be the existence of MOB NBR-ADV period, MOB NBR-ADV offset , and MOB NBR-ADV interval contained in AAI_MOB-ADV. For instance, if the MOB NBR-ADV message is broadcasted in a period of 1000 frames, the MOB NBR-ADV offset is 8 frames, and the MOB NBR-ADV will be transmitted in 10 frames (including all the fragments), the AMS will has to listen from the frame 1000*N+8 to 1000*N+8+10, (N=0,1,2…..). The AMS’s operation in MZone should not be impacted by the switch for MOB_NBR-ADV in LZone.

======================== End of Proposed Text 3=============================