| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference mitigation for femtocell (16.4.11) |  |
| Date Submitted | 2010/3/16 |  |
| Source(s) | Kanchei(Ken) Loa, Yi-Ting Lin, Tsung-Yu Tsai, Yung-Lan Tseng, Chun-Yen Hsu, Chiu-Wen Chen, Chun-Che Chien, Youn-Tai Lee Institute for Information Industry (III) Jong-Kae (JK) Fwu, Hujun Yin, Yi Hsuan Intel Corporation Whai-En Chen National Ilan University (NIU) Shiann-Tsong Sheu National Central University (NCU) Yih Guang Jan, Yang-Han Lee Tamkang University (TKU) I.K. Fu, Pei-kai Liao, Paul Cheng MediaTek Inc. Mihyun Lee, Hokyu Choi, Hyunkyu Yu Samsung Electronics Linghang Fan NEC Zheng Yan-Xiu, Chung-Lien Ho, Chang Lan Tsai, Yu-Chuan Fang, Yung-Han Chen ITRI Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu ZTE | loa@iii.org.tw hcy@iii.org.tw jong-kae.fwu@intel.com wechen@niu.edu.tw stsheu@ce.ncu.edu.tw yihjan@yahoo.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D4 |  |
| Abstract | The contribution proposes the interference mitigation scheme for femtocell |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference mitigation for femtocell

Kanchei(Ken) Loa, Yi-Ting Lin, Tsung-Yu Tsai, Yung-Lan Tseng, Chun-Yen Hsu, Chiu-Wen Chen, Chun-Che Chien, Youn-Tai Lee

Institute for Information Industry (III)

Jong-Kae (JK) Fwu, Hujun Yin, Yi Hsuan

Intel Corporation

Whai-En Chen, Shih-Yuan Cheng

National Ilan University (NIU)

Shiann-Tsong Sheu, Chih-Cheng Yang

National Central University (NCU)

Yih Guang Jan, Yang-Han Lee

Tamkang University (TKU)

I.K. Fu, Pei-kai Liao, Paul Cheng

MediaTek Inc.

Mihyun Lee, Hokyu Choi, Hyunkyu Yu

Samsung Electronics

Linghang Fan

NEC

Zheng Yan-Xiu, Chung-Lien Ho, Chang Lan Tsai, Yu-Chuan Fang, Yung-Han Chen

ITRI

Feng Xie, Lin Chen, Zhaohua Lu, Yang Liu

ZTE

- Introduction

Since the femtocell is not deployed regularly as macrocell, current frequency partition based interference mitigation scheme may be less efficiency and limits the flexibility of interference avoidance especially when the number of Femto ABSs is large. An updated IM mechanism is proposed for femtocell to improve the IM performance.

- Interference mitigation for femtocell

In our interference mitigation mechanism, Femto BS may select better location of the A-MAP region under different scenarios (femtocell startup, reuse-1 vs. power boosted reuse-3, and Femto reconfiguration). The main purpose of the design is that whatever IM schemes or algorithms are adopted in the system (e.g. TDM-based, FDM-based), the negotiation or allocation of the usable resource for Femto ABSs actually can be done in the backhaul, and the information needs to be transmitted to the AMSs is the result of data scheduling, that is, A-MAP. Thus, it indeed needs some mechanisms to protect the A-MAP from interference, and the detail of IM mechanism is done in the backhaul and basically transparent to the AMSs (of course, AMS could transmit measurement report to the serving ABS to assist IM). To do this, we propose that Femto ABS can be semi-statically reconfigured to a different CellID based on interference condition (e.g. by SON server) in order to change its power boosted reuse 3 partition. Therefore, the A-MAP can be changed to another frequency partition with less interference.

- Proposed changes for 802.16m draft amendment

[Add the following text in the section 16.4.11 on page 704 (line 54) as shown below]

The Femto ABS may scan and measure the interference level from neighboring femtocells at startup time, then be configured with the desired IDcell by network entity, e.g. SON server with the best interference condition to transmit A-MAP in corresponding frequency partitions. When the A-MAP is transmitted by a Femto ABS, it follows the same rule as macro ABSs.

Based on interference condition, Femto ABS may switch primary frequency partition between reuse 1 partition and power boosted reuse 3 partition.

Femto ABS may be semi-statically reconfigured to a different IDcell by network entity, e.g. SON server based on interference condition (e.g. via entering the initialization state) in order to change its power boosted reuse 3 partition.