| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Modulation and Coding Scheme Configuration for Enhanced Multicast Broadcast Service in IEEE 802.16m/D4 (16.7.3) |  |
| Date Submitted | 2010-3-4 |  |
| Source(s) | Zheng Yan-Xiu, Yu-Chuan Fang, Chun-Yuan Chiu, Chang-Lan Tsai, Chung-Lien Ho, Yung-Han Chen ITRI | E-mail: zhengyanxiu@itri.org.tw |
| Re: | Call for LB #31 on “ P802.16m/D4”: Target topic: “16.7.3” |  |
| Abstract | This contribution proposes amendment text describing MCS configuration for enhanced multicast broadcast service in IEEE 802.16m |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#31. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Modulation and Coding Scheme Configuration for Enhanced Multicast Broadcast Service in IEEE 802.16m/D4 (16.7.3)

Zheng Yan-Xiu, Chang-Lan Tsai, Yu-Chuan Fang, Chung-Lien Ho

ITRI

- Introduction

This contribution proposes amendment text describing the modulation and coding scheme (MCS) configuration for enhanced multicast broadcast service (E-MBS) in IEEE 802.16m. In an E-MBS network, the location and channel quality of an AMS is unknown. However, AMSs can move within the network and the channel quality of the AMS becomes unknown. In order to maintain the link reliability, the lowest modulation and coding scheme (MCS) is often used but significantly degrades bandwidth efficiency. In order to improve the bandwidth efficiency, this contribution provides an MCS configuration procedure to maintain the reception performance and E-MBS network could serve based the chosen MCS.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D4, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D4

===============   Start of Proposed Text ==========================

[Note to Editor: Add the following text after line 60 in Page 740 in 16.7.3]

- 16.7.3.3 Support of E-MBS

An SON server could configure the modulation and coding scheme (MCS) for E-MBS. The server will invite AMSs to measure the signal strength for one E-MBS zone configuration. The reference signal shall be E-MBS Zone Specific pilot patterns in 16.3.5.4.4. These AMS could report RSSI, CINR, location, etc. The server will collect these reports to calculate on which MCS almost all AMSs could receive E-MBS service with adequate quality. Once the samples are large enough, the MCS configuration would approach the optimum MCS setting.

=================== End of Proposed Text =======================