| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reuse factor for CINR calculation for scan reports (16.2.3) |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Daniel Cohn Alvarion Jia Lin Huawei | daniel.cohn@alvarion.com linjia@huawei.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Define the reuse factor to be assumed for CINR calculations for scan report purposes |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reuse factor for CINR calculation for scan report purposes (16.2.3)

Daniel Cohn
Alvarion

Jia Lin
Huawei

Problem Statement

802.16m/D4 supports AMS reporting signal metrics to its serving ABS. These reports are typically used by the ABS to trigger ABS-initiated handover process, e.g. as a response to AMS changes in position (mobility-triggered handover) or to better distribute the load among ABSs (load-triggered handover). In particular CINR metrics allows the ABS to estimate spectral efficiency of the AMS when served by different ABSs.

802.16m support flexible reuse schemes that can coexist in the same ABS (aka fractional frequency reuse or FFR) so there is a need for CINR reports from the AMS to reflect interference conditions in the different reuse schemes.

In addition, RSSI and CINR descriptions in the SCN-REP message use 802.16e concepts.

Suggested Solution

It is suggested that SCN-REP messages will include CINR metrics reflecting reuse 1 and reuse 3 conditions, when CINR measurements are requested by the ABS.

While only reuse-1 and reuse-3 CINR metrics can be derived from A-Preamble readings, they can be a useful reference even when other reuse schemes are used.

It is also suggested that RSSI and CINR descriptions in the SCN-REP message be updated to reflect changes in 802.16m.

Suggested Standard Modifications

1.- Modify table 690 in section 16.2.3.15, as follows:

| Name | Value | Usage |
| --- | --- | --- |
| BS reuse-3 CINR mean | The BS CINR Mean parameter indicates the CINR measured by the AMS from the particular ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame pSA-Preamble that are active intransmitted by the serving ABS's segment and averaged as specified in equation (1). When SCN-REP is sent in response to trigger, the trigger averaging parameter is taken from the relevant trigger. Otherwise, it is taken from the AAI_SCD messageover the measurement period. | Included when Report metric Bit 0 = 1 |
| BS reuse-1 CINR mean | The BS CINR Mean parameter indicates the CINR measured by the AMS from the particular ABS. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the all the subcarriers of the frame SA-Preamble and averaged as specified in equation (1). When SCN-REP is sent in response to trigger, the trigger averaging parameter is taken from the relevant trigger. Otherwise, it is taken from the AAI_SCD message | Included when Report metric Bit 0 = 1 |
| BS RSSI mean | The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the particular ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. The measurement shall be performed on the frame pSA-Preamble and averaged as specified in equation (1), using the trigger averaging parameter advertised in the AAI_SCD messageover the measurement period. | Included when Report metric Bit 1 = 1 |