| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reuse factor for CINR calculation for scan reports (16.2.3) |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Daniel Cohn, Ariel Doubchak Alvarion Jia Lin Huawei | daniel.cohn@alvarion.com linjia@huawei.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Define the reuse factor to be assumed for CINR calculations for scan report purposes |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reuse factor for CINR calculation for scan report purposes (16.2.3)

Daniel Cohn, Ariel Doubchak
Alvarion

Problem Statement

802.16m/D4 supports AMS reporting signal metrics to its serving ABS. These reports are typically used by the ABS to trigger ABS-initiated handover process, e.g. as a response to AMS changes in position (mobility-triggered handover) or to better distribute the load among ABSs (load-triggered handover). In particular CINR metrics allows the ABS to estimate spectral efficiency of the AMS when served by different ABSs.

802.16m support flexible reuse schemes that can coexist in the same ABS (aka fractional frequency reuse or FFR) so there is a need for CINR reports from the AMS to reflect interference conditions in the different reuse schemes.

In addition, RSSI and CINR descriptions in the SCN-REP message use 802.16e concepts.

Suggested Solution

When CINR measurements are requested by the ABS, SCN-REP messages will include neighbor ABS CINR metrics reflecting conditions in all the neighbor ABS frequency partitions. If the neighbor ABS is included in the AAI_NBR-ADV message, AMS can obtain FFR information for the neighbor ABS from the SFH information in this message. Otherwise, ABS should include FFR information in the SCN-RSP message that triggers the SCN-REP transmission.

Specific formulas for RSSI and CINR calculations in different reuse schemes should be included in the relevant PHY sections (indicated as TBD below). See contribution C10/0306 for an example.

Suggested Standard Modifications

1.- Modify table 690 in section 16.2.3.15, page 91, as follows:

| Name | Value | Usage |
| --- | --- | --- |
| For (i=1; i<=numFp; i++) { |  | numFp is the number of frequency partitions at the neighbor ABS |
| BS CINR mean for frequency partition FPi | The BS CINR Mean parameter indicates the CINR measured calculated by the AMS from the particular ABS, for the FFR parameters in frequency partition FPi. The value shall be interpreted as a signed byte with units of 0.5 dB. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period.as specified in 16.3.4.13. When SCN-REP is sent in response to trigger, the trigger averaging parameter is taken from the relevant trigger. Otherwise, it is taken from the AAI_SCD message | Included when Report metric Bit 0 = 1 |
| BS RSSI mean | The BS RSSI Mean parameter indicates the Received Signal Strength measured by the AMS from the particular ABS. The value shall be interpreted as an unsigned byte with units of 0.25 dB, e.g., 0x00 is interpreted as -103.75 dBm. An MS shall be able to report values in the range -103.75 dBm to -40 dBm. The measurement shall be performed on the subcarriers of the frame preamble that are active in the serving ABS's segment and averaged over the measurement period.as specified in 16.3.4.13. When SCN-REP is sent in response to trigger, the trigger averaging parameter is taken from the relevant trigger. Otherwise, it is taken from the AAI_SCD message | Included when Report metric Bit 1 = 1 |

2.- Modify table 689 (AAI_SCN-RSP) in section 16.2.3.14 in page 90 after line 45, as follows:

| Name | Value | Usage |
| --- | --- | --- |
| Preamble Index of Recommended BS ID | Preamble Index of Recommended BS ID | Shall be included when N_Recommended_BS_Full > 0 |
| FFR configuration of recommended BS ID | DFPC value of recommended BS ID (0..15) | Shall be included when N_Recommended_BS_Full > 0 and recommended BS ID is not included in AAI_NBR-ADV |
| FFR power configuration | fpPowerConfig of recommended ABS as specified in table 814 | Shall be included when N_Recommended_BS_Full > 0 and recommended BS ID is not included in AAI_NBR-ADV |