| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Simplifying handover message flow in serving ABS (16.2.6) |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Daniel Cohn Alvarion Philip Barber Huawei | daniel.cohn@alvarion.com pbarber@huawei.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Define AMS behavior in handover process when no alternative T-ABS exists |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Simplifying handover message flow in serving ABS (16.2.6)

Daniel Cohn
Alvarion

Philip Barber
Huawei

Problem Statement

802.16m/D4 states that “If the AAI_HO-CMD message includes no target ABS, or if all the target ABSs that are included in the AAI_HO-CMD message are unreachable as defined above, the AMS shall inform the serving ABS of it's preferred target ABS by sending the AAI_HO-IND message with HO Event Code 0b01 before expiration of Disconnect Time. If a serving ABS receives the AAI_HO-IND message with HO Event Code 0b01, it shall respond to the AMS by sending AAI_HO-CMD with a target ABS which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait until receiving the AAI_HO-CMD message before expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS as defined above. If the AMS has no preferred target ABS, it may perform HO cancellation as described in section 16.2.6.3.6. If the AAI_HO-CMD message includes no target ABS, or all the target ABSs that are included in the AAI_HO-CMD message are unreachable, the Disconnect Time defines the deadline by which the AMS shall respond by sending AAI_HO-IND to the serving ABS.”

The paragraph above presents several problems, as follows:

- According to the above statement, the AMS can always send HO-IND with a new T-ABS when it is not satisfied with the choice of T-ABSs presented by the ABS. The ABS is mandated to respond with HO-CMD which may again not include a satisfactory T-ABS, which can trigger the AMS to send another HO-IND message with a T-ABS, with the AMS and ABS entering a potentially never ending loop of HO-IND/CMD.
The original purpose of the HO-IND message is to notify the S-ABS of the AMS T-ABS final choice, not to initiate a new round of negotiations regarding the candidate T-ABS list (this negotiation is performed in the HO-REQ/CMD exchange). Therefore ABS should not be mandated to respond to HO-IND with HO-CMD. If the AMS wishes to initiate a new T-ABS negotiation round, it can always send a new AAI_HO-REQ message to initiate a new handover process.

- The text “the AMS shall inform the serving ABS of it's preferred target ABS” implies that the AMS will always have a new preferred target ABS, even though the same paragraph later refers to the case where “the AMS has no preferred target ABS”

- The last sentence (“If the AAI_HO-CMD includes no target ABS .... the Disconnect Time defines the deadline...”) is redundant as the first sentence already specifies that AMS should send HO-IND before expiration of Disconnect Time.

- “Disconnect Time” is an absolute time reference, not a time interval length, so cannot "expire".

Suggested Standard Modifications

1.- Modify section 16.2.6.3.4, page 208, lines 64-65 and page 209, lines 1-11  as follows:

If the AAI_HO-CMD message includes no target ABS, or if all the target ABSs that are included in the AAI_HO-CMD message are unreachable as defined above, the AMS shall may inform the serving ABS of it's a preferred target ABS that was not included in the AAI_HO-CMD message by sending the AAI_HO-IND message with HO Event Code 0b01 before expiration of Disconnect Time. If a serving ABS receives the AAI_HO-IND message with a target ABS that was not included in the AAI_HO-CMD message HO Event Code 0b01, it shall respond to the AMS by sending AAI_HO-CMD with a target ABS which may include the target ABS proposed by the AMS in the AAI_HO-IND message. The AMS shall wait until receiving the AAI_HO-CMD message before expiration of Disconnect Time unless the AMS cannot maintain communication with the serving ABS as defined above. If the AMS has no preferred target ABS, it may perform HO cancellation as described in section 16.2.6.3.6. If the AAI_HO-CMD message includes no target ABS, or all the target ABSs that are included in the AAI_HO-CMD message are unreachable, the Disconnect Time defines the deadline by which the AMS shall respond by sending AAI_HO-IND to the serving ABS.