| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SFH deltas in HO-CMD (16.2.3) |  |
| Date Submitted | 2010-03-04 |  |
| Source(s) | Daniel Cohn Alvarion Kelvin Chou Mediatek Philip Barber Huawei | daniel.cohn@alvarion.com kelvin.chou@mediatek.com pbarber@huawei.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Allow ABS to send neighbor SFH deltas in HO-CMD message |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SFH deltas in HO-CMD (16.2.3)

Daniel Cohn
Alvarion

Kelvin Chou

Mediatek

Philip Barber
Huawei

Motivation

There are scenarios where the ABS wants to command an AMS to perform handover to a neighbor ABS that is not advertised in the AAI_NBR-ADV message. One such scenario is that of a multi-layered network where the policy is to use AMS-initiated handover for mobility-triggered handover within each layer and to use ABS-initiated handover for load-triggered handover between layers. In this scenario the ABS might choose not to advertise inter-layer neighbor ABSs to avoid AMSs attempting handover to those.

In these scenarios, the AMS has not acquired SFH information for the target ABS (normally transmitted in the NBR-ADV message) and thus cannot perform fast network re-entry.

Suggested Solution

The proposed solution allows the ABS to transmit SFH deltas (reference to the ABS SFH) in the HO-CMD message. SFH delta format will be the same as that in the AAI_NBR-ADV (not fully defined in D4)

Suggested Standard Modifications

1.- Modify table 686 in section 16.2.3.11,  page 83 after line 51, as follows:

| Name | Value | Usage |
| --- | --- | --- |
| Mode | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. |  |
| HO Reentry Mode | 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| HO Reentry Interleaving Interval | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2. | Shall be included when HO Reentry Mode is set to 1 |
| HO Reentry Iteration | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| Disconnect time offset | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Should be included when Mode = 0b00. |
| Resource_Retain_Time |  | Should be included when Mode = 0b00. Shall be included when Mode = 0b01. |
| Target ABS/YBS ID |  | Shall be included for each target ABS when Mode=0b00 |
| Preamble Index | Preamble Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| FA index | FA Index of Target ABS | Shall be included for each target ABS when Mode=0b00 |
| S-SFH change count | The change count of this BS's S-SFH IE | Shall be included for each target ABS when Mode=0b00 |
| Action Time | (Mode=0b00) Action Time included in this message is the absolute frame number at the serving ABS (Mode=0b01) This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| CDMA_RNG_FLAG |  | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. |
| Dedicated Ranging Code Flag | 1 | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. May be included when CDMA_RNG_FLAG=1 |
| Dedicated Ranging Opportunity Flag | 1 | If set to 0, it indicates that no dedicated ranging opprotunity is provided. If set to 1, it indicates that no dedicated ranging opportunity is provided. May be included when CDMA_RNG_FLAG=1 |
| Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. Shall be included when Dedicated Ranging Code Flag =1 |
| Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity. Shall be included when Dedicated Opportunity Code Flag=1 |
| Subframe Index | 3 | Indicates the subframe index of the allocated ranging opportunity. Shall be included when Dedicated Opportunity Code Flag=1 |
| HO process optimization | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| Seamless HO | Indicates whether seamless HO is supported |  |
| Ranging initiation deadline |  | Shall be included for each target ABS when Mode=0b00 |
| Service level prediction | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| Physical_Carrier_Index | Physical carrier index of the Target ABS where AMS will perform network reentry procedure. | Should be included for each target ABS which is multi-carrier ABS when Mode = 0b00 |
| Carrier Preassignment Indication | 1: information of pre-assigned secondary carriers by the target ABS is included in this message; 0: no pre-assigned secondary carrier | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| N_Preassigned_Carriers | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| Pre-assigned secondary carrier information | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| LZone Presambile Index | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| Pre allocated Basic CID | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |
| SFH delta information | Variable | Delta encoding with reference to the serving ABS. May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |