| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SFH deltas in HO-CMD (16.2.3) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Daniel Cohn Alvarion Kelvin Chou Mediatek Philip Barber Huawei Inuk Jung LG Electronics Jaehyuk Jang Samsung | daniel.cohn@alvarion.com kelvin.chou@mediatek.com pbarber@huawei.com inuk.jung@lge.com jack.jang@samsung.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Allow ABS to send neighbor SFH deltas in HO-CMD message |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SFH deltas in HO-CMD (16.2.3)

Daniel Cohn
Alvarion

Kelvin Chou

Mediatek

Philip Barber
Huawei

Inuk Jung
LG Electronics

Jaehyuk Jang
Samsung

Motivation

There are scenarios where the ABS wants to command an AMS to perform handover to a neighbor ABS that is not advertised in the AAI_NBR-ADV message. One such scenario is that of a multi-layered network where the policy is to use AMS-initiated handover for mobility-triggered handover within each layer and to use ABS-initiated handover for load-triggered handover between layers. In this scenario the ABS might choose not to advertise inter-layer neighbor ABSs to avoid AMSs attempting handover to those.

In these scenarios, the AMS has not acquired SFH and SCD information for the target ABS (normally transmitted in the NBR-ADV message) and thus cannot perform fast network re-entry.

Suggested Solution

The proposed solution allows the ABS to transmit SFH and SCD deltas (reference to the ABS SFH) in the HO-CMD message. SFH and SCD delta format will be the same as that in the AAI_NBR-ADV (not fully defined in D4). SFH and SCD change counts are also added to the message

Suggested Standard Modifications

1.- Modify table 686 in section 16.2.3.11,  page 83 after line 51, as follows:

| Name | Value | Usage |
| --- | --- | --- |
| Pre allocated Basic CID | Used by the AMS to derive its own primary CID and transport CID in target YBS. | May be included when HO to a LZone or a YBS |
| SFH delta information | Variable | Delta encoding with reference to the serving ABS. May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |
| S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information. Should be included when SFH delta information is included. |
| SCD delta information | Variable | Delta encoding with reference to the serving ABS. May be included for each target ABS when Mode = 0b00 and SCD delta information is not included in the AAI_NBR-ADV message for the target ABS |
| SCD change count | 4 | SCD change count of the reference for the included SCD delta information. Should be included when SCD delta information is included. |