| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Unicast Service Control Information within Advanced MAP (16.3.6.2.2) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yaniv Weizman Alvarion | yaniv.weizman@alvarion.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Change the term Unicast service control information carried by A-MAP |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Unicast Service Control Information within Advanced MAP (16.3.6.2.2)

Yaniv Weizman
Alvarion

Problem Statement

According to existing text, the A-MAP carries unicast service control information. The term unicast service refers to data intended for a particular entity (AMS). However, the A-MAP carries broadcast service control information elements as well such as pointed by Broadcast Assignment A-MAP IE. Moreover, some A-MAPs do point to any unicast service such as Non-User specific A-MAP.

In addition, the term unicast in A-MAP sections is used without the service reference, leading to a confused ascription to the A-MAP IE instead for the service.

Suggested Solution

It is suggested to remove the term unicast from A-MAP description as suggested below.

Every time the term unicast is used to describe unicast service, add the service reference.

Suggested Standard Modifications

Modify text in page 434, line 61 in section 16.3.6.2.2, as follows:

The Advanced MAP (A-MAP) carries unicast service control information. Unicast service control information consists of user-specific control information and non-user-specific control information.

Modify text in page 435, line 24 in section 16.3.6.3.6, as follows:

A-MAP regions shall be present in all DL unicast AAI subframes. When default TTI is used, DL data allocations corresponding to an A-MAP region occupy resources in the AAI subframe where the A-MAP region is located.

Modify text in page 455, line 24 in section 16.3.6.5.2, as follows:

16.3.6.5.2 Unicast Control Information Elements

A-MAP IE is defined as the basic element of unicast service control.