| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | A-MAPs location and corresponding data allocations within subframe (16.3.6.2.2) |  |
| Date Submitted | [The date this revision of the document is uploaded, in the format 2007-06-19. Tell the truth. Automatic numbering is prohibited.] |  |
| Source(s) | Yaniv Weizman Alvarion | yaniv.weizman@alvarion.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Emphasize the location on different A-MAPs within DL subframe for FFR and non FFR configuration. |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

A-MAPs location and corresponding data allocations within subframe (16.3.6.2.2)

Yaniv Weizman
Alvarion

Problem Statement

The location of different A-MAPs and corresponding data allocations in frequency partitions of DL and UL subframes is not clear both for FFR and non FFR configurations.

Suggested Solution

Emphasize the DL subframe frequency partition location for each A-MAP in case of FFR and non FFR configurations and their corresponding data allocations location within DL and UL subframes.

Suggested Standard Modifications

Modify text in page 435, line 24 in section 16.3.6.2.2, as follows:

A-MAP regions shall be present in all DL unicast AAI subframes. When default TTI is used, DL data allocations corresponding to an A-MAP region occupy resources in any frequency partition within the AAI subframe where the A-MAP region is located. UL data allocations corresponding to an A-MAP region occupy resources in any frequency partition within the AAI subframe according to HARQ timing tables.

Modify text in page 435, line 48 in section 16.3.6.2.2, as follows:

If non FFR is used, in a DL AAI subframe, non-user specific, HARQ feedback, power control A-MAPs are located once in the frequency partition called the primary frequency partition. The number of assignment A-MAPs in the primary frequency partition is signaled through non-user specific A-MAP.

If FFR is used in a DL AAI subframe, both the reuse 1 partition and the power-boosted reuse 3 partition may contain an A-MAP region. In a DL AAI subframe, non-user specific, HARQ feedback, and power control A-MAPs are located once in a frequency partition called the primary frequency partition. The primary frequency partition can be either the reuse 1 partition or the power-boosted reuse 3 partition, which is indicated by ABS through S-SFH SP1 IE. Assignment A-MAPs can be located only in one of the following options: in the reuse 1 partition or in the power-boosted reuse 3 partition or in both reuse 1 and reuse 3 partitions. The number of assignment A-MAPs in each frequency partition is signaled through non-user specific A-MAP.

The structure of an A-MAP region within primary frequency partition is illustrated in the example in Figure 517. The resource occupied by each A-MAP may vary depending on the system configuration and scheduler operation.