| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Using SFH resources for data allocations in case of absence of S-SFH SP (16.3.6.2.1.2) |  |
| Date Submitted | [The date this revision of the document is uploaded, in the format 2007-06-19. Tell the truth. Automatic numbering is prohibited.] |  |
| Source(s) | Yaniv Weizman Alvarion | yaniv.weizman@alvarion.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Allow the use of SFH resources for data allocations in case of absence of S-SFH SP |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Using SFH resources for data allocations in case of absence of S-SFH SP (16.3.6.2.1.2)

Yaniv Weizman
Alvarion

Problem Statement

The current text in section 16.3.6.2.1.2, line 53 allows the use of S-SFH SP resources in case where no S-SFG SP is present within SFH for control or A-MAP. There might be scenarios where resources can be used for data transmission as well (in case where resources left after A-MAPs and/or control information allocation).

Previous text within that section, in page 433 line 27 already describes it correctly (including the data section).

Suggested Solution

It is suggested to remove line 53 in section 16.3.6.2.1.2 allowing the resources to be allocated for data as well.

Suggested Standard Modifications

Modify text in page 434, line 53 in section 16.3.6.2.1.2, as follows:

When there is no S-SFH SP in the SFH, the SFH resources are used for transmitting other control information or A-MAP.