| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Specify neighbor ABS frequency in AAI_HO-CMD message (16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Daniel Cohn Alvarion Inuk Jung LG Electronics Jaehyuk Jang Samsung | daniel.cohn@alvarion.com inuk.jung@lge.com jack.jang@samsung.com |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Allow ABS to send neighbor ABS frequency information in HO-CMD message |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Specify neighbor ABS frequency in AAI_HO-CMD message (16.2.3)

Daniel Cohn
Alvarion

Inuk Jung
LG Electronics

Jaehyuk Jang
Samsung

Motivation

There are scenarios where the ABS wants to command an AMS to perform handover to a neighbor ABS that is not advertised in the AAI_NBR-ADV message. One such scenario is that of a multi-layered network where the policy is to use AMS-initiated handover for mobility-triggered handover within each layer and to use ABS-initiated handover for load-triggered handover between layers. In this scenario the ABS might choose not to advertise inter-layer neighbor ABSs to avoid AMSs attempting handover to those.

In these scenarios, the ABS needs to send enough PHY information in the AAI_HO-CMD for the AMS to identify the neighbor ABS it should handover to.

The AAI_HO-CMD message in D4 includes neighbor ABS preamble index but it does not include cyclic prefix length, center frequency and channel bandwidth. It does include “FA index” but, according to IEEE 802.16-2009, these indices rely on external conventions that cannot always be assumed to be known by AMS and ABS.

Suggested Standard Modifications

1.- Modify table 686 in section 16.2.3.11,  page 81,  line 58-60, as follows:

| Name | Value | Usage |
| --- | --- | --- |
| FA index Physical carrier index | FA Index of Target ABS As defined in AAI_Global-Config message | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| Channel bandwidth | Channel bandwidth of recommended target ABS in units of 125 KHz. Field length is 1 byte. | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| CP length | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |