| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Variable-size MAC Signaling Header Format (16.2.2.1.3) |  |
| Date Submitted | 2009-03-15 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 on P802.16m/D4 |  |
| Abstract | The contribution proposes a variable-size MAC Signaling Header format (16.2.2.1.3). |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Variable-size MAC Signaling Header Format (16.2.2.1.3)

Lei Wang

InterDigital Communications, LLC

- Introduction

In the 802.16m/D4, the MAC signaling header is of fixed size, i.e., 6 bytes, as shown in Table 655. Why do we have to have a fixed size MAC signaling header? The following answer was given: the minimum burst size is of 6 bytes. I certainly cannot agree with such a reason for having a fix 6-byte MAC signaling header, because:

- with minimum data burst size of 6 bytes, then the MAC data is of 4 bytes, due to CRC in the burst;

- Please note that MAC signaling header can also be concatenated with other MAC PDUs in a data burst, so the mini-size of 6 bytes of a data burst shall not be used as a binding factor of the signaling header design. Of course, if the group agrees, we can use 6 bytes as the max size for our 16m MAC signaling header.

- It is important to allow MAC signaling headers to be of variable sizes, because the MAC signaling headers are designed to efficiently transport short control/signaling information. As shown in the current MAC signaling headers, with the fixed size, some unnecessary padding bits are needed, thus resulting in unnecessary overhead. For example, in the AMS Battery Report Header on page 46 Table 661, there is 35 bits or 38 bits reserved fields.

In addition, allowing variable-size MAC signaling header does not impact on the 16m MAC implementation complexity, because the 16m MAC already requires the processing capability of processing variable size MAC PDUs, variable-size MAC extended headers, etc.

This contribution proposes variable-size MAC Signaling Header format.

- Suggested changes in the 802.16m/D4

The following is the proposed change in the 802.16m/D4. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: page 40, line 16

Change Table 655 as follows:

Table 655—MAC Signaling Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MAC Signaling Header { |  |  |
| Flow ID | 4 | Flow identifier. This field indicates MAC signaling header. |
| Type | 4 | MAC signaling header type. |
| Length | 3 | Indicates the length of the signaling header: 0b000 and 0b001: reserved 0b010: 2 bytes 0b011: 3 bytes 0b100: 4 bytes 0b101: 5 bytes 0b110: 6 bytes 0b111: reserved |
| Reserved | 1 | For nibble alignment.. |
| Contents | 40 variable; ≤36 | MAC signaling header Contents, with the size in bytes indicated by the length field. The number of bits is length field *8-12 |
| } |  |  |

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”