| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes related to the UL HARQ Procedure for Anonymous UL Allocations (16.2.11 and 16.2.14.2.1.2) |  |
| Date Submitted | 2009-03-03 |  |
| Source(s) | Lei Wang InterDigital Communications, LLC | Voice : +1 858 205-7286 E-mail: leiw@billeigean.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31 on P802.16m/D4 |  |
| Abstract | The contribution proposes changes related to the UL HARQ Procedure for Anonymous UL Allocations (16.2.11 and 16.2.14.2.1.2) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes related to the UL HARQ Procedure for Anonymous UL Allocations (16.2.11 and 16.2.14.2.1.2)

Lei Wang

InterDigital Communications, LLC

- Introduction

In the 802.16m draft standard [2], there are two types of UL unicast allocations: non-anonymous and anonymous, depending on whether or not the AMS’s STID information is provided in the allocation IE. With a non-anonymous UL allocation, the AMS’s STID information is provided by the allocation IE, e.g, UL Basic Assignment A-MAP IE. With an anonymous UL allocation, the STID of the AMS is not given in the allocation IE, instead, the contention-based access signal that the AMS sent in previous UL frame is used to identify the AMS, e.g., the RA-ID in the CDMA allocation IE, and the BR opportunity and BR sequence in the BR-ACK IE.

This contribution identifies a problem with the UL HARQ retransmission procedure for the anonymous UL allocations and also provides a remedy to the identified problem.

- Problem Discussion

The 802.16m UL unicast data transmission uses a synchronous HARQ retransmission procedure, in which the HARQ retransmissions are implicitly allocated at the predefined interval and locations until the HARQ data transmission is successful or it reaches the max allowed HARQ retransmissions. As specified in the 802.16m draft standard [2], before reaching the max number of HARQ retransmissions but still not successful, the ABS can change or stop the UL allocations for the synchronous UL HARQ retransmissions by sending a basic UL allocation IE with the same ACID by with AI_SN not-toggled or toggled, respectively.

However, the above mentioned mechanisms do not apply to the unicast UL allocations anonymously allocated by CDMA allocation IE or BR-ACK IE,  because there is no  ACID or AI_SN fields in such unicast UL allocations, and the AMS of the UL allocation is identified by RAID or the used random access channel and random  access preamble.

Note that the same reasons that the ABS needs to change or stop the synchronous UL HARQ retransmissions for the non-anonymous UL allocations also apply to the anonymous UL allocations. In addition, there is one more reason that ABS needs to stop the HARQ retransmissions for an anonymous UL allocation, because of the non-zero probability of collision in an anonymous UL allocation, i.e., when more than one AMS transmitted the same preamble sequence in the same random access channel in the random access procedures, e.g., initial ranging, periodic ranging, contention-based bandwidth request.

- Proposed Solution

When the ABS needs to stop / change the HARQ retransmission allocations for an anonymous UL allocation, it sends a CDMA allocation IE with the corresponding RA-ID. If the CDMA allocation IE has zero resource allocation, it indicates to stop the HARQ retransmissions for an anonymous UL allocation, otherwise it indicates a change in the HARQ retransmissions for an anonymous UL allocation.

- Suggested changes in the 802.16m/D4

Based on the above discussion, we propose the following changes in the 802.16m/D4. Note that the new text is marked with blue and underline; the deleted text are marked with red and strikethrough.

Suggested change #1: on page 503, line 52

Change the row of “Resource Index” in Table 832, line 52 page 503 as follows:

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| CDMA_Allocation_A-MAP IE() { |  |  |
| ……. | ….. | ….. |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size. If all bits are set to 1, it means a zero resource allocation that indicates a termination of the UL HARQ retransmission for the anonymous UL allocation identified by the RAID masked in the CRC of this CDMA A-MAP IE. |
| …… | ….. | …… |
| } |  |  |

Suggested change #2: on page 278, line 63

Insert the following paragraph in line 63 on page 278:

For an anonymous unicast UL allocation, i.e., an UL allocation without the AMS’s STID information provided in the allocation A-MAP IE, e.g., the UL allocations given by CDMA allocation A-MAP IE or BR ACK A-MAP IE, the HARQ procedure is the same as the non-anonymous UL allocations, e.g., allocated by a UL Basic Assignment A-MAP IE. The ABS may change or terminate the UL HARQ retransmission allocation for an anonymous unicast UL allocation by sending a CDMA allocation A-MAP IE with the RA-ID corresponding to the AMS' random access attributes, e.g., superframe number, frame_index, random access preamble code index, and random access opportunity, for the anonymous unicast UL allocation. If all bits in the Resource Index field are set to 1 in a CDMA Allocation A-MAP IE, it indicates a termination of UL HARQ retransmissions for the anonymous UL allocation with the same RA-ID masked in the CRC of the CDMA allocation A-MAP IE.

Suggested change #3: on page 256, line 4

Change the paragraph in line 4 on page 256 as follows:

During the 3-step BR procedure of Figure 429, if the ABS is unable to decode the quick access message, the ABS falls back to the 5-step BR procedure illustrated in Figure 430. In that case, in Step 2, the ABS shall provide an UL grant to the AMS using a BR ACK A-MAP IE or CDMA Allocation A-MAP IE. The maximum HARQ retransmission of the allocation mode through the BR ACK A-MAP IE or CDMA Allocation A-MAP IE is set to the default value defined in section 16.2.14.2. The ABS can stop or change the HARQ retransmission allocations for the UL grant given by Step 3 by sending a CDMA allocation A-MAP IE with the RA-ID corresponding to the AMS' random access attributes at Step 1, where a CDMA allocation IE with its 11-bit Resource Index field of the value 0x8FF indicates a stop of the HARQ retransmissions. The AMS may restart the BR procedure if it receives a CDMA allocation IE indicating a stop of the HARQ retransmission. In Step 3, the AMS transmits a standalone BR header.

Suggested change #4: on page 258, line 62

Change the paragraph in line 62 on page 258

The maximum number of the HARQ retransmission is set to the default value defined in section 16.2.14.2. The ABS can stop or change the HARQ retransmission allocations by sending a CDMA allocation A-MAP IE with the RA-ID corresponding to the AMS' random access attributes at the Step 1 of the contention-based random access bandwidth request procedure, where the CDMA allocation IE with its 11-bit Resource Index field of the value 0x8FF indicates a stop of the HARQ retransmissions.  HARQ retransmission control information cannot be changed during retransmission process.

Suggested change #5: on page 522, line 18

Append the following text at the end of the paragraph in line 18 on page 522

The ABS can stop or change the HARQ retransmission allocations by sending a CDMA allocation A-MAP IE with the RA-ID corresponding to the AMS' random access attributes at the Step 1 of the contention-based random access bandwidth request procedure, where the CDMA allocation IE with its 11-bit Resource Index field of the value 0x8FF indicates a stop of the HARQ retransmissions.

- References

[1] IEEE Std 802.16-2009

[2] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”