| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Indication Method of Frequency Partition for UL Control Channels (16.3.8) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Jinyoung Chun, HanGyu Cho and Jin Sam Kwak LG Electronics Chiwoo Lim, Jeongho Park Samsung Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> chiwoo.lim@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31 |  |
| Abstract | This contribution provides the indication method of frequency partition for UL control channels. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Indication Method of Frequency Partition for UL Control Channels

Jinyoung Chun, HanGyu Cho and Jin Sam Kwak

LG Electronics

Chiwoo Lim and Jeongho Park

Samsung Electronics

- Introduction

In the last meeting, the text was included that ‘If FFR is used in an UL AAI subframe, the UL control channels can be used in the reuse 1 partition and the power-boosted reuse 3 partition’. But there is no description about the method which partition is selected.

- Problem statements

The DL frequency partition for non-user specific, HARQ feedback, and power control A-MAP is indicated by ‘Primary frequency partition location’ in S-SFH SP1. UL frequency partition for UL control channels except the ranging channel can be also indicated in S-SFH SP1, but it is very expensive channel and it may have not enough room. Therefore we suggest two options. One is the indication by S-SFH SP1. And the other is the implicit method that the UL frequency partition is aligned with the DL frequency partition if there are both frequency partitions in UL.

- Text proposal in the P802.16m/D4

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Add the row in line 27 of Table 811- S-SFH SP1 IE format]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … |  |  |
| DL Primary frequency partition location | 1 | 0b0: Reuse 1 partition 0b1: Power-boosted reuse 3 partition |
| Frequency partition location for UL control channels | 1 | 0b0: Reuse 1 partition 0b1: Power-boosted reuse 3 partition |
| … |  |  |

[Modify the text in page 19 of page 596 as below:]

If FFR is used in an UL AAI subframe, the UL control channels can be are used in the reuse 1 partition and or the power-boosted reuse 3 partition by S-SFH SP1.

[Modify the text on page 596 line 23 as following text]

The number of bandwidth request channels in frequency partition FPi in a UL AAI subframe is Nbwr.

The number of bandwidth request channels in an UL AAI subframe is Nbwr.

[Modify the text on page 596 line 40-49 as following text]

Let UL_FEEDBACK_SIZE distributed LRUs in frequency partition FPi be reserved for feedback channels in the units of LRU. The number of feedback channels in frequency partition FPi, is LFB,FPi.

LFB,FPi = Nfb∙UL_FEEDBACK_SIZE                                         (267)

Let UL_FEEDBACK_SIZE distributed LRUs in an UL AAI subframe be reserved for feedback channels in the units of LRU. The number of feedback channels in an UL AAI subframe, is LFB.

LFB = Nfb∙UL_FEEDBACK_SIZE                                           (XXX)

[Modify the term on page 596 line 56 as following term]

3∙LFB,FPi -1

3∙LFB -1

[Modify the term on page 597 line 14 as following term]

LFB,FPi -1

LFB -1

[Modify the term on page 597 line 21 as following term]

LHFB,FPi

LHFB

[Modify the term on page 597 line32 as following term]

3∙LHFB,FPi -1

3∙LHFB /2 -1

[Modify the term on page 597 line 50 as following term]

LHFB,FPi -1

LHFB -1

-----------------------------------------------   Text End  -----------------------------------------------------------