| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Change of Frame Configuration considering MIMO Midamble (16.3.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jinyoung Chun, Jin Sam Kwak LG Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31 |  |
| Abstract | This contribution provides the change of frame configuration considering MIMO midamble. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Change of Frame Configuration considering MIMO Midamble

Jinyoung Chun, Jin Sam Kwak

LG Electronics

- Introduction

In the last meeting, it is decided that MIMO midamble shall be located in the 2nd DL AAI subframe every frame. Most of 2nd DL AAI subframes consist of Type-2 AAI subframe for MIMO midamble if the DL frame has Type-2 AAI subframes but some cases are not, especially in FDD. Therefore I suggest all 2nd AAI subframes in Table 780, 781, and 782 are formed by Type-2 AAI subframe if the DL frame has Type-2 AAI subframes, which means a symbol for MIMO midamble and Type-1 subframe for data/control channels.

- Problem statements

According to the Frame configuration in Table 780, 781, and 782, the first subframes are described as DL Type1. But the exact meaning is a symbol for PA/SA-preamble and Type-3 subframe for data/conntrol channels. Therefore most of DL subframes consist of type 3 and type 1 subframes except the below cases. The frames consists of Type 1, 2, and 3 (except the symbols of PA/SA-preamble and midamble) and it’s very complicate to implement all types.

Table 780 Frame Configuration and Indexing (5/10/20MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 5 | 5/10/20 | 1/16 | 4 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type1 | N/A |
| 29 | 5/10/20 | 1/4 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |

Table 781 Frame Configuration and Indexing (8.75MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 4 | 8.75 | 1/16 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 |  |  |
| 8 | 8.75 | 1/8 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |

Table 782 Frame Configuration and Indexing (7MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| 6 | 7 | 1/8 | 2 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type2 | D/U Type2 | D/U Type2 |  |  |  | N/A |
| 11 | 7 | 1/4 | 2 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 |  |  |  | N/A |

Therefore I suggest minimizing the complicated cases if possible. The second rows in each table have a Type-2 subframe but it’s not in 2nd subframe. So in the cases, some configurations can remove Type-2 DL AAI subframe by moving the Type-2 AAI subframe to 2nd AAI subframe. Then the frames just consist of Type-3 and Type-1 AAI subframes.

- Text proposal in the P802.16m/D4

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Modify the table 780, 781, and 782 of page 369~271 in the section 16.3.3.7 Set of frame configurations as below:]

Table 780 Frame Configuration and Indexing (5/10/20MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 | 5/10/20 | 1/4 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 Type2 | D/U Type1 | D/U Type2 Type1 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 781 Frame Configuration and Indexing (8.75MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 | 8.75 | 1/8 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 Type2 | D/U Type1 | D/U Type2 Type1 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 782 Frame Configuration and Indexing (7MHz channel bandwidth)

| No | BW | CP | Frame configuration index | Duplex | D:U | Subframe provision |  |  | AAI subframe Type |  |  |  |  |  |  |  | TTG/RTG (us) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | DL offset | DL Length | UL Length | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | 7 | 1/4 | 2 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 Type2 | D/U Type2 Type1 | D/U Type1 | D/U Type1 |  |  |  | N/A |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

-----------------------------------------------   Text End  -----------------------------------------------------------