| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Allocation Method of UL Sounding Channel in Frame Structure supporting WirelessMAN-OFDMA (16.3.6) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Jinyoung Chun, Wookbong Lee, Jin Sam Kwak, and Bin-Chul Ihm LG Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31 |  |
| Abstract | This contribution provides the allocation method of UL sounding channel in frame structure to support WirelessMAN-OFDMA. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Allocation Method of UL Sounding Channel in Frame Structure supporting WirelessMAN-OFDMA (16.3.6)

Jinyoung Chun, Wookbong Lee, Jin Sam Kwak and Bin-Chul Ihm

LG Electronics

- Introduction

When frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner as defined in 15.3.3.4, ABS can’t allocate the sounding channel in the unit of PRU. Therefore I suggest that sounding subband is formed by the logical DLRUs when frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner.

- Text proposal in the 802.16m draft

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-----------------------------------------------   Text Start  -----------------------------------------------------------

[Add the below text in line 6, page 503 in 16.3.6.5.2.4.6 UL sounding command A-MAP IE]

Table 831 specifies the fields of UL Sounding Command A-MAP IE used by the ABS to request sounding transmission by the AMS. Decimal equivalent of the sounding AAI subframe indicates the AAI subframe number with sounding symbol (the first AAI subframe in frame is indexed 0). The sounding subband bitmap field is used to indicate the sounding subbands used in the sounding allocation. For that purpose, the Nused contiguous subcarriers are divided into sounding subbands, where each sounding subband compromises N1 * Nsc adjacent subcarriers with N1= 4 and Nsc = 18 for NFFT = 512, 1024 and 2048. The MSB of the Sounding subband bitmap field corresponds to the sounding subband with lowest subcarrier indexes. When frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner, sounding subband means logical subband, which consists of 4 DLRUs with continuous indices. The MSB of the Sounding subband bitmap field corresponds to the sounding subband with lowest DLRUs. The DLRU index is after the subchannelization in 16.3.8.3.5.3. The three periodicity bits are used to indicate the AMS to periodically repeat the sounding transmission. Setting periodicity bits to 0b000 indicates a single sounding command or terminates the sounding if periodic sounding command is being performed.

-----------------------------------------------   Text End  -----------------------------------------------------------