| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Allocation Method of UL Sounding Channel in Frame Structure supporting WirelessMAN-OFDMA (16.3.6) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Jinyoung Chun, Wookbong Lee, Jin Sam Kwak, and Bin-Chul Ihm LG Electronics Jeongho Park, Mihyun Lee Samsung Electronics | jiny.chun@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirc #31 |  |
| Abstract | This contribution provides the allocation method of UL sounding channel in frame structure to support WirelessMAN-OFDMA. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Allocation Method of UL Sounding Channel in Frame Structure supporting WirelessMAN-OFDMA (16.3.6)

Jinyoung Chun, Wookbong Lee, Jin Sam Kwak and Bin-Chul Ihm

LG Electronics

Jeongho Park, Mihyun Lee

Samsung Electronics

- Introduction

When frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner as defined in 15.3.3.4, sounding channels of both legacy and AAI have different frequency granularity and time location. Therefore I suggest the allocation of the sounding channel when frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner.

- Problem statements

The legacy sounding channel is allocated based on the physical frequency band consisted of continuous 18 subcarriers and is located in the last symbol(s) of a permutation zone (see 8.4.5.4.2). And the AAI sounding channel is allocated based on sounding band consisted of continuous 4*18 subcarriers and is located in the first symbol of subframe.

Therefore we suggest the sounding channel occupies the last symbol of the last UL subframe for both of legacy and AAI subframe with frequency granularity based on 4*18 subcarriers as below figure 1 when frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner. Then we can’t use the 3 symbols including sounding channel in Lzone to allocate UL bursts.

Figure 1. the location of sounding channel

- Text proposal in the 802.16m draft

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

-----------------------------------------------   Text Start  -----------------------------------------------------------

[Add the below text in line 59, page 609 in 16.3.9.1.3.1 Sounding PHY structure]

16.3.9.1.3.1 Sounding PHY structure

The sounding signal occupies a single OFDMA symbol in the UL sub-frame. The sounding symbol in the UL sub-frame is located in the lastfirst symbol. Each UL sub-frame can contain only one sounding symbol. For type-1 AAI subframe, the sounding signal shall not be transmitted in the LRU which contains other control channels. For type-2 AAI subframe, sounding signals can be transmitted in any resource unit. For the six-symbol PRU case, the remaining 5 consecutive symbols are formed to be a five-symbol PRU used for data transmission, as shown in Figure 563. For the seven-symbol PRU case, the remaining 6 consecutive symbols are formed to be a six-symbol PRU for data transmission, as shown in Figure 563. Multiple UL AAI subframes in a 5-ms radio frame can be used for sounding. The number of subcarriers for the sounding in a PRU is 18 adjacent subcarriers.

Figure 563—Sounding PHY structures for (a) 6-symbol PRU and (b) 7-symbol PRU cases.

When frame structure is supporting the WirelessMAN-OFDMA MSs in PUSC zone by FDM manner, the sounding channels of both LZone and MZone are allocated in a symbol based on the unit of sounding subband, i.e., continuous 4*18 subcarriers by UL Sounding Command IE (8.4.5.4.24) and UL Sounding Command A-MAP IE (16.3.6.5.2.4.6), respectively.

-----------------------------------------------   Text End  -----------------------------------------------------------