| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for HARQ retransmission interval on the DL (16.2.14.2.1.1) |  |
| Date Submitted | 2010–03–05 |  |
| Source(s) | Amir Rubin Intel Corporation | amir.rubin@intel.com |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D4 |  |
| Abstract | The contribution proposes the text additions for restricting the retransmission interval on DL HARQ burst. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for HARQ retransmission interval on the DL (16.2.14.2.1.1)

Amir Rubin (Intel corp)

- Introduction

The DL HARQ retransmission interval period, termed "T_ReTx_Interval", is not defined in the standard. This value is required for defining the system operation e.g. in the sense of delay distributions of DL PDUs and for allowing focused AMS design of the HARQ buffering. Therefore, this contribution aims limiting the retransmission interval when the amount of soft bits potentially buffered at the AMS is high. In that case, there’s an incentive to lower the buffer occupancy by making quick retransmissions. If the occupancy is low due to low throughput or low error rate, the retransmission interval limitation is not significant and therefore relaxed.

It is also important to limit the listening interval of devices in sleep mode. Therefore, this contribution limits the T_ReTx_Interval so that it doesn’t exceed 4 frames.

- References

[1] IEEE P802.16m/D4, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems” December 2009.

[2] IEEE 802.16-2009, “IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”.

[3] IEEE 802.16m System Description Document (SDD)

- Proposed changes for 802.16m draft amendment

--------------------------------------------Begin of Proposed Text Change 1--------------------------------------------------

<Add in page 278 line 36, section 16.2.14.2.1.1 Downlink >

For TDD operation: The T_ReTx_Interval for all bursts in a frame is determined as:

Min (2 * Frame_length * RTT_Factor_TDD, 4* Frame_length)

RTT_Factor_TDD is:

Max (1, AMS DL HARQ buffering capability in a period of 2 frames / amount of soft bytes potentially buffered at the AMS)

Where “AMS DL HARQ buffering capability in a period in a period of 2 frames” is:

(AMS DL HARQ buffering capability+1) * 600 * Amount of DL subframes in a period of 2 frames.

For FDD operation: The T_ReTx_Interval is determined as follows:

Min (1.5 * Frame_length * RTT_Factor_FDD, 4* Frame_length)

RTT_Factor_FDD is:

Max (1, AMS DL HARQ buffering capability in a period of 1.5 frames / amount of soft bytes potentially buffered at the AMS)

Where AMS DL HARQ buffering capability in a period in a period of 1.5 frames is:

(AMS DL HARQ buffering capability+1) * 600 * Amount of DL subframes in a period of 1.5 frames.

Notably, the value of T_ReTx_Interval increases as the amount of buffered bytes at the AMS decreases. However, it doesn’t drop below periods of 2 frames for TDD 1.5 frames for FDD.

T_ReTx_Interval doesn’t exceed 4 frames to limit the listening interval of devices in sleep mode.

The value of “amount of soft bytes potentially buffered at the AMS” shall decrease as the DL processing period of the ABS is decreased. This is due to the decreased period over which AMS feedbacks have not been processed by the ABS and that shall be considered as indicating failure for the calculation of: T_ReTx_Interval.

--------------------------------------------End of Proposed Text Change 1--------------------------------------------------

--------------------------------------------Begin of Proposed Text Change 2--------------------------------------------------

<Change in page 116 line 25, section 16.2.3.29 AAI_System Configuration Descriptor (SCD) Message >

T_ReTx_IntervalINTERGER (0..8)

--------------------------------------------End of Proposed Text Change 2--------------------------------------------------