| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clarifications on S-SFH contents (D4 - 16.3.6.5.1.2) |  |
| Date Submitted | 2010-03-15 |  |
| Source(s) | Heejeong Cho, Jinsoo Choi, Jin-young Chun, DongCheol Kim, Ki Seon Ryu, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes some modification on section 16.3.6.5.1 (Broadcast Control Information Elements) for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clarifications on S-SFH contents (D4 - 16.3.6.5.1.2)

Heejeong Cho, Jinsoo Choi, Jin-young Chun, DongCheol Kim, Ki Seon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for clarifications for S-SFH contents in the IEEE 802.16m DRAFT Amendment [1].

- Modifications and key-descriptions

- Use the unified terminology

  - UL ACK/NACK channel  UL HARQ feedback channel

  - DL ACK/NACK channel  DL HARQ Feedback A-MAP

  - A-A-MAP  Assignment A-MAP

  - Number of Distributed LRUs for UL feedback channel per a UL AAI subframe  UL_FEEDBACK_SIZE

  - Ranging channel [2]

- Re-arrangement of the fields

  - Move Uplink AAI subframes for sounding, ABS EIRP, Cell bar information, UL_N_MAX_ReTx, DL_N_MAX_ReTx and TUL_Rx_Processing to the upper row

- Add clean-up text

  - WirelessMAN-OFDMA with FDM-based UL PUSC Zone

  - Duplex mode with FDD

  - EIRxPIR,min

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] IEEE C80216m-10/0111, “The Clarification of the Ranging Channels in the IEEE P802.16m/D4 (16.3.9.1.4, 16.3.9.2.4, 16.2.3.29 and 16.3.6.5.1.2)”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D4, page 448, line 1, make the following changes to subclause 16.3.6.5.1.2]

16.3.6.5.1.2 S-SFH IE

Table 811—S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |  |
| --- | --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |  |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |  |
| LSBs of 48 bit ABS MAC ID | 12 | Specifies the 12 least significant bits of ABS ID |  |
| Number of UL ACK/NACK HARQ feedback channels per HARQ feedback region | 2 | Describes LHFB in 16.3.8.3.3.2. Channel numbers represented by the two bits (0, 1,2, 3) are as follows. For 5 MHz band, 6, 12, 18, 24 For 10 MHz band, 6, 12, 24, 30 For 20 MHz band, 12, 24, 48, 60 |  |
| Number of DL ACK/NACK channels HARQ Feedback A-MAPs per HF-A-MAP region | 2 | Channel numbers represented by the two bits (0, 1,2, 3) are as follows. For 5 MHz band, 4, 8, 12, 16 For 10 MHz band, 8, 16, 24, 32 For 20 MHz band, 16, 32, 48, 64 |  |
| Power control channel resource size indicator | 2 |  |  |
| Primary frequency partition location | 1 | 0b0: Reuse 1 partition 0b1: Power-boosted reuse 3 partition |  |
| A-A-MAP MCS selection | 1 | 0b0: QPSK 1/2 and QPSK 1/4 can be used for assignment A-MAP in reuse-1 partition. QPSK 1/2 is used for assignment A-MAP in the power-boosted reuse 3 partition of FFR. 0b1: QPSK 1/2 and QPSK 1/8 can be used for assignment A-MAP in reuse-1 partition. QPSK 1/4 is used for assignment A-MAP in the power-boosted reuse 3 partition of FFR. |  |
| DCASSB,0 | 5/4/3 | See 16.3.5.3.1 DL CRU/DRU allocation Indicate the number of subband-based CRUs in FP0 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| DCASMB,0 | 5/4/3 | See 16.3.5.3.1 DL CRU/DRU allocation Indicate the number of miniband-based CRUs in FP0 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| DCASi | 3/2/1 | See 16.3.5.3.1 DL CRU/DRU allocation Indicate the number of total allocated subband and miniband CRUs, in a unit of a subband, for FPi(i≥0) For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |  |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table Table 780, Table 781,and Table 782 |  |
| Uplink AAI subframes for sounding | 3 | This value represents the number of uplink AAI subframes with sounding symbols. 0b000 – no sounding symbols 0b001 – 1 AAI subframe 0b010 – 2 AAI subframes 0b011 – 3 AAI subframes 0b100 – 4 AAI subframes 0b101-111 – reserved The sounding symbols shall be placed in AAI subframes in accordance to their type. First, sounding symbols shall be allocated in uplink AAI subframes of type 2 starting from the first in time AAI subframe of type 2. If the number of uplink AAI subframe of type 2 is less than the number of AAI subframes for sounding, sounding symbols shall be allocated in the AAI subframes of other types in the following order: type 1. For these types of uplink AAI subframes sounding symbols shall be allocated in the similar way as for type 2. Type 3 uplink AAI subframes are not used for sounding. |  |
| ABS EIRP | 7 | Signed in units of 1 dBm |  |
| Cell bar information | 1 | If Cell Bar bit = 0b1, this cell is not allowed for network entry or re-entry |  |
| UL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for UL HARQ 0b0: 4 0b1: 8 |  |
| DL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for DL HARQ 0b0: 4 0b1: 8 |  |
| TUL_Rx_Processing | 1 | Specifies the ABS's Rx processing time for UL HARQ for F= 8 in FDD or D + U = 8 in TDD 0b0: 3 AAI subframes 0b1: 4 AAI subframes |  |
| WirelessMAN-OFDMA support | 1 | Indicates whether frame configuration supports WirelessMAN-OFDMA systems or not 0b0 : No support of WirelessMAN-OFDMA 0b1 : Support of WirelessMAN-OFDMA |  |
| If(Support of WirelessMAN-OFDMA support = 0b1 with FDM-based UL PUSC Zone){ |  |  | Indicates support of the WirelessMAN-OFDMA with FDM-based UL PUSC Zone, when the WirelessMAN-OFDMA support is 0b1 and Frame configuration index is equal to - 5, 7, 9, 11, 13 or 15 for 5/10/20 MHz channel bandwidth according to Table 780; - 4or 6 or 8 for 8.75 MHz channel bandwidth according to Table 781; - 3 (with CP=1/8) for 7 MHz channel bandwidth according to Table 782. |
| Allocation periodicity of ranging channelthe RCH | 2 | Indicates the periodicity of ranging channelthe RCH allocation according to the Table 898. |  |
| Subframe offset of ranging channelthe RCH | 2 | Indicates the subframe offset (OSF) of ranging channelthe RCH allocation related to the Table 901898. The range of values is 0 ≤ OSF ≤ 3. |  |
| Start RP code information of ranging channelthe RCH | 4 | Indicates the knskns which is the parameter for start of the RP code group (Srns0). Srns0(kns)= 16×kns+1 The range of values is 0 ≤ kns ≤ 15. |  |
| Ranging preamble RP code partition information for the RCH | 4 | Indicates the number of the initial, handover and periodic RP codes (N, O and MNIN, NHO and NPE) according to the Table 902. |  |
| UL_Permbase | 7 | Indicate UL_Permbase used in WirelessMAN-OFDMA system with FDM-based UL PUSC Zone. |  |
| Reserved | 9/6/3 | For 2048 FFT size, 9 bits For 1024 FFT size, 6 bits For 512 FFT size, 3 bits |  |
| } else if (WirelessMAN-OFDMA support = 0b0) { |  |  |  |
| if(Femtocell) { |  | for 16m Femtocell |  |
| Allocation periodicity of ranging channel for synchronized AMSsthe S-RCH | 2 | Indicates the periodicity of ranging channelthe S-RCH allocation according to the Table 9001. |  |
| Subframe offset of ranging channelthe S-RCH | 2 | Indicates the subframe offset (OSF) of ranging channelthe S-RCH allocation related to the Table 9001. The range of values is 0 ≤ OSF ≤ 3. |  |
| Start RP code information of ranging channel for synchronized AMSsthe S-RCH | 4 | Indicates the ks which is the parameter controlling the start root index of ranging preamble the RP codes (rs0). rs0= 6×ks+1 The range of values is 0 ≤ ks ≤ 15. |  |
| Ranging preamble RP code partition information for the S-RCH | 4 | Indicates the number of initial, handover and periodic RP codes (N, O and M NIN, NPE and NHO) according to the Table 900. |  |
| Reserved | 3 |  |  |
| } else { |  |  |  |
| Allocation periodicity of ranging channel for non-synchronized AMSsthe NS-RCH | 2 | Indicates the periodicity of ranging channelthe NS-RCH allocation according to the Table 8978. |  |
| Subframe offset of ranging channel for non-synchronized AMSsthe NS-RCH | 2 | Indicates the subframe offset (OSF) of ranging channelthe NS-RCH allocation related to the Table 901898. The range of values is 0 ≤ OSF ≤ 3. |  |
| Start RP code information of ranging channel for non-synchronized AMSs the NS-RCH | 4 | Indicates the kns which is the parameter controlling the start root index of ranging preamble the RP codes (rns0). rns0(kns)=4×kns+1 for ranging channel the NS-RCH format 0. rns0(kns)=16×kns+1 for ranging channel the NS-RCH format 1. The range of values is 0 ≤ kns ≤ 15. |  |
| Ranging preamble RP code partition information for non-synchronized AMSsthe NS-RCH | 4 | Indicates the number of initial and handover ranging preamble RP codes (NIN and NHO) according to the Table 8997. |  |
| Number of the cyclic shifted ranging preamble RP codes per root index for non-synchronized AMSsthe NS-RCH | 2 | Indicates the number of cyclic shifted codes per root index (Mns) for ranging preamble the RP codes according to the Table 896. |  |
| Ranging channel NS-RCH formats for non-synchronized AMSs | 1 | Indicates the ranging channel NS-RCH formats number of Table 891 |  |
| } |  |  |  |
| UCASSB,0 | 5/4/3 | See 16.3.8.3.1UL CRU/DRU allocation Indicate the number of subband-based CRUs in FP0 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| UCASMB,0 | 5/4/3 | See 16.3.8.3.1 UL CRU/DRU allocation Indicate the number of miniband-based CRUs in FP0 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |  |
| UCASi | 3/2/1 | See 16.3.8.3.1 UL CRU/DRU allocation Indicate the number of total allocated CRUs, in a unit of a subband, for FPi(i≥0) For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |  |
| } |  |  |  |
| Uplink AAI subframes for sounding | 3 | This value represents the number of uplink AAI subframes with sounding symbols. 0b000 – no sounding symbols 0b001 – 1 AAI subframe 0b010 – 2 AAI subframes 0b011 – 3 AAI subframes 0b100 – 4 AAI subframes 0b101-111 – reserved The sounding symbols shall be placed in AAI subframes in accordance to their type. First, sounding symbols shall be allocated in uplink AAI subframes of type 2 starting from the first in time AAI subframe of type 2. If the number of uplink AAI subframe of type 2 is less than the number of AAI subframes for sounding, sounding symbols shall be allocated in the AAI subframes of other types in the following order: type 1. For these types of uplink AAI subframes sounding symbols shall be allocated in the similar way as for type 2. Type 3 uplink AAI subframes are not used for sounding. |  |
| ABS EIRP | 7 | Signed in units of 1 dBm |  |
| Cell bar information | 1 | If Cell Bar bit = 1, this cell is not allowed for network entry or re-entry |  |
| UL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for UL HARQ 0b0: 4 0b1: 8 |  |
| DL_N_MAX_ReTx | 1 | Specifies the maximum retransmission number for DL HARQ 0b0: 4 0b1: 8 |  |
| TUL_Rx_Processing | 1 | Specifies the ABS's Rx processing time for UL HARQ for F= 8 in FDD or D + U = 8 in TDD 0b0: 3 AAI subframes 0b1: 4 AAI subframes |  |
| Reserved | TBD |  |  |
| } |  |  |  |

Table 812—S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| If (Duplexing mode == FDD) { |  | The duplexing mode is obtained from the frame configuration index set in S-SFH SP1 IE Indicates that the duplex mode is FDD, when the Frame configuration index in S-SFH SP1 IE is equal to - 4 (with CP= 1/16 and 1/8) or 3 (with CP= 1/4) for 5/10/20 MHz channel bandwidth according to Table 780; - 3 for 8.75 MHz channel bandwidth according to Table 781; - 2 for 7 MHz channel bandwidth according to Table 782. |
| UL carrier frequency | 6 |  |
| UL bandwidth | 3 | The frequency spacing for UL channel is same with DL channel. 0b000: 512 FFT 0b001: 1024 FFT 0b010: 2048 FFT 0b011 to 0b111: reserved |
| } |  |  |
| MSB bytes of 48 bit ABS MAC ID | 36 | Specifies 36 MSB of BS ID |
| MAC protocol revision | 4 | Version number of AAI supported on this channel |
| DSAC | 5/4/3 | See 16.3.5.2.31 DL Frequency partitioningSubband partitioning Indicate the number of subbands KSB as defined in Table 783 to 785 For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| DFPC | 4/3/3 | See 16.3.5.2.3 DL Frequency partitioning Indicate the frequency partition configuration as defined in Table 786 to 788 For 2048 FFT size, 4 bits For 1024 FFT size, 3 bits For 512 FFT size, 3 bits |
| DFPSC | 3/2/1 | See 16.3.5.2.3 DL Frequency partitioning Indicate the number of subbands allocated to FPi, i>0 For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bit |
| If (No support of WirelessMAN-OFDMA with FDM-based UL PUSC Zone){ |  | Indicates no support of the WirelessMAN-OFDMA with FDM-based UL PUSC Zone, when the WirelessMAN-OFDMA support in S-SFH SP1 IE is 0b0 or when WirelessMAN-OFDMA support is 0b1 and Frame configuration index in S-SFH SP1 IE is not equal to - 5, 7, 9, 11, 13 or 15 for 5/10/20 MHz channel bandwidth according to Table 780; - 4or 6 or 8 for 8.75 MHz channel bandwidth according to Table 781; - 3 (with CP=1/8) for 7 MHz channel bandwidth according to Table 782. |
| USAC | 5/4/3 | See 16.3.8.2.3 UL Frequency partitioning For 2048 FFT size, 5 bits For 1024 FFT size, 4 bits For 512 FFT size, 3 bits |
| UFPC | 4/3/3 | See 16.3.8.2.3 UL Frequency partitioning For 2048 FFT size, 4 bits For 1024 FFT size, 3 bits For 512 FFT size, 3 bits |
| UFPSC | 3/2/1 | See 16.3.8.2.3 UL Frequency partitioning For 2048 FFT size, 3 bits For 1024 FFT size, 2 bits For 512 FFT size, 1 bits |
| } |  |  |
| AMS Transmit Power Limitation Level | 5 | Unsigned 5-bit integer. Specifies the maximum allowed AMS transmit power. Values indicate power levels in 1 dB steps starting from 0 dBm |
| EIRxPIR,min | 5 | Initial ranging minimum equivalent isotropic received power at ABS. Signed in units of 1 dBm |
| reserved |  |  |
| } |  |  |

Table 813—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Rate of change of S-SFH info | 4 | Minimum duration over which the contents of the S-SFH does not change 0b0000: 16 0b0001: 32 0b0010: 64 0b00011 ~ 0b1111: reserved |
| SA-Preamble sequence soft partitioning information | 4 | Specifies the partition information of SA-Preamble sequence for non-macro ABS as public and CSG femto BS as defined in Table 798 |
| FFR partition resource metrics | 0 or 4 or 8 | 0 bit: if FPCT=1. 4 bits or 8 bits: as As defined in Table 772 |
| IoT correction value for UL power control | 10 | The 10-bits IoT correct value is used to support the correction of 5 IoT values (IOT_Sounding, IOT_FP0, IOT_FP1, IOT_FP2, IOT_FP3) defined in AAI_UL_NI message, each 2 bits are expressed as the correction value: 0b00: +1dB 0b01: +0.5dB 0b10: 0 dB 0b11: -0.5dB The correction value is accumulated on IoT values from the latest AAI_ULPC_NI message until the new AAI_ULPC_NI message received and applied. |
| Number of Distributed LRUs for UL feedback channel per a UL AAI subframe UL_FEEDBACK_SIZE | 4 | The number of UL feedback channels according to UL_FEEDBACK_SIZE is specified in 16.3.8.3.3.2 Number of Distributed LRUs for UL feedback channels per a UL AAI subframe (Refer 16.3.8.3.3.2) |
| # Tx antenna | 2 | 0b00: 2 antennas 0b01: 4 antennas 0b10: 8 antennas 0b11: reserved |
| SP scheduling periodicity information | TBD4 | Indicate transmission periodicity of S-SFH SPs according to the Table 807. |
| HO Ranging backoff start | 4 | Initial backoff window size for AMS performing initial handover ranging during HO process, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) |
| HO Ranging backoff end | 4 | Final backoff window size for AMS performing initial handover ranging during HO process, expressed as a power of 2. Values of n range 0-15 |
| Initial ranging backoff start | 4 | Initial backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 |
| Initial ranging backoff end | 4 | Final backoff window size for initial ranging contention, expressed as a power of 2. Values of n range 0-15 |
| UL BW REQ channel information | 3 | Indicate the AAI subframe allocating an UL BW REQ channel. If the number of UL AAI subfrmaes are more than four, UL BW REQ channels are allocated only to first four UL AAI subframes (i.e., maximum number of UL BW REQ channel in a frame is four) 0b000: First UL AAI subframe in the first frame in every superframe 0b001: First UL AAI subframe in the first and second frame in every superframe 0b001010: First UL AAI subframe in every frame 0b010011: Every 2 UL AAI subframes starting from the first UL AAI subframe in every frame 0b100: 4 UL AAI subframes starting from the first UL AAI subframe in every frame 0b011101~111: reserved |
| Bandwidth request backoff start | 4 | Initial backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 (the highest order bits shall be unused and set to 0) |
| Bandwidth request backoff end | 4 | Final backoff window size for contention BRs, expressed as a power of 2. Values of n range 0-15 |
| fpPowerConfig | 4 3 | The power boosting/de-boosting values are listed in Table 814 |
| Reserved | TBD |  |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------