| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the CDMA-based Ranging Procedure (D4 - 16.2.6.3.5.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Heejeong Cho, HyunWoo Lee, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes CDMA-based ranging procedure. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the CDMA-based Ranging Procedure (D4 - 16.2.6.3.5.1)

Heejeong Cho, HyunWoo Lee, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the ranging procedure in the IEEE 802.16m DRAFT Amendment [1]. Ranging is a collection of processes by which the AMS and ABS maintain the quality of the RF communication link between them. It consists of three procedures — initial ranging, periodic ranging and handover ranging. Initial ranging allows an AMS joining the network to acquire correct transmission parameters, such as time offset and power level, so that the AMS can communicate with the ABS. Periodic ranging allows the AMS to adjust transmission parameters so that the AMS can maintain UL communications with the ABS. Handover ranging is allows an AMS re-entering the network to acquire correct transmission parameters so that the AMS can communicate with the target ABS.

Upon reception of ranging preamble codes, ABS should AAI_RNG-ACK message in response to them, but RNG-ACK expiry time is not defined anywhere in D4. Therefore we need to define RNG-ACK expiry time for ranging procedures. However, it is not efficient for all ranging procedures to apply the same value of timer because AMS needs to know decoding status of its HO ranging preamble code as soon as possible in order to minimize handover interruption time [2]. Therefore we propose that RNG-ACK expiry time for handover ranging is shorter than that for initial / periodic ranging.

In addition, because ABS can know which AMS sent the detected dedicated ranging preamble code, it is possible to transmit assignment information for AAI_RNG-ACK, AAI_RNG-REQ and AAI_RNG-RSP in unicast manner.

- Modifications and key-descriptions

- RNG-ACK expiry time

  - Ranging procedure

    - ABS transmits AAI_RNG-ACK within T31 Timer (which is set to timer value for HO ranging) from the frame where at least one HO ranging preamble code is detected.

    - ABS transmits AAI_RNG-ACK within T31 Timer (which is set to timer value for initial ranging) from the frame where at least one initial ranging preamble code is detected.

    - If AMS does not receive AAI_RNG-ACK in response to its ranging preamble code and opportunity or UL grant (e.g., CDMA Allocation A-MAP IE) until T31 Timer is expired, AMS considers that it is NACK.

< Handover ranging>                                   <Initial ranging>

Figure 1 – Example of the proposed CDMA-based ranging procedure

- Upon receiving dedicated ranging preamble code,

  - AAI_RNG-ACK may be allocated by DL basic assignment A-MAP IE, not Broadcast assignment A-MAP IE.

  - AAI_RNG-REQ and AAI_RNG-RSP are allocated by UL basic assignment A-MAP IE and DL basic assignment A-MAP IE, respectively.

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

[2] IEEE 802.16m-07/002r9, “IEEE 802.16m System Requirements”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D4, page 210, line 48, make the following changes to subclause 16.2.6.3.5.1]

16.2.6.3.5.1 CDMA-based HO Ranging Procedure

If the serving ABS indicates in AAI_HO-CMD that CDMA-based HO ranging is not omitted by setting CDMA_RNG_FLAG=1 and a dedicated ranging preamble code/opportunity is assigned to the AMS by target ABS, the AMS shall transmits the dedicated ranging preamble code to the target ABS during network reentry. A servingtarget ABS may schedule a ranging channel (opportunity) for handover purpose only if Ranging channel allocation periodicity in S-SFH SP1 is not set to 0 (i.e., every frame). If a ranging channel is scheduled by the target ABS for handover purpose only, the serving ABS shall also inform the AMS about the resource allocation of the ranging channel, and the ranging opportunity index where the ranging opportunity is located via the AAI_HO-CMD message. In such case, the AMS should use that ranging channel in order to avoid excessive multiple access interference. Upon reception of the dedicated ranging preamble code and if ranging is successful, the target ABS shall allocate uplink resources for AMS to send AAI_RNG-REQ message and/or UL data through the UL basic assignment A-MAP IE with unicast STID. If the target ABS does not receive the dedicated ranging preamble code within Ranging_Initiation_Deadline Timer, the target ABS shall discard the pre-assigned STID of the AMS.

If CDMA-based HO ranging is not omitted and if an AMS does not have a dedicated ranging preamble code or and a dedicated ranging opportunity at the target ABS, the AMS shall transmit a random handover ranging preamble code at the earliest available ranging opportunity.

An AAI_RNG-ACK message shall be sent within T31 Timer from the frame where at least one HO ranging preamble code is detected. If the ABS provides UL resource allocation before sending the response to the successfully received ranging preamble code, the ABS may omit the response in the AAI_RNG-ACK.

The AMS may restart CDMA-based HO ranging procedure when any of the following conditions is met:

- After sending HO ranging preamble code, AMS receives neither AAI_RNG-ACK message corresponding to the HO ranging preamble code and opportunity selected by the AMS nor UL resource allocation (i.e., Basic assignment A-MAP IE, CDMA Allocation A-MAP IE) until the T31 Timer expires;

- AMS does not receive either UL resource allocation in the T3 after receiving a success status notification or AAI_RNG-RSP in the T3 after sending AAI_RNG-REQ;

- AAI_RNG-REQ/RSP transaction is not completed in 128 frames.

[On the IEEE P802.16m/D4, page 295, line 1, make the following changes to subclause 16.2.15.3]

16.2.15.3 Initial ranging and automatic adjustments

Ranging channel and ranging preamble codes for initial ranging are specified in 16.3.9.2.4. Each ranging channel indicates a ranging opportunity.

An AMS that wishes to perform initial ranging shall take the following steps:

- The AMS, after acquiring downlink synchronization and uplink transmission parameters, shall select one ranging opportunity based on the random backoff. The random backoff shall use a binary truncated exponent algorithm. After selecting the ranging opportunity, the AMS shall choose a ranging preamble code (from the Initial Ranging domain) using a uniform random process. The AMS shall send the selected ranging preamble code to the ABS in the selected ranging opportunity.

- The ABS should respond with an AAI_RNG-ACK message in a predefined, subsequent DL AAI subframe within T31 Timer from the frame where at least one initial ranging preamble code is detected as defined in 16.2.3.3. The AAI_RNG-ACK message provides responses to all the successfully received and decoded detected ranging preamble codes in all the ranging opportunities in a frame indicated by the frame identifier previous UL AAI subframe . If all the decoded detected ranging preamble codes prove 'success' status and the ABS provides all UL BW allocations for each detected ranging preamble codes before the T31 Timer is expiredpredefined DL AAI subframe that AAI_RNG-ACK is to be transmitted, the AAI_RNG-ACK may be omitted. If the AMS finds in the RNG-ACK bitmap that no ranging preamble code has been successfully decoded in the ranging opportunity selected by the AMS, or it does not find either a response in the AAI_RNG-ACK message or UL BW allocation to its ranging preamble code receives neither AAI_RNG-ACK message corresponding to the initial ranging preamble code and opportunity selected by the AMS nor UL resource allocation (i.e., CDMA Allocation A-MAP IE) until the T31 Timer expires, the AMS considers its initial ranging request is failed and restarts the initial ranging procedure.

- There are three possible ranging status responses from ABS to AMS provided in the AAI_RNG-ACK message:
- continue: the ABS provides the needed adjustments (e.g., time, power, and possibly frequency corrections) and a status notification of "continue".
- success: the ABS provides a status notification of "success", but may have adjustment suggestions to the AMS if necessary. With IR status successs, the ABS shall provide the AMS an UL BW allocation for the AMS to send AAI_RNG-REQ message.
- abort: the ABS requests the AMS to abort the ranging process.

- Based on the received response of ranging status, the AMS performs the followings.
- Upon receiving a Continue status notification and parameter adjustments in the AAI_RNG-ACK message, the AMS shall adjust its parameters accordingly and continue the ranging process as done on the first entry (using random selection rather than random backoff) with ranging preamble code randomly chosen from the initial ranging domain sent on the initial ranging region opportunity.
- Upon receiving a Success status notification, the AMS shall wait for the ABS to provide UL BW allocation. If the AMS has not received CDMA Allocation A-MAP IE for UL BW allocation by T3 after sending a ranging preamble code, it restarts the initial ranging procedure or, if 'ranging retries' is exhausted, it retries DL PHY synchronization. When receiving an UL BW allocation, the AMS shall send the AAI_RNG-REQ message. If the granted BW allocation cannot accommodate the entire AAI_RNG-REQ message, the AMS may fragment the AAI_RNG-REQ message to fit the provided BW allocation, and requests additional UL bandwidth through either BR without STID header (refer to 16.2.2.1.3.2) or PBREH as defined in 16.2.2.2.8 for the remaining fragments. In response to the BR without STID header or PBREH, the ABS shall provide UL BW allocation through CDMA Allocation A-MAP IE, which is identified by the same RA-ID used for the previous BW allocation. The RA-ID is used until AAI_RNG-RSP transmission is completed, but if AMS does not receive UL bandwidth allocation in T3 or the AAI_RNG-REQ/RSP transaction is not completed in 128 frames, it sends ranging code to perform ranging procedure again.

[On the IEEE P802.16m/D4, page 69, line 4, make the following changes to subclause 16.2.3.3]

16.2.3.3 AAI_RNG-ACK

Upon reception of ranging preamble codes, the AAI_RNG-ACK message is allocated by DL basic Broadcast assignment A-MAP IE using broadcast STID. If ABS receives a dedicated ranging preamble code, AAI_RNG-ACK message in response to the ranging preamble code may be allocated by DL basic assignment A-MAP IE with unicast STID. When the AAI_RNG-ACK message is sent unsolicitedly to specific user, the AAI_RNG-ACK message is allocated by DL basic assignment A-MAP IE with unicast STID. When the AMS receives an unsolicited AAI_RNG-ACK message, it shall reset the periodic ranging timer and adjust its PHY parameters as notified in the AAI_RNG-ACK message. Upon reception of ranging preamble codes, the broadcast AAI_RNG-ACK provides responses to all the successfully received and detected ranging preamble codes in all the ranging opportunities in a frame indicated by the frame identifier. If no periodic ranging preamble code is detected and all the ranging statuses of the detected initial/HO ranging preambles are equal to 'success', the AAI_RNG-ACK may be omitted by allocating UL resources with CDMA Allocation A-MAP IEs or basic assignment A-MAP IEs for all the detected initial/HO ranging preambles. The AAI_RNG-ACK shall include the following parameters:

Table 678—Parameters for AAI_RNG-ACK

| Name | Value | Description |
| --- | --- | --- |
| Management Message Type | AAI_RNG-ACK |  |
| Unicast indication | If set to 1, this message is a unicast message; otherwise this is a broadcast message |  |
| Number of frame identifiers (N_Frames) | The number of frame identifiers included in this RNG-ACK message | Shall be included when the unicast indication is 0b0. |
| Frame identifier | The frame identifier is produced by concatenating the following two values: 1.The 2 least significant bits of the superframe number 2.The frame index within the superframe (ranging from 0b00 to 0b11). | Identifies the each frame which contains the ranging opportunities to which this message refers when the unicast indication is 0b0. |
| RNG-ACK Bitmap | The size of this bitmap is decided by the number of ranging slots opportunities in the referenced frame. Each bit indicates the decoding status of the corresponding ranging opportunity. 0b0: No ranging code is detected; 0b1: At least one code is detected. | Shall be included when the unicast indication is 0b0.AAI_RNG-ACK is sent with broadcast STID |
| Number of received codes |  | The number of ranging preamble code indices detected in this corresponding ranging opportunity. Shall be included for each bit=0b1 in the RNG-ACK Bitmap when the unicast indication is 0b0.AAI_RNG-ACK is sent with broadcast STID |
| Ranging preamble Code Index |  | Ranging preamble cCode index received in this ranging opportunity. Shall be included for each received ranging codes if Number of received codes > 0 when the unicast indication is 0b0.AAI_RNG-ACK is sent with broadcast STID |
| Ranging status | Used to indicated whether UL messages are ranging preamble code is received within acceptable limits by the ABS 0b01: continue 0b11: abort 0b00: success 0b00=success, 0b01= abort, 0b11= continue | When the unicast indication is 0b0AAI_RNG-ACK is sent with broadcast STID, the parameter shall be included for each received ranging code if Number of received codes > 0. When the unicast indication is 0b1AAI_RNG-ACK is sent with unicast STID, the parameter is included for the specific user. |
| Adjustment parameters indication (API) | Bit#0: Time offset adjustment indication. Bit#1: Power level adjustment indication Bit#2: Frequency offset adjustment indication | When the unicast indication is 0b0AAI_RNG-ACK is sent with broadcast STID, this parameter shall be included for each received ranging code when Ranging status is 0b001 or 0b11. When the unicast indication is 0b1AAI_RNG-ACK is sent with unicast STID, the parameter is included for the specific user when Ranging status is 0b001 or 0b11, and in this case Bit#2 is always set to 0. |
| Timing offset adjustment | 105 bits Amount of advance time required to adjust AMS transmission. Signed in units of 1/Fs. | Shall be included when Time offset adjustment indication bit is 1 |
| Power level adjustment | 3 bits | Shall be included when Power level adjustment indication bit is 1 |
| Frequency offset adjustment | 8 bits Relative change in transmission frequency. The correction is 2% of the subcarrier spacing (Δf) multiplied by the 9-bit number interpreted as a signed integer. | Shall be included when Frequency offset adjustment indication bit is 1 |

[On the IEEE P802.16m/D4, page 31, line 24, make the following changes to subclause 10.1]

10. Parameters and constants

10.1 Global values

| System | Name | Time reference | Minimum value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| BS | T50 | See 6.3.23.1 | 100ms | 250ms | 500ms |
| AMS, ABS | T31 | AMS : AAI_RNG-ACK reception timeout following the transmission of a ranging preamble code. ABS : AAI_RNG-ACK transmission timeout following the reception of a ranging preamble code. | 10 ms for handover ranging preamble code 20 ms for initial/periodic ranging preamble code. | - | - |

-----------------------------------------------------------End of the Text---------------------------------------------------------