| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for clarifications on P-SFH contents (D4 - 16.3.6.5.1.1) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Heejeong Cho, Jinsoo Choi, Ki Seon Ryu, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes some modification on section 16.3.6.5.1 (Broadcast Control Information Elements) for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for clarifications on P-SFH contents (D4 - 16.3.6.5.1.1)

Heejeong Cho, Jinsoo Choi, Ki Seon Ryu, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for clarifications for P-SFH contents in the IEEE 802.16m DRAFT Amendment [1].

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

----------------------------------------------------------- Start of the Proposal Text ---------------------------------------------------------

[On the IEEE P802.16m/D4, page 446, line 17, make the following changes to subclause 16.3.6.5.1.1]

16.3.6.5.1.1 P-SFH IE

Table 809—P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| LSB of superframe number | 4 | Part of superframe number |
| S-SFH change count | 4 | Indicates the value of S-SFH change count associated with the S-SFH SPx IE(s) transmitted in this superframe |
| S-SFH size | 42 | The units of LRU for S-SFH |
| S-SFH number of repetitions | 21 | Indicate the transmission format (repetition) used for S-SFH. 0b0: 6 repetition, 0b1: 2 repetition |
| S-SFH scheduling information bitmatp | 3 | 0b000: no S-SFH If 1st bit (LSB)= 1, S-SFH includes SP1 IE. Otherwise no SP1 IE If 2nd bit = 1, S-SFH includes SP2 IE. Otherwise no SP2 IE If 3rd bit = 1, S-SFH includes SP3 IE. Otherwise no SP3 IE |
| S-SFH SP change bitmap | 3 | Indicates the change of S-SFH SPx IE(s) associated with the S-SFH change count. If in S-SFH SP1 IE is changed, then bit#0 (LSB) is toggled. Otherwise no change in SP1. If in S-SFH SP2 IE is changed, then bit#1 is toggled. Otherwise no change in SP2. If in S-SFH SP3 IE is changed, then bit#2 (MSB) is toggled. Otherwise no change in SP3. |
| Start super-frame offset where new S-SFH information is used | 2 |  |
| Reserved | 32 | The reserved bits are for future extension. |
| } |  |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------