| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on coverage loss detection in sleep mode (D4 - 16.2.26) |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Giwon Park, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7188 E-mail: giwon.park@lge.com |
| Re: | IEEE 802.16-10/0011, IEEE 802.16 Working Group Letter Ballot #31 |  |
| Abstract | This contribution proposes methods of AMS’s coverage detection during the sleep mode. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on coverage loss detection in sleep mode (D4 - 16.2.26)

Giwon Park, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics

- Introduction

D4 [1] does not define how the ABS and the AMS detect coverage loss in sleep mode. This contribution proposes the procedures of coverage loss detection in sleep mode.

- Modifications and key-descriptions

- AMS’s coverage loss detection in sleep mode

  - When the AMS missed the SFH in sleep mode, it shall awake until it receives the SHF.

Figure 1. SFH miss of AMS in sleep mode

  - If the AMS missed the SFH several times, AMS shall regard it as link loss from the ABS and shall perform the coverage loss recovery procedure.

Figure 2. AMS’s coverage loss detection in sleep mode

- ABS’s coverage detection in sleep mode

  - Allocation of UL Grant

    - For AMS's keep alive check, ABS shall grant the UL burst in the sleep mode.

  - Active_ABS_timer based UL grant (Scenario 1)

    - An ABS transmits the AAI_SLP-RSP message with active_ABS_timer at sleep mode initiation.

    - An active_ABS_timer is started in start_frame_number which is indicated in AAI_SLP_RSP message.

    - Upon expiration of the active_ABS_timer, to check whether an AMS is still alive, the ABS shall grant UL burst to the AMS.

    - The AMS can implicitly know allocation time for UL grant based on active_ABS_timer and AMS shall transmit the padding PDU or an MPDU on the UL grant.

    - Although there is additional message overhead, AMS can implicitly know allocation time for UL grant.

Figure 3. Coverage loss detection an ABS’s behavior

  - Active_ABS_timer based UL grant (Scenario 2)

    - When the listening widow is overlapped with UL grant time based on active_ABS_timer which is used in active state, ABS allocates the UL grant to AMS.

      - Active_ABS_timer is not transmitted to the AMSs via AAI_SLP-RSP.

    - Although there is no additional message overhead, if the listening window is not overlapped with the UL grant time, ABS can not grant the UL burst to AMS.

  - Active_ABS_timer based UL grant (Scenario 3)

    - In this scenario, ABS may grant the UL burst at listening window which is the nearest to the point of active_ABS_timer’s expiration.

Figure 4. Coverage loss detection an ABS’s behavior

- References

[1] IEEE P802.16m/D4, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------Start of the Text-------------------------------------------------------------------

[On the IEEE P802.16m/D4, page 337, line64, make the following changes to subclause 16.2.26.1]

16.2.26.1 Coverage loss detection at ABS and ABS’s behavior

Upon each expiration of the active_ABS_timer, to check whether an AMS is still alive in active mode, the ABS shall grant UL burst to the AMS and the AMS shall transmit a the padding PDU or an MPDU on the UL grant.

In sleep mode, ABS may grant the UL burst at the listening window which is the nearest to the point of active_ABS_timer’s expiration.

-----------------------------------------------------------End of the Text-------------------------------------------------------------------