| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for LBS Basic Capabilities in AAI_SBC-REQ/RSP message(16.2.3) |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Lei Zhou, Xufeng Zheng, Beijing Samsung Telecom R&D Center Sangheon Kim, Jaehyuk Jang, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | P802.16m/D4 |  |
| Abstract | The contribution proposes to clarify LBS basic capabilities in AAI_SBC-REQ/RSP message. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for LBS Basic Capabilities in AAI_SBC-REQ/RSP message (16.2.3)

Lei Zhou, Xufeng Zheng,

Beijing Samsung Telecom R&D Center

Sangheon Kim, Jaehyuk Jang, Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Introduction

Basic capabilities on LBS have been included in 802.16m/D4 and basic capabilities including some metrics such as CINR, RSSI, RTD and RD on LBS which should be indicated during AAI-SBC-REQ/RSP negotiation is mentioned in AAI_SCN-RSP message. So we propose to clarify LBS basic capabilities in AAI_SBC-REQ/RSP message.

- The Proposed Text in AWD

-----------------------------------------  Proposed Text Start  -------------------------------------------------

[Adopt the following text modification started from line#32 Page 70 in P802.16m/D4]

16.2.3.4 AAI_SBC-REQ

An AAI_SBC-REQ message, to which HARQ operation is applied, is transmitted by AMS to negotiate basic capability during network entry.

The AAI_SBC-REQ message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed. In Table 679, the CAPABILITY_INDEX transmitted in the AAI_SBC-REQ message refers to the maximum "Capability Class" that the AMS can support. The maximum value of CAPABILITY_INDEX is denoted by [TBD] bits.

Table679: AAI_SBC-REQ message format

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| AAI_SBC-REQ_Message_Format() { | - | - |
| Management Message Type = 26 | 8 | - |
| CAPABILITY_INDEX | TBD |  |
| } | - | - |

The following parameters may be included and parameter sets are mapped to cabability index( the mapping is TBD):

- Authorization policy support

    - If Bit #0=0, EAP-based authorization is not supported;

    - If Bit #0=1: EAP-based authorization is supported

- PN Window Size : Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks

- Auth type for EAP : Auth Type for EAP shall only be included when EAP-based authorization is supported.

    - If Bit #0=0, device authentication

    - If Bit #0=1, user authentication

- Scanning report metric capability

    - Bit 0: BS CINR mean

    - Bit 1: BS RSSI mean

    - Bit 2: Relative delay

    - Bit 3: BS RTD

    - Bits 4-5: Reserved; shall be set to zero

16.2.3.5 AAI_SBC-RSP

An AAI_SBC-RSP message, to which HARQ operation is applied, is transmitted by ABS in response to a received AAI_SBC-REQ during initialization.

The AAI_SBC-RSP message shall be encrypted and not contain CMAC Tuple during HO reentry if authentication has been completed. In Table 680, the CAPABILITY_INDEX transmitted in the AAI_SBC-RSP message refers to the "Capability Class" that the ABS has allowed the AMS to use during the session. The value of CAPABILITY_INDEX signaled in the AAI_SBC-RSP message is numerically smaller than or equal to the CAPABILITY_INDEX signaled in the AAI_SBC-REQ message by the AMS.

Table 680: AAI_SBC-RSP message format

| Syntax | Size (Bits) | Notes |
| --- | --- | --- |
| AAI_SBC-RSP_Message_Format() { | - | - |
| Management Message Type = 27 | 8 | - |
| CAPABILITY_INDEX | TBD |  |
| } | - | - |

The following parameters may be included and parameter sets are mapped to cabability index( the mapping is TBD):

- Authorization policy support

    - If Bit #0=0, EAP-based authorization is not supported;

    - If Bit #0=1: EAP-based authorization is supported

      - PN Window Size : Specifies the size capability of the receiver PN window. The receiver shall track PNs within this window to prevent replay attacks

      - Scanning report metric capability

    - Bit 0: BS CINR mean

    - Bit 1: BS RSSI mean

    - Bit 2: Relative delay

    - Bit 3: BS RTD

    - Bits 4-5: Reserved; shall be set to zero

----------------------------------------- Proposed Text End  ------------------------------------