| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimized and Simplified Resource Allocation field in Subband Assignment A-MAP IE |  |
| Date Submitted | 2010-03-05 |  |
| Source(s) | Yanfeng Guan, Xianming Chen, Huiying Fang ZTE Corporation | guan.yanfeng@zte.com.cn |
| Re: | Comment on “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P802.16m/D4” 16.3.6 Downlink Control Structure & 16.3.8 Uplink Control Structure |  |
| Abstract | This contribution proposes text optimizing the “Resource Allocation” filed in the Subband A-A-MAP IEs. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimized and Simplified Resource Allocation field in Subband Assignment A-MAP IE

Yanfeng Guan, Xianming Chen, Huiying Fang

ZTE Corporation

- Introduction

In the current 802.16m draft amendment [1], four types of Assignment A-MAP IEs (DL/UL Basic A-A-MAP and DL/UL Subband A-A-MAP) are defined and used for dynamic scheduling. ONLY DL/UL Subband Assignment A-MAP IEs can be used for an allocation of non-contiguous subband-based LRUs, and the “Resource Index” (RI) field specifies the resource location and size.

- Problem

For RI field in DL/UL Subband Assignment A-MAP IEs, there are still some problems:

  - Three types of resource units (DLRU, NLRU and SLRU) are supported; ABS shall be able to assign any allocation size as possible because there is no any restriction on supportable allocation size in the channel coding section. For example, Table 924 in D4 shows the supportable allocation size no matter what type of resource unit.

Table 924—Minimal size index as a function of the allocation size

| Allocation size | IMinimalSize | Allocation size | IMinimalSize | Allocation size | IMinimalSize |
| --- | --- | --- | --- | --- | --- |
| 1 ~ 3 | 1 | 16 ~ 18 | 15 | 58 ~ 64 | 26 |
| 4 | 2 | 19 ~ 20 | 16 | 65 ~ 72 | 27 |
| 5 | 4 | 21 ~ 22 | 17 | 73 ~ 82 | 28 |
| 6 | 6 | 23 ~ 25 | 18 | 83 ~ 90 | 29 |
| 7 | 8 | 26 ~ 28 | 19 | 91 ~ 102 | 30 |
| 8 | 9 | 29 ~ 32 | 20 | 103 ~ 116 | 31 |
| 9 | 10 | 33 ~ 35 | 21 | 117 ~ 131 | 32 |
| 10 ~ 11 | 11 | 36 ~ 40 | 22 | 132 ~ 145 | 33 |
| 12 | 12 | 41 ~ 45 | 23 | 146 ~ 164 | 34 |
| 13 | 13 | 46 ~ 50 | 24 | 165 ~ 184 | 35 |
| 14 ~ 15 | 14 | 51 ~ 57 | 25 | 185 ~ 192 | 36 |

In current D4, only two different resource allocation granularities (a subband and a half subband) in many cases are supported, as a result of that, the odd discontinuous SLRU allocation can’t be supported. For example, if the required allocation size by AMS is odd (for example 3 SLRUs), ABS can only assign 2 or 4 SLRUs for it because SLRU indexing method only support even SLRU allocation. If the allocation size is 2 SLRUs, the actual coding rate will be remarkably increased, and then it leads to the high error bit rate. Otherwise, the allocation size is 4 SLRUs, the resource scheduling efficiency will decrease because 3 SLRUs are enough under the given burst size and CQI.

Hence, it’s not reasonable to limit the allocation size on the BS scheduler, and all the odd allocation size except 1, 3 and 5 should be especially supported because they will be often used in the resource scheduling process. This is very important for the transmission of some small bursts such as VoIP bursts.

  - The allocation size of odd SLRUs should be supported to meet the rate matching mechanism in the channel coding. Because of the rate matching mechanism, the actual code rate often approaches to the code rate related to the nominal MCS from the AMS feedback, but differs with that. For example, if  the CQI feedback from AMS is MCS index ‘1000’ in Table 1, the most expected code rate is between 128/256 and 184/256.

Table 1 Nominal MCS and the Ratio of the adjacent two MCS Rate

| MCS Index | Modulation | Code Rate | Ratio of the adjacent two MCS Rate |
| --- | --- | --- | --- |
| 0000 | QPSK | 31/256 | 48/31=1.55 |
| 0001 | QPSK | 48/256 | 71/48=1.48 |
| 0010 | QPSK | 71/256 | 101/71=1.42 |
| 0011 | QPSK | 101/256 | 135/101=1.34 |
| 0100 | QPSK | 135/256 | 171/135=1.27 |
| 0101 | QPSK | 171/256 | 102*2/171=1.19 |
| 0110 | 16QAM | 102/256 | 128/102=1.25 |
| 0111 | 16QAM | 128/256 | 155/128=1.21 |
| 1000 | 16QAM | 155/256 | 184/155=1.19 |
| 1001 | 16QAM | 184/256 | 135*6/184*4=1.10 |
| 1010 | 64QAM | 135/256 | 157/135=1.16 |
| 1011 | 64QAM | 157/256 | 181/157=1.15 |
| 1100 | 64QAM | 181/256 | 205/181=1.13 |
| 1101 | 64QAM | 205/256 | 225/205=1.10 |
| 1110 | 64QAM | 225/256 | 237/225=1.05 |
| 1111 | 64QAM | 237/256 | / |

However, the actual code rate will remarkably differ with the expected code rate because of the allocation granularity. If the expected MCS index and code rate are 1000 and 155/256, respectively, and the expected allocation size is 3, no matter 2 SLRUs or 4 SLRUs are assigned, the actual code rate will be in out of the range [128/256, 184/256], that is not the expectation of ABS.

Table 2 Ratio of Size Difference and Expected Size

| Expected Allocation Size | Assigned Allocation Size | Size Difference | Ratio of Size Difference and Expected Size |
| --- | --- | --- | --- |
| 3 SLRUs | 2 SLRUs | 1 SLRU | 33.3% |
| 3 SLRUs | 4 SLRUs | 1 SLRU | 33.3% |
| 5 SLRUs | 4 SLRUs | 1 SLRU | 20.0% |
| 5 SLRUs | 6 SLRUs | 1 SLRU | 20.0% |
| 7 SLRUs | 6 SLRUs | 1 SLRU | 14.3% |
| 7 SLRUs | 8 SLRUs | 1 SLRU | 14.3% |

In a word, we can find that the maximum possible ratio of the adjacent two Nominal MCS rate is 1.55, and there are a half of ratios less than 1.20. Therefore, the advantage of the code matching mechanism will be reduced if odd number of allocation size can’t be supported. If the granularity of half subband is prohibited and only one subband granularity is supported, the problem will be enlarged.

Table 3 Ratio of Size Difference and Expected Size

| Expected Allocation Size | Assigned Allocation Size | size Difference | Ratio of Size Difference and Expected Size |
| --- | --- | --- | --- |
| 2 SLRUs | 4 SLRUs | 2 SLRU | 50.0% |
| 3 SLRUs | 4 SLRUs | 1 SLRU | 33.3% |
| 5 SLRUs | 4 SLRUs | 1 SLRU | 20.0% |
| 5 SLRUs | 8 SLRUs | 3 SLRU | 60.0% |
| 6 SLRUs | 4 SLRUs | 2 SLRU | 33.3% |
| 6 SLRUs | 8 SLRUs | 2 SLRU | 33.3% |

  - When the total subband number is more than 11, Single IE or two IEs may be used for one allocation instance. When two IEs are used, the overhead and the error probability of decoding the IEs for one allocation instance will increase, so we should use single IE for one allocation instance as possible as we can. However, single IE is ONLY for the allocation and indexing for 2 or 3 non-contiguous subbands, that is to say, ONLY 1501 cases can be indexed by using the single IE for the maximum subband number (21). In addition, there are some other problems:

    - According to some MIMO Feedback mode, the Best-M Subbands should be provided, and M can be 4, min (5, YSB) or min (10, YSB). However, only Best-2 or 3 Subbands can be assigned when using the single IE no matter how many Best Subbands there are.

    - The current method for single IE depends on the look-up table; it’s very complicated for the implementation and parsing IE.

So, we should support the Best-M Subband feedback up to the maximum requirement such as min (5, YSB) or min (10, YSB) when using the single IE. At the same time, a bitmap-based new method in section 3 is proposed for simplifying the method instead of the complicated look-up table.

  - The whole subband indexing method should be specified according to the subband number, in order to adapt it to any allowed system bandwidth.

- Proposed Method

- 3.1 Method 1

The total subband number is denoted by YSB, and the RA field is still 11-bit length.

- When 1≤YSB≤3, 4 SLRUs in each subband is indicated by 3 bits. ONLY one Subband IE is for one resource allocation instance.

Figure 1- Enhanced SLRU Indexing Method when YSB≤3

Obviously, the enhanced method has more strong indexing ability than that in Draft 4, it can index any needed case under the same overhead limitation (11bits). But it’s very important and helpful for the resource allocation when YSB is very low.

- When 4≤YSB≤11, the granularity is a half subband (2 SLRUs) for the first 11-YSB subbands, and a whole subband for remained 2YSB-11 subbands. ONLY one Subband IE is for one resource allocation instance. Figure 3 shows the general case.

Figure 2- Enhanced SLRU Indexing Method when 4≤YSB≤11

- When 12≤YSB≤21, the granularity is always a whole subband. ONE or TWO Subband IEs are for one resource allocation instance.

a) When ONE Subband IE is used, the MSB bit is denoted as the Group Indication (GI) and other 10 bits is used to index the resource by using the Bitmap method. If 1-bit GI is set 0b0, the first 10 subbands will be indexed by using the bitmap method; otherwise, the last 10 subbands will be indexed.

Figure 3- Enhanced SLRU Indexing Method when 12≤YSB≤21

b) When TWO Subband IEs are used, the RA fields of the two IEs shall be concatenated to form a 22-bit field, referred to as the Concatenated-RA field (C-RA field). The LSB of the RA field of the IE occurring last in the A-AMAP region shall be interpreted as the LSB of the C-RA field. The ABS shall map the two IEs carrying the single instance of the allocation to MLRUs with contiguous logical indices. The AMS shall infer that two IEs refer to the same instance of a resource allocation from the values of the ACID and SPID fields. The AMS shall interpret the C-RA field in this case as defined below.

Figure 4- Enhanced SLRU Indexing Method when 12≤YSB≤21

Actually, SINGLE IE-based method is easier and has strong indexing ability because it only depends on the Bitmap and indicates 1936 cases when the subband number is 21.

- 3.2 Method 2

The basic idea is totally similar to the method 1; the method 2 can make the resource indexing better.

The total subband number is denoted by YSB, and the RA field is extended to 12-bit length.

- When 1≤YSB≤4, 4 SLRUs in each subband is indicated by 3 bits. ONLY one Subband IE is for one resource allocation instance.

Figure 5- Enhanced SLRU Indexing Method when YSB≤4

Obviously, the enhanced method has more strong indexing ability for the 5MHz system.

- When 5≤YSB≤11, the granularity is a half subband (2 SLRUs) for the first Min(12-YSB, YSB) subbands, and a whole subband for other 2YSB-12 subbands. ONLY ONE Subband IE is for one resource allocation instance. Figure 3 shows the general case.

Figure 6- Enhanced SLRU Indexing Method when 5≤YSB≤11

- When 12≤YSB≤21, the granularity is always a whole subband. ONE or TWO Subband IEs are for one resource allocation instance.

12 bits are partitioned into two parts. The MSB is denoted as the Group Indication (GI) and other 11 bits is denoted as Resource Indication Field (RIF) and used to index the resource. If 1-bit GI is set as 0b0, the first 11 subbands will be indexed by using the bitmap; otherwise, the last 11 subbands will be indexed.

Figure 7- Enhanced SLRU Indexing Method when 12≤YSB≤21

      - Only one IE is enough if the required SBs are in first or last 11 SBs;

      - If the required SBs are in two groups, ABS can sent two IEs with same structure. No matter how many IEs are decoded successfully by a AMS, this AMS doesn’t have wrong understanding and affect other AMSs because the meaning of MSB in each IE is same and there is no strong relation between these two IEs.

Based on the considerations above, we propose some enhanced modifications on the resource index field in DL/UL Basic/Subband-Assignment A-MAP IE.

- References

[1] "DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface", P802.16m/D3 Dec. 2009.

[2] IEEE 802.16m-07/002r8, “IEEE 802.16m System Requirements Document”

[3] IEEE 802.16m-08/003r7, “IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

[5] IEEE 802.16m-09/1327r1, “Proposed Changes to the DL and UL A-MAP Information Element Types in AWD ”

[6] IEEE 802.16m-09/1331r2, “Proposed AWD text specifying the “Resource Allocation” field in the A-MAP IEs AWD ”

[7] IEEE 802.16m-09/1334r1, “AWD Proposal for DL/UL Resource Indexing in IEEE 802.16m ”

[8] IEEE 802.16m-09/1516r “Proposed “Resource Allocation” Field in Basic Assignment A-MAP IE in IEEE802.16m ”

[9] IEEE 802.16m-09/1814r “Proposed “Resource Allocation” Field in Basic Assignment A-MAP IE in IEEE802.16m ”

[10] IEEE 802.16m-09/2895 “Proposed “Resource Allocation” Field in Basic Assignment A-MAP IE in IEEE802.16m ”

- Proposed Text

Remedy: Replace the text from line 56 on page 471 to line 56 on page 493 in the 16.3.6.5.2.4.3 DL Sub-band Assignment A-MAP IE as follows:

------------------------------------------------ Proposed Text Starts -----------------------------------------------

The Resource Index field is 12-bit length. According to the total subband number YSB, three cases are partitioned as follows.

When 1≤YSB≤4, 4 SLRUs in each subband are partitioned three allocation units, which consists of the first two SLRUs, the third SLRU and the fourth SLRU, respectively. Each allocation unit is indicated by 1 bit, if this bit is set as 0b0, this allocation unit is assigned; otherwise, this allocation unit is not assigned. ABS sends one Subband Assignment A-MAP IE for one resource allocation instance. Figure X shows the interpretation of the Resource Index when 1≤YSB≤4.

Figure X- Interpretation of the Resource Index, when 1≤YSB≤4

When 5≤YSB≤11, the allocation unit consists of a half subband for the first Min(12-YSB, YSB) subbands, and a whole subband for last 2YSB-12 subbands when 2YSB-12 is more than 0. ABS sends one Subband Assignment A-MAP IE for one resource allocation instance. Figure Y shows the interpretation of the Resource Index when 5≤YSB≤11.

Figure Y- Interpretation of the Resource Index, when 5≤YSB≤11

When 12≤YSB≤21, the allocation unit consists of a whole subband. The total subbands are partitioned into two groups, the first group consists of the first 11 subbands, and the second group consists of the last 11 subbands. 12 bits are partitioned into two parts, the MSB is denoted as the Group Indication (GI) and used to indicate which group is selected, and other 11 LSBs is denoted as Resource Indication Field (RIF) and used to index which subbands are assigned in selected group. If 1-bit GI is set as 0b0, the first group is selected, otherwise the second group is selected. Each bit in RIF is set as 0b1, the subband is assigned, otherwise, it is not assigned. Figure Z shows the interpretation of the Resource Index when 12≤YSB≤21.

If the required subbands are in first or second group, ABS sends one Subband Assignment A-MAP IE for one resource allocation instance; if the required subbands are in first and second groups, ABS sends two one Subband Assignment A-MAP IEs for one resource allocation instance.

Figure Z- Interpretation of the Resource Index, when 12≤YSB≤21

------------------------------------------------ Proposed Text Ends ------------------------------------------------